3GPP TSG-RAN WG4 Meeting #102-e						                                     R4-2206117

Electronic Meeting, 21st of February – 3rd of March, 2022

Agenda item:			10.13.3.3

Source:	Inmarsat

Title:	Draft TP for TS 38.108 Section 6.6.4 Operating band unwanted emissions

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#101-bis-e, a work split has been proposed to populate Section 6 of TS 38.108. In this document, an initial draft TP is proposed to update some of the subsections related to RF Conducted requirements.

# Discussion

In this contribution is proposed to add information with respect to Section 6.6 Unwanted emissions of TS 38.108 [1] based on justifications and recommendations provided in TR 38.863 [2].

Proposal 1: To update the TS with the attached TP

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TS with the attached TP

# Reference

[1] TS 38.108, “NR; Satellite Node radio transmission and reception”;

[2]	TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”;

# Text proposals for TS 38.108

------------------------------------------------<Start of TP>-----------------------------------------------

### 6.6.4	Operating band unwanted emissions

#### 6.6.4.1	General

Unless otherwise stated, the operating band unwanted emission (OBUE) limits for SAN in FR1 are defined from ΔfOBUE below the lowest frequency of each supported downlink operating band up to ΔfOBUE above the highest frequency of each supported downlink operating band. The values of ΔfOBUE are defined in table 6.6.1-1 for the NR operating bands.

The requirements shall apply whatever the type of transmitter considered and for all transmission modes foreseen by the manufacturer’s specification.

Basic limits are specified in the tables below, where:

-	f is the separation between the channel edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the channel edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency ΔfOBUE outside the downlink operating band, where ΔfOBUE is defined in table 6.6.1-1.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a multi-carrier single-band connector or a single-band connector configured for intra-band contiguous carrier aggregation the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency band.

-	The operating band unwanted emission basic limits of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from ΔfOBUE MHz below the lowest frequency, up to ΔfOBUE MHz above the highest frequency of the supported downlink operating band without any carrier transmitted.

The application of either Category A or Category B basic limits shall be the same as for Transmitter spurious emissions in clause 6.6.5.

------------------------------------------------<End of TP>-------------------------------------------------