3GPP TSG-RAN WG4 Meeting #102-e	R4-2206566

Electronic Meeting, 21 Feb - 3 Mar, 2022

Title:	Reply LS on power control for NR-DC

Response to:	(R1-2104018) Further Reply LS on power control for NR-DC

Release:	Rel-16

Work Item:	LTE_NR_DC_CA_enh-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

Name: Jinqiang Xing

E-mail Address: xingjinqiang@OPPO.com

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN1 for the Further Reply LS on power control for NR-DC.

Regarding the feasibility of independent power control for the mentioned two cases:

1) uplink CCs of MCG and uplink CCs of SCG are in different frequency bands in FR2.

2) uplink CCs of MCG and uplink CCs of SCG are in the same frequency band in FR2.

RAN4 has discussed the definition of independent power control, and the understanding is that it means power control is per CG, and there is no total power limitation.

Up to now RAN4 hasn’t introduce NR-DC band combination for FR2, therefore UL CA is referred here and it is RAN4 understanding that same conclusion can be applied to NR-DC.

For intra-band UL CA, total max TRP and max EIRP limitation are defined in section 6.2A.4 of TS 38.101-2, thus power sharing is needed when the maximum power limitation was reached. Therefore, it is not independent power control.

For inter-band UL CA:

RAN4 has discussed two kinds of UEs to support inter-band CA, one is called CBM (Common Beam Management), and the other is called IBM (Independent Beam Management), and in Rel-17 RAN4 only defined requirements for IBM in inter-band UL CA. It is agreed that independent power control might be possible in some case for IBM but is not guaranteed UE behavior. RAN4 hasn’t discussed CBM for inter-band UL CA up to Rel-17.

In addition, RAN4 do not plan to discuss p-NR-FR2 or p-UE-FR2 in Rel-17.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #103-e	16  -  27 May 2022         E-meeting