3GPP TSG-RAN WG4 Meeting #103-e					R4-2207785

Electronic Meeting, May 9 – 20, 2022

Agenda item:	7

Source: 	Apple

Title: 	Further discussion on R17 feature list

Document for:	Discussion

- Introduction

In this document, we continue to share our views on Rel-17 NR UE features based on the agreed feature list in R4-2206571. We propose UE capabilities not yet included in R4-2206571 or comment on the UE capabilities included in R4-2206571.

- NR UE feature

  - Extending current NR operation to 71GHz

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X.Extending current NR operation to 71GHz | X-1 | UE support of CBW for supported SCS | Capability of supported CBW - 400MHz for 120kHz SCS - {800, 1600} MHz for 480kHz SCS - {800, 1600, 2000} MHz for 960kHz SCS NOTE 1: this capability may need to be split into three capabilities, i.e. one for each supported SCS NOTE 2: 100 MHz is a mandatory CBW if the UE supports 120 kHz SCS NOTE 3: 400 MHz is a mandatory CBW if the UE supports 480 kHz or 960 kHz SCS |  | yes | no | The network does not know if UE can transmit or receive with a specific CBW whose support is optional | per Band | No | No |  |  | Optional with capability signalling |

- Improved ON/ON transient period as the optional UE capabilities for 480 and 960 kHz SCS was discussed during the RAN4 #102 meeting, and there was no agreement.  It is proposed to deprioritize this discussion in Rel-17 and to consider it in Rel-18.

- UE beam direction-only switching time was discussed during the RAN4 #102 meeting, with a majority of companies supporting reusing 200 nsec as in FR2-1. As such, no UE capability is needed.

  - NR and MR-DC Measurement Gap Enhancements

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Concurrent measurement gap | 19-2-1 | Concurrent measurement gaps | o Support of concurrent gaps with association between inter-RAT measurement objects and multiple gap patterns | 19-2 | yes | no | Network can only associate inter-RAT measurement objects to one measurement gap | Per UE | No | No |  |  | Optional with capability signalling |
| Concurrent measurement gap | 19-2-2 | Concurrent measurement gaps | o Indicate the supported maximum gap overhead | 19-2 | yes | no | Network does not know the maximum gap overhead supported by the UE | Per UE | No | No |  | The definition of gap overhead is specified in TS38.133 Type 1: 30% Type 2: unlimited | Optional with capability signalling |

  - NR RF requirement enhancements for frequency range 1 (FR1)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- In the context of requirements for intra-band UL CA + UL MIMO, differentiation of requirement applicability based on PA architecture may be needed.

During RAN4 #101-bis, the following was captured in the email discussion summary:

Issue 4-1: TxD support per band per band combination Issue 4-2: New power class signalling per band per band combination Continue to discuss how to solve the ambiguity of per-band power class for PC2 CA/DC in 2nd round. Option 1(Apple): indicate TxD support per band per band combination Option 2 (MediaTek, Huawei, Ericsson): indicate new power class per band per band combination Other options are not precluded. Recommended WF: More companies prefer to consider option 2 to solve the issue. And some companies believe these two issues can be discussed separately. Continue to discuss in next meeting.

Thus, we should discuss and converge on a solution during the RAN4 #103 meeting.

  - NR RF requirement enhancements for frequency range 2 (FR2)

The table below is copied from R4-2206571. Apple views are provided after the table.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL gap for Tx power management | 17-1 | Support of UL gap for Tx power management | Capability of performing BPS sensing for Tx power management. The UE indicating this capability shall meet the corresponding enhanced UE requirements defined in Section TBD. |  | yes | no | UE does not support UL gap for Tx power management | Per band | No | FR2 only |  |  | Optional with capability signalling |
| UL gap pattern for Tx power management | [17-2] | Support of UL gap patterns for Tx power management | Capability of supporting UL gap patterns (UL MGP #0, #1, #2, #3 as specified in TS 38.133) needed for performing BPS sensing for Tx power management. The UE indicating this capability shall meet the corresponding enhanced UE requirements defined in Section TBD. | 17-1 | yes | no | The UE does not support specified UL gap patterns needed for Tx power management | Per UE | No | FR2 only |  | UE is mandated to support at least one of UL MGP #1 and #3 when it indicate support of UL gap for Tx power management (FG 17-1). All other gap patterns except for the one or two selected mandatory gap pattern(s) are optional | Optional with capability signalling |
| [UL gap for coherent UL MIMO ] | [17-3] | Support of UL gap for coherent UL MIMO | Capability of performing coherent UL MIMO calibration in UL gap. The UE indicating this capability shall meet the corresponding enhanced UE requirements defined in Section TBD. |  | yes | no | UE does not support UL gap for coherent UL MIMO calibration | per UE | No | FR2 only |  |  | Optional with capability signalling |
| 17. FR2 interband CA | 17-4 | Support of beam management | Capability of support of specific beam management type. |  | yes | no | UE does not support FR2 interband CA | Per band combination | No | FR2 only |  | Indicate the supported beam management type for inter-band CA within FR2. Beam management type can be independent beam management (IBM) or common beam management (CBM), or both. The capability is only applicable to band combinations with two bands. | Optional with capability signalling |
| DC-location | [17-5] | Support of UL DC location(s) report | [Capability of support for the extended DC location reporting (based on indicated default DC location) for at least 2 UL CCs.] |  | yes | no | UE does not support the Rel-17 extended UL DC location reporting | [Per band per BC] | No | No |  |  | Optional with capability signalling |
| New CA BW clases | [17-6] | Support of new CA BW Classes | RAN4 has introduced new CA BW Classes ‘R, S, T, U’ for REL17 |  | yes | no | UE does not support the Rel-17 extended FBG2 bandwidths | per band | No | FR2 only |  |  | Optional with capability signalling |
| [FBG 3+2] | [17-7] | [Support of new CA BW Classes] | RAN4 may introduce new fall back group and or new CA BW classes under FBG3+2 discussion |  | yes | no |  | per band | No | FR2 only |  |  | Optional with capability signalling |
| UL transmission in FR2 bands within an UL gap when the UL gap is activated | 17-8 | Support of UL transmission in FR2 bands within an UL gap when the UL gap is activated in inter-band UL CA | UE indicates the constituent band(s) for which UL transmission is supported within an UL gap when the UL gap is activated in inter-band UL CA. | 17-1 |  |  | The UE does not support UL transmission within an UL gap when the UL gap is activated in inter-band UL CA. | Per band per band combination | No | FR2 only |  |  | Optional with capability signalling |

- Based on the agreed WF R4-2206604, brackets [17-2] can be removed.

  - Further enhancement on NR demodulation performance

The table below is copied from R4-2206571. Apple views are provided after the table.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_perf_enh2_Demod | 24-1 | CRS-IM (Interference Mitigation) in DSS scenario | Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS Note: In the DSS scenario, serving and neighboring cells are both operating with dynamic spectrum sharing (DSS) of NR and LTE. | 5-28 (Rate-matching around LTE CRS) | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in DSS scenario | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD | Note: UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. | Optional with capability signaling |
| 24. NR_perf_enh2_Demod | 24-2 | CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth Note: In the non-DSS scenario, serving cell is operating in NR, and neighboring cells are operating in LTE. |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| 24. NR_perf_enh2_Demod | 24-3 | CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| 24. NR_perf_enh2_Demod | [24-4] | CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| 24. NR_perf_enh2_Demod | [24-5] | CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Support of neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth |  | Yes | N/A | NR UE does not support neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth | Per FSPC | No | Applicable only to FR1 | Support mixture of FDD/TDD |  | Optional with capability signaling |
| 24. NR_perf_enh2_Demod | 24-6 | MMSE-IRC receiver for scenarios with inter-cell and intra-cell inter-user interference | Support of MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference |  | No | No | UE can’t apply mitigation of inter-cell and intra-cell inter-user interference | Per UE | No | FR1 only | N/A |  | Optional without capability signalling for Rel-15 and Rel-16 Mandatory without capability signalling from Rel-17 |

- Feature groups [24-4] and [24-5] should be updated based on the agreements whether CRS-IM requirements will be introduced in 30KHz SCS

  - Increasing UE power high limit for CA and DC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Because the work item on increasing UE power high limit for CA and DC has just started, we recommend returning to this topic of signaling related to this feature in the next meeting.

  - Tx Diversity

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TxD | X-1 | TxD support per band per band combination | Support of transmit diversity per band per band combination | TxD support per band | Yes | No | UE uses a single Tx | Per FSPC (per CC per band per BC) | No | FR1 only | N/A |  | Optional with capability signalling |

- New signaling was agreed to be introduced in Rel-17, and RAN2 has introduced a per-band optional capability with a restriction to FR1 only

- In the case of a UE supporting TxD in a particular band within a band combination, we should consider the scenario of a UE falling back to single Tx operation (i.e. in a single carrier configuration the UE supports TxD, but when used with inter-band CA the UE falls back to single Tx); we do have this signaling for EN-DC, but not for inter-band CA; thus, we need an additional per band per band combination capability that indicates that the UE supports TxD in band X applicable to all inter-band UL CA combinations