3GPP TSG-RAN WG4 Meeting #103-e                                                                                      R4-2208226

Electronic Meeting, May 09 – May 20, 2022

Source: 	CATT

Title: 	Discussion on the FFT sizes for 52.6-71GHz

Agenda item:	9.15.3.1

Document for:	Approval

- Introduction

The FFT sizes need to be further discussed. In this contribution, we provide our views on FFT size.

- Discussion

FFT sizes

The selection of FFT sizes needs to meet the following two conditions:

- The sampling frequency () needs to be larger than or equal to the channel bandwidth (). So we have.

- The FFT sizes () needs to be the nearest power of two of.

Based on above two conditions, the FFT sizes can be calculated using the following equation 1.

(Equation 1)

Where,

nextpow2(N) returns the first P such that 2.^P >= abs(N).

The FFT sizes for FR2-2 are shown in the following table 2-1 using equation 1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

Proposal 1: To adopt the FFT sizes for FR2-2 in the following table 2-1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

- Conclusion

This contribution provides analysis on FFT sizes for 52.6-71GHz. The following proposals are concluded as follows:

Proposal 1: To adopt the FFT sizes for FR2-2 in the following table 2-1.

Table 2-1: FFT sizes for FR2-2

| SCS (kHz) | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| --- | --- | --- | --- | --- | --- |
|  | FFT size | FFT size | FFT size | FFT size | FFT size |
| 120 | 1024 | 4096 | N/A | N/A | N/A |
| 480 | N/A | 1024 | 2048 | 4096 | N/A |
| 960 | N/A | 512 | 1024 | 2048 | 4096 |

- References

- R4-2207216，WF on >52.6 GHz BS RF Tx Requirements，Nokia, Nokia Shanghai Bell，RAN4#102-e

- R4-2207445, Email discussion summary for [102-e][312] NR_exto71GHz_BSRF, Moderator (Nokia), RAN4#102-e

- R4-2206536, WF on 60 GHz UE RF, Qualcomm Incorporated, RAN4#102-e