3GPP TSG-RAN WG4 Meeting #103-e                                                                  R4-2208439

Online, 9th - 20th, May, 2022

Source:	Samsung, Spark

Title:	TP for TR 38.717-03-01: CA_n5-n40-n78

Agenda item:			8.6.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-01 to include CA_n5-n40-n78 according to the request in [1].

The fallback mode CA_n5-n40 is proposed in this meeting as R4-2208438.

2. Reference

- R4-2203966, Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL.

# 3. Text Proposal

<Start of Text Proposal>

6.x	CA_n5-n40-n78

6.x.1	Operating bands for CA

Table 6.x.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n5-n40-n78 | n5, n40, n78 |

6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- |
| CA_n5A-n40A-n78A | - | n5 | 5, 10, 15, 20, 251 |  | 0 |
|  |  | n40 | 5x, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100 |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink. NOTE x: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. |  |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 [3] and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

6.x.3	Co-existence studies

Co-existence studies can be omitted because fallback combinations cover these requirements and analysis

6.x.4	∆TIB,c and ∆RIB,c values

For CA_n5-n40-n78, the TIB,c and RIB,c values are defined as below.

Table 6.X.4-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n40-n78 | n5 | 0.6 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 6.X.4-2: ΔRIB,c  due to NR CA (three bands)

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n40-n78 | n5 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

6.x.5	REFSENS requirements

MSD requirements are captured in lower order combinations.

<End of Text Proposal>