# 3GPP TSG-RAN WG4 Meeting #103-e	R4-2208620

# Electronic Meeting, 9th - 20th May, 2022

Source: 	vivo

Title: 	Discussion on remaining Rx RF requirements for FR2-2

Agenda Item:	9.15.3.2

Document for:	Approval

Introduction

In the last meeting, we made great progress on REFSENS and EIS spherical coverage for PC1, PC2 and PC3 [1]. However, there are many UE Rx requirements left unfinished. In this contribution, we further discussed the UE Rx requirements for FR2-2.

Discussion

In RAN#95E, the following remaining issues in RAN4 are identified in the exception sheet for 52.6G~71GHz WI.

Core requirements including channel raster, sync raster, MPR, A-MPR, SEM, Spurious, Blocking, CA, ACS, Max input power, FRCs, Mandatory/optional UE channel bandwidths

In the following discussion, we focus on the remaining Rx RF requirements for FR2-2.

Reference sensitivity for FR2-2

In the last meeting, we agreed to adopt -79 dBm for PC1 REFSENS, -80.3 dBm for PC2 REFSENS, -72 dBm for PC3 REFSENS for 400MHz channel bandwidth in band n263. For other channel bandwidths, the REFSENS values can be obtained by scaling the REFSENS for 400MHz. The reference sensitivity for PC1, PC2, PC3 in band n263 is proposed as Table 1:

Table 1: Reference sensitivity for power class 1, 2, 3

| Operating band | Power Class (UE type) | REFSENS (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n263 | PC1 | -85.0 | -79.0 | -76.0 | -73.0 | -72.0 |
|  | PC2 | -86.3 | -80.3 | -77.3 | -74.3 | -73.3 |
|  | PC3 | -78.0 | -72.0 | -69.0 | -66.0 | -65.0 |

In the last meeting, we agreed to use the spherical coverage drops from each power class to determine the EIS spherical coverage. For PC1, 11.5 dB gain drop was adopted at 85%-tile CDF for spherical coverage; For PC2, 15.1 dB gain drop was adopted at 60%-tile CDF for spherical coverage; For PC3, 11.8 dB gain drop was adopted at 50%-tile CDF for spherical coverage. Thus, EIS spherical coverage can be specified as follows:

Table 2: EIS spherical coverage for power class 1,2,3

| Operating band | Power Class (UE type) | Spherical Coverage | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n263 | PC1 | EIS at 85th %-tile CCDF (dBm) | -73.5 | -67.5 | -64.5 | -61.5 | -60.5 |
|  | PC2 | EIS at 60th %-tile CCDF (dBm) | -71.2 | -65.2 | -62.2 | -59.2 | -58.2 |
|  | PC3 | EIS at 50th %-tile CCDF (dBm) | -66.2 | -60.2 | -57.2 | -54.2 | -53.2 |

ACS for FR2-2

For UE ACS, it was agreed to adopt 21 dB for the requirement out of the conclusion from co-existence study. Thus, we proposed the following table as UE ACS requirement:

Table 3: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| n263 | dB | 21 | 21 | 21 | 21 | 21 |

For UE ACS test parameters, Case 1 and Case 2, they are in reference with FR2-1.

Table 4: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| PInterferer for band n263 | dBm | REFSENS +33.5 dB | REFSENS +33.5 dB | REFSENS +33.5 dB | REFSENS +33.5 dB | REFSENS +33.5 dB |
| BWInterferer | MHz | 100 | 400 | 800 | 1600 | 2000 |
| FInterferer (offset) | MHz | 100 / -100 | 400 / -400 | 800 / -800 | 1600 / -1600 | 2000 / -2000 |

Table 5: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 100 MHz | 400 MHz | 800 MHz | 1600 MHz | 2000 MHz |
| Power in Transmission Bandwidth Configuration for band n263 | dBm | -44.5 | -44.5 | -44.5 | -44.5 | -44.5 |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz | 100 | 400 | 800 | 1600 | 2000 |
| FInterferer (offset) | MHz | 100 / -100 | 400 / -400 | 800 / -800 | 1600 / -1600 | 2000 / -2000 |

Conclusion

This contribution further discusses UE Rx RF requirements for FR2-2. The REFSENS and ACS requirements are proposed in this contribution.

- References

[1] R4-2206536, WF on 60 GHz UE RF, Qualcomm Incorporated, RAN4#102-e.