3GPP TSG-RAN WG4 Meeting #103-e                                                             R4-2208847

Electronic Meeting, May 09th – May 20th, 2022

Source:	Samsung

Title:	Discussion on RRM Performance Requirements for FR2 HST

Agenda item:			9.8.3

Document for:	Discussion

# 1. Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [1, RP-202118] (which has been further revised to [2, RP-210800]).  On the RAN#95-e, the core requirement of FR2 HST has been completed [3] [4].

Accordingly, the performance part of RRM requirement shall begin from RAN4#103-e, for the following objectives on RRM performance requirements captured in WID [2].

- Specify the RRM performance requirements of measurement accuracy if identified. - Specify the RRM test cases related to new core requirements.

In this contribution, we would like to provide our viewpoints on RRM performance requirement for FR2 HST.

# 2 Discussion

## 2.1 Summary of Impacted RRM Core Requirements

Based on the approved CR [5, R4-2206016] and the corresponding agreement for FR2 HST, the impacted RRM core requirements can be summarized as below.

Observation 1: The impacted RRM core requirements for FR2 HST can be summarized as below:

| TS 38.133 section number | TS 38.133 section title | Summary of Impacted Core Requirement |
| --- | --- | --- |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.2 | Cell Re-selection | Cell re-selection for FR2 PC6 |
| 6 | RRC_CONNECTED state mobility |  |
| 6.2 | RRC Connection Mobility Control | Re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 |
| 7 | Timing |  |
| 7.1.2.1 | Gradual timing adjustment | Tq for FR2 PC6 |
| 7.1.2.3 | One shot large UL timing adjustment for FR2 Power Class 6 UE | New requirement for FR2 PC6 |
| 8 | Signalling characteristics |  |
| 8.1.2 | Requirements for SSB based radio link monitoring | SSB based RLM requirement for FR2 PC6 |
| 8.5.2 | Requirements for SSB based beam failure detection | SSB based BFD requirement for FR2 PC6 |
| 8.10.3A | MAC-CE based TCI state switch delay in HST FR2 scenarios | MAC-CE based TCI state switch delay for FR2 PC6 |
| 9 | Measurement Procedure |  |
| 9.2.5 | NR intra-frequency measurements without measurement gaps | NR intra-frequency measurements without measurement gaps for FR2 PC6 |
| 9.5 | L1-RSRP measurements for Reporting | SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6 |
| 9.8 | L1-SINR measurements for Reporting | L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6 |

## 2.2 Planning on RRM Performance Test Cases

Based on the above summary, the following test cases are expected to be modified and introduced for FR2 HST performance requirement as below.

Proposal 1: The RRM performance test cases are expected to be introduced or modified for the corresponding core reuiqrement as below:

| TC Index | Necessity of New Test Case | Corresponding Core Requirement | Core CR Responsibility | TC CR Responsibility |
| --- | --- | --- | --- | --- |
| TC1 | Reuse test case of existing TC A.7.1.1.1, but updated detection and evaluation periods for PC6 UE. | 4.2, for cell re-selection for FR2 PC6 | ZTE |  |
| TC2 | Reuse test case of existing TC A.7.3.2.1.1, but updated time period for re-establishment for PC6 UE | 6.2 for re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 | Ericsson |  |
|  | No new test cases are needed. Note: FR2 PC6 requirement can be verified by existing TC A.7.4.1.1 | 7.1.2.1 for Tq for FR2 PC6 | Nokia |  |
| TC3 | New test case is needed for one shot large UL timing adjustment | 7.1.2.3 for new requirement for FR2 PC6 | Nokia |  |
| TC4-TC7 | Reuse test case of existing TC A.7.5.1.1 – A.7.5.1.4, but updated time periods for PC6 UE. | 8.1.2 for SSB based RLM requirement for FR2 PC6 | CATT |  |
| TC8-TC9 | Reuse test cases of the existing TC A.7.5.5.1 – A.7.5.5.2, but updated time periods for PC6 UE. | 8.5.2 for SSB based BFD requirement for FR2 PC6 | CATT |  |
| TC10 | New test case is needed for MAC CE based TCI state switching delay for FR2 PC6 | 8.10.3A for MAC-CE based TCI state switch delay for FR2 PC6 | Samsung |  |
| TC11-TC12 | Reuse test case of existing TC A.7.6.1.1 – A.7.6.1.2, but updated time periods for PC6 UE. | 9.2.5 for NR intra-frequency measurements without measurement gaps for FR2 PC6 | Nokia |  |
| TC13-TC14 | Reuse test case of existing TC A.7.6.3.1 – A.7.6.3.2, but updated time periods for PC6 UE. | 9.5 for SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6 | ZTE |  |
| TC15 | Reuse test case of existing TC A.7.6.6.2, but updated time periods for PC6 UE. | 9.8 for L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6 | Huawei |  |

Accordingly, the work-split planning colunm is provided, which is used to trigger the discussion on collecting companies’ preference to be the CR drafter for each test case.

# 3. Conclusion

In this contribution, we provided our viewpoints on RRM performance requirement for FR2 HST, accordingly the following observations and proposals are obtained:

Observation 1: The impacted RRM core requirements for FR2 HST can be summarized as below:

| TS 38.133 section number | TS 38.133 section title | Summary of Impacted Core Requirement |
| --- | --- | --- |
| 4 | SA: RRC_IDLE state mobility |  |
| 4.2 | Cell Re-selection | Cell re-selection for FR2 PC6 |
| 6 | RRC_CONNECTED state mobility |  |
| 6.2 | RRC Connection Mobility Control | Re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 |
| 7 | Timing |  |
| 7.1.2.1 | Gradual timing adjustment | Tq for FR2 PC6 |
| 7.1.2.3 | One shot large UL timing adjustment for FR2 Power Class 6 UE | New requirement for FR2 PC6 |
| 8 | Signalling characteristics |  |
| 8.1.2 | Requirements for SSB based radio link monitoring | SSB based RLM requirement for FR2 PC6 |
| 8.5.2 | Requirements for SSB based beam failure detection | SSB based BFD requirement for FR2 PC6 |
| 8.10.3A | MAC-CE based TCI state switch delay in HST FR2 scenarios | MAC-CE based TCI state switch delay for FR2 PC6 |
| 9 | Measurement Procedure |  |
| 9.2.5 | NR intra-frequency measurements without measurement gaps | NR intra-frequency measurements without measurement gaps for FR2 PC6 |
| 9.5 | L1-RSRP measurements for Reporting | SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6 |
| 9.8 | L1-SINR measurements for Reporting | L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6 |

Proposal 1: The RRM performance test cases are expected to be introduced or modified for the corresponding core reuiqrement as below:

| TC Index | Necessity of New Test Case | Corresponding Core Requirement | Core CR Responsibility | TC CR Responsibility |
| --- | --- | --- | --- | --- |
| TC1 | Reuse test case of existing TC A.7.1.1.1, but updated detection and evaluation periods for PC6 UE. | 4.2, for cell re-selection for FR2 PC6 | ZTE |  |
| TC2 | Reuse test case of existing TC A.7.3.2.1.1, but updated time period for re-establishment for PC6 UE | 6.2 for re-establishment to NR intra-frequency cell delay requirement for FR2 PC6 | Ericsson |  |
|  | No new test cases are needed. Note: FR2 PC6 requirement can be verified by existing TC A.7.4.1.1 | 7.1.2.1 for Tq for FR2 PC6 | Nokia |  |
| TC3 | New test case is needed for one shot large UL timing adjustment | 7.1.2.3 for new requirement for FR2 PC6 | Nokia |  |
| TC4-TC7 | Reuse test case of existing TC A.7.5.1.1 – A.7.5.1.4, but updated time periods for PC6 UE. | 8.1.2 for SSB based RLM requirement for FR2 PC6 | CATT |  |
| TC8-TC9 | Reuse test cases of the existing TC A.7.5.5.1 – A.7.5.5.2, but updated time periods for PC6 UE. | 8.5.2 for SSB based BFD requirement for FR2 PC6 | CATT |  |
| TC10 | New test case is needed for MAC CE based TCI state switching delay for FR2 PC6 | 8.10.3A for MAC-CE based TCI state switch delay for FR2 PC6 | Samsung |  |
| TC11-TC12 | Reuse test case of existing TC A.7.6.1.1 – A.7.6.1.2, but updated time periods for PC6 UE. | 9.2.5 for NR intra-frequency measurements without measurement gaps for FR2 PC6 | Nokia |  |
| TC13-TC14 | Reuse test case of existing TC A.7.6.3.1 – A.7.6.3.2, but updated time periods for PC6 UE. | 9.5 for SSB based L1-RSRP measurement, and scheduling restriction for FR2 PC6 | ZTE |  |
| TC15 | Reuse test case of existing TC A.7.6.6.2, but updated time periods for PC6 UE. | 9.8 for L1-SINR measurement with SSB based CMR and dedicated IMR configured, and scheduling restriction for FR2 PC6 | Huawei |  |

# 5. Reference

[1] RP-202118, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia.

[2] RP-210800, “Revised WID on NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3] RP-220925, “Status report for WI NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[4] RP-220316, “WI summary for NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[5] R4-2206016, “Big CR to TS 38.133 on HST FR2 RRM Core Requirements”, Nokia, Nokia Shanghai Bell.