3GPP TSG-RAN WG4 Meeting #103-e	R4-2209071

Electronic Meeting, 9 May – 20 May 2022

Agenda Item:	9.8.4.1

Source:	Ericsson

Title:	PDSCH demodulation requirements for HST FR2

Document for:	Discussion

# 1	Introduction

RAN4#102-e agreed with the draft CR on the introduction of PDSCH demodulation requirements for HST FR2 [1]. This contribution proposes the Noc level applicable for PC6 UE.

This contribution also shows our simulation results based on the simulation assumption [2].

# 2	Noc level

RAN4 introduced a new UE power class 6 assuming train mounted device and RAN4 also agreed the PDSCH demodulation requirements defined for HST FR2 are appliable for UE capable of power class 6 [1]. Since Noc level for FR2 demodulation requirements depends on the UE power class according to TS38.101-4 4.5.3.2, RAN4 need to specify the Noc level for PC6. Table 1 shows Noc values derived from REFSENS values specified in TS38.101-2 Table 7.3.2.6-1, according to TS38.101-4 4.5.3.3. We propose to specify the Noc values in TS38.101-4 Table 4.5.3.2-1.

Table 1	Noc power level for PC6.

| Operating bands | Reference sensitivity (TS 38.101-2 Table 7.3.2.6-1, 50MHz) | Noc values |
| --- | --- | --- |
| n257 | -92.6 | -162.4 |
| n258 | -92.8 | -162.6 |
| n261 | -92.6 | -162.4 |

# 3	Simulation results

Table 2 summarizes our simulation results PDSCH demodulation for FR2 HST-DPS according to [2]

Table 2	Simulation results of PDSCH for HST-DPS in FR2.

| Test number | SNR to achieve 70% of the maximum throughput (dB) (Alignment results) | SNR to achieve 70% of the maximum throughput (dB) (Impairment results) |
| --- | --- | --- |
| 1-1: HST-DPS-FR2-BI-B | 10.5 | 12.5 |
| 1-2: HST-DPS-FR2-UNI-A | 10.5 | 12.5 |

# 4	Summary

Proposal 1: Specify Noc power level for PC6 in TS38.101-4 Table 4.5.3.2-1 as follows:

| Operating bands | n257 | n258 | n261 |
| --- | --- | --- | --- |
| Noc level (dBm/Hz) | -162.4 | -162.6 | -162.4 |

# References

- R4-2207232, “Big draftCR for 38.101-4: FR2 HST”, Samsung.

- R4-2207231, “Simulation assumption for PDSCH requirement”, Intel Corporation.