3GPP TSG-RAN WG4 Meeting #103-e	R4-2209286

# Electronic Meeting, 9 - 20 May, 2022

Source:	Huawei, HiSilicon

Title:	TP for 38.717-02-01 CA_n1A-n78C with UL_n78C

Agenda item:	8.5.2

Document for:	Approval

1 Background

This contribution provides updated text proposal on the NR CA band combination CA_n1A-n78C with UL_n78C.

2 Text Proposal

---Start of changes---

6.X	CA_n1-n78

6.X.1	Common for 1 band UL and 2 bands UL CA

6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination CA_n1-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination CA_n1-n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78C | CA_n78C | n1 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n78 | CA_n78C_BCS0 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n78 | CA_n78C_BCS0 |  |

6.X.1.3	UE Co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

Referring to clause 7.3.2.2 from TR 38.862, we can get the IMD order for contiguous UL CA as below.

WGap = lowest band edge of n78 - highest DL band edge of n1 = 3300 – 2170 = 1130 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 500) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 13.

Since IMD order is > 11, MSD can be ignored.

6.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB have been specified in current spec.

6.X.1.5	REFSENs requirements

There is no MSD exception requirement.

6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

---End of changes---

Reference

[1]	RP-211058, “Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation