3GPP TSG-RAN WG4 Meeting # 103-e              									     R4-2210004

Electronic Meeting, 9st of May – 20th of May, 2022

Agenda item:			9.12.3.3

Source:	THALES

Title:		Draft text proposal for Clause 7.3.2.2.5 Transmitter spurious emissions - TR38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 6 and Section 7 of TR 38.863. In this document, a TP is proposed to update some of the subsections related to RF requirements in Section 7.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.2.2.5 Transmitter spurious emissions  for TR 38.863 [1] taking into account latest agreements to be considered for TS 38.108.

Proposal 1: To update the TR with the attached TP

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with the attached TP

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

------------------------------------------------<Start of TP>-----------------------------------------------

##### 7.3.2.2.5	Transmitter spurious emission

Transmitter basic limits for spurious emissions are presented in Table 7.3.2.2.5-1

Table 7.3.2.2.5-1 SAN transmitter spurious emission limits

| Spurious frequency range | Basic limit GEO class (dBm) | Basic limit LEO class (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 30 MHz – 5th harmonic (NOTE 2, 4, 5) | When Prated,c,sys ≤ 47dBm : -13 When Prated,c,sys > 47dBm : Prated,c,sys-60 ( NOTE 3) |  | 4 kHz (NOTE 1) | See NOTES 1, 2, 3, 4 ,5 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [x], s4.1. NOTE 2: Lower and Upper frequency as in ITU-R SM.329 [x], s2.5, Table 1. Applies only for the fundamental frequency range 600 MHz - 5.2 GHz (e.g. for n255 and n256 frequency bands). NOTE 3: Values as in ITU-R SM.329 [x], Table 10. NOTE 4: Lower frequency limit is replaced by 800 MHz, according to ITU-R SM.329-12, when using waveguide section, spurious domain emission measurements below 0.7 times the waveguide cut-off frequency are not required. NOTE 5: This spurious frequency range applies only to SAN type 1-H. |  |  |  |  |

The ITU-R SM.329 [x] defines Unwanted emissions in the spurious domain. Measurement bandwidth is listed in section 4.1, lower and upper frequencies are specified in the Table 1 of section 2.5 and the values are taken from the table 10.

------------------------------------------------<End of TP>-------------------------------------------------