3GPP TSG-RAN WG4 Meeting #103-e	R4-2210614

E-meeting, 9th May 2022 - 20th May 2022

Title:	WF on NR SL enhancements RRM requirements

Source:	LG Electronics

Agenda item:	9.14.4/9.14.5/4.1.5.2(V2X related)

Document for:	Approval

WF – 1.1 Scheduling availability when switching TDM based intra-band con-current SL operation

Agreement

- Keep the existing scheduling availability requirements

- No further discussion

WF – 1.2 Condition to Relax SyncRef UE detection requirements for Asynchronized SLSS measurement & search

GTW Agreement

Keep existing requirements

WF – 1.3 SLSS evaluation period

Agreement

- Update the SLSS evaluation period as follows

| SL-DRX cycleNote 1 [ms] | Tevaluate,SLSS [ms] |
| --- | --- |
| No SL-DRX | 4 x S-SSB periods |
| SL-DRX cycle ≤ 160ms | 4 x S-SSB periods |
| SL-DRX cycle > 160ms | 4 x SL-DRX cycle |
| Note 1: If multiple SL-DRX cycles are configured for SL UE, the SL-DRX cycle in the requirement is the shortest one. When the shortest SL-DRX cycle UE used changes, the requirements do not apply to the time of transition. |  |

WF – 1.4 Avoidance of Interruption to WAN due to SL-DRX

Agreement

- No further discussion

WF – 1.5 Interruption requirement during switching between SL Tx and Uu

Agreement

- Revisit the interruption requirement during switching between SL Tx and Uu once CA on Uu for Uu-SL intra-band co-existence scenario is introduced in Rel-17 RF spec.

WF – 2.1 Test for SyncRef UE as synchronization reference source when SL-DRX is used

GTW agreement

- T1=3s, T2=5.24s, T3=5.24s

- SL-DRX cycle = 320ms

WF – 2.2 Test for Selection/Reselection of V2X SyncRef source under SL-DRX (GNSS/FR1 NR Cell as the highest priority)

Agreement

- SL-DRX cycle = 320ms

WF – 2.3 Test for V2X UE Autonomous Resource Selection/Reselection (Periodic Sensing/Contiguous Sensing/SL-DRX)

Agreement

- SL-DRX cycle = 40ms

- Draft CR of R4-2207738 is agreeable

WF – 2.4 Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case

Agreement

- SL-DRX cycle = 640ms

WF – 2.5 SL-DRX configuration : SL.DRX.x1

GTW agreement

- SL.DRX.1 = 40ms, SL.DRX.2 = 320ms, SL.DRX.3 = 640ms

WF – 3.1 Rel-14/16 maintenance

Agreement

- Draft CR of R4-2210086 is agreeable

- Draft CR of R4-2210084 is agreeable