3GPP TSG-RAN WG4 Meeting # 103-e              									     R4-2210865

Electronic Meeting, 9st of May – 20th of May, 2022                       (revision of R4-2210116)

Agenda item:			9.12.3.3

Source:	THALES

Title:	Draft pCR for Clause 7.3.2.2.4.2 Operating band unwanted emissions - TR 38.863

Document for:	Approval

# Introduction

During RAN WG4 Meeting RAN4#102-e, a work split has been proposed to complete Section 6 and Section 7 of TR 38.863. In this document, some material for the TR 38.863 OBUE Clause is proposed to update some of the subsections related to RF requirements.

# Discussion

In this contribution is proposed to add information with respect to Section 7.3.2.2.4.2 Operating band unwanted emissions of TR 38.863 [1] by including some information related to agreements for TS 38.108.

Proposal 1: To update the TR with attached TP.

# Conclusions

In this contribution, the following proposal is made:

Proposal 1: To update the TR with attached TP.

# Reference

[1]	TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

# Text proposals for TR 38.863

------------------------------------------------------<Start of TP>----------------------------------------------

7.3.2.2.4.2		Operating band unwanted emissions

The SAN Operating Band Unwanted Emissions (OBUE) requirements for GEO and LEO classes are therefore defined as described in Table 7.3.2.2.4.2-1.

Table 7.3.2.2.4.2-1: SAN OBUE basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits dBm | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2* BWChannel | 0.002 MHz f_offset < 2× BWChannel + 0.002 MHz | maxSE limit, PSDchannel –ΔSat_TypedB- 40×log10 f_offset-0.002BWChannel*2+1dBm | 4kHz |
| NOTE 1: PSDchannel = Prated,c – 10log10(BWChannel) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause. NOTE 3: PSD attenuation as in ITU-R SM.1541-6, Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_Type=0 dB for GEO class and ΔSat_Type=3 dB for LEO class. |  |  |  |

ITU-R SM.1541-6 [x] specify the OoB mask requirements. The OoB domain emission as attached Annex 5 should be attenuated below the maximum psd by the following and the attenuation can’t be greater than the spurious emission limit.

The OoB domain emissions of a station operating in the bands allocated to the FSS and MSS should be attenuated below the maximum psd, in a reference bandwidth of 4 kHz (for systems operating above 15 GHz a reference bandwidth of 1 MHz may be used in place of 4 kHz) within the necessary bandwidth, by the following:

where F is the frequency offset from the edge of the total assigned band, expressed as a percentage of necessary bandwidth. It is noted that the OoB emission domain starts at the edges of the total assigned band.

We can get the “F” = Frequency offset / channel bandwidth × 100 as above. Thus, we can specify the OoB mask requirements as below based on the ITU regulation when we use f_offset and BWchannel to replace “F”.

----------------------------------------------------<End of TP >---------------------------------------------------