3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214037

# Electronic Meeting, August 15 – August 26, 2022

Source: 	Huawei

Title: 	TP to TS 38.114: exclusion bands, performance assessment, performance criteria (4.4, 5.1, 5.2, 6.1, 6.2)

Agenda Item:		9.5.4

Document for:	Approval

# Introduction

During RAN4#103-e meeting, work-split for the completion of the TS 38.114 was agreed in [1], as extracted in the table below.

In this contribution we provide TP to TS 38.114 for the following sections, as per the agreed work-split:

- 4.4 Exclusion bands

- 5.1 Performance assessment: General

- 5.2 Performance assessment: NR repeater

- 6.1 Performance criteria for continuous phenomena for BS

- 6.2 Performance criteria for transient phenomena for BS

Agreement on Work split： TS38.114 work split for next meeting (i.e. Aug meeting #104 meeting) Clauses in TS38.114; Status; Work Split. 1 Scope; Completed. 2 Reference; Completed (completed in R4-2202986). 3 Definitions, symbols and abbreviations; Completed (completed in R4-2204358). Section 4: Test condition. 4.1 General; Ongoing; ZTE. 4.2 Arrangements for establishing a communication work; Ongoing; ZTE. 4.3 Narrow band responses on receivers; Completed. 4.4 Exclusion bands; Ongoing; Huawei. 4.5 NR repeaters test configurations; Ongoing; Ericsson. Section 5: Performance assessment. 5.1 General; Ongoing Ongoing; Huawei. 5.2 NR repeater. 5.3 Ancillary equipment; Completed. Section 6: Performance criteria. 6.1 Performance criteria for continuous phenomena for BS; Ongoing Ongoing; Huawei. 6.2 Performance criteria for transient phenomena for BS. 6.3 Performance criteria for continuous phenomena for Ancillary equipment; Completed. 6.4 Performance criteria for transient phenomena for Ancillary equipment; Completed. Section 7: Applicability overview. 7.1 Emission; Completed. 7.2 Immunity; Completed. Section 8: Emission. 8.1 Test configurations; Ongoing; Nokia. 8.2 Radiated emission; Ongoing; Nokia. 8.3 Conducted emission DC power input/output port; Completed. 8.4 Conducted emissions, AC mains power input/output port; Completed. 8.5 Conducted emissions, telecommunication port; Completed. 8.6 Harmonic Current emissions (AC mains input port); Completed. 8.7 Voltage fluctuations and flicker (AC mains input port); Completed. Section 9: Immunity. 9.1 Test configurations; Ongoing; ZTE. 9.2 RF electromagnetic field (80MHz - 6000MHz); Completed. 9.3 Electrostatic discharge; Completed. 9.4 Fast transient common mode; Completed. 9.5 RF common mode (0.15 MHz - 80 MHz); Completed. 9.6 Voltage dips and interruptions; Completed. 9.7 Surges, common and differential mode; Completed. Moderator recommendation: Encourage companies to bring a single TP to TS38.114 per company for all the assigned sections in the next meeting. For example, sub-clause 4.1/4.2 and clause 9.1 are assigned for company A, then capturing both sub-clause 4.1/4.2 and clause 9.1 in one TP under the sub-agenda above in next meeting is highly recommended. Please strictly abide by the rule announced by Chair, which is: one TP per one agenda item per company.

# Discussion

The following was considered while drafting the TP:

- Exclusion zones:

Referring to the TS 38.106, the structure of the RF requirements differ from NR BS significantly, i.e. there is no usual breakdown to Tx and Rx requirements, e.g. only Rx spur emissions requirement is defined for the NR repeater’s receiver. Still, considering that specifics of the RI testing, the definitions of the Rx exclusion band is considered to be valid also for NR repeater. As there is no “ΔfOOB” defined in the NR repeater specification, the definition of the Rx exclusion zone needs to be adjusted compared to the NR BS EMC spec TS 38.113. Furthermore, please note that the E-UTRA repeater EMC specification also considers both the Tx and Rx exclusion zones.

- Performance criteria: text was aligned with the performance criteria for EUTRA repeaters in TS 36.113.

- Assessment of performance

Referring to the WF in R4-2210628, the criteria for performance assessment for NR Repeater EMC testing was still not decided. As extracted below:

The agreements for the following open issues are as follows: Issue 2-1 Could throughput be used as performance assessment for NR Repeater EMC? - Agreement 1: Throughput can not be used as the performance assessment for NR Repeater EMC. Issue 2-2 Which candidate criteria for performance assessment should be adopted for NR Repeater EMC testing? Option 1: Gain; Option 2: Power accuracy; Option 3: Others are not precluded; - Agreement 2: Require further discussion. FFS

For sake of alignment with the EUTRA repeater EMC specification, the Option 1 was followed in the TP to TS 38.114, i.e. gain was used as the performance criteria. This is subject to the final decision on the candidate criteria for performance assessment of the NR Repeater.

# Conclusions

Based on the previous discussions and the work-split agreed in [1], the following proposal is formulated:

Proposal 1: Approve the attached TP to TS 38.114, covering the following sections:

- 4.4 Exclusion bands

- 5.1 Performance assessment: General

- 5.2 Performance assessment: NR repeater

- 6.1 Performance criteria for continuous phenomena for BS

- 6.2 Performance criteria for transient phenomena for BS

# References

[1]		R4-2210506	Email discussion summary for [103-e][303] NR_EMC, RAN4#103-e

# Annex A: TP to TS 38.114

------------------------------ Modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

- 3GPP TS 38.106: "NR Repeater Radio Transmission and Reception"

- 3GPP TS 38.115-1: "NR; Repeater conformance testing - Part 1: Conducted conformance testing"

- 3GPP TS 38.115-2: "NR; Repeater conformance testing - Part 1: Radiated conformance testing"

- CISPR 32: "Electromagnetic compatibility of multimedia equipment - Emission requirements".

- IEC 61000-6-1: "Electromagnetic compatibility (EMC) - Part 6-1: Generic standards - Immunity for residential, commercial and light-industrial environments".

- IEC 61000-6-3: "Electromagnetic compatibility (EMC) - Part 6-3: Generic standards - Emission standard for residential, commercial and light-industrial environments".

- IEC 61000-3-2: "Electromagnetic compatibility (EMC) - Part 3-2: Limits - Limits for harmonic current emissions (equipment input current ≤ 16 A per phase)".

- IEC 61000-3-3: "Electromagnetic compatibility (EMC) - Part 3-3: Limits - Limitation of voltage changes, voltage fluctuations and flicker in low-voltage supply systems, for equipment with rated current ≤ 16 A per phase and not subject to conditional connection".

- IEC 61000-3-11: "Electromagnetic compatibility (EMC) - Part 3-11: Limits – Limitation of voltage changes, voltage fluctuations and flicker in low-voltage supply systems -  Equipment with rated current ≤ 75 A and subject to conditional connections".

- IEC 61000-3-12: "Electromagnetic compatibility (EMC) - Part 3-12: Limits - Limits for harmonic currents produced by equipment connected to public low-voltage system with input current >16 A and ≤ 75 A per phase".

- IEC 61000-4-2: "Electromagnetic compatibility (EMC) - Part 4-2: Testing and measurement techniques - Electrostatic discharge immunity test".

- IEC 61000-4-3: "Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test".

- IEC 61000-4-4: "Electromagnetic compatibility (EMC) - Part 4-4: Testing and measurement techniques - Electrical fast transient/burst immunity test".

- IEC 61000-4-5: "Electromagnetic compatibility (EMC) - Part 4-5: Testing and measurement techniques - Surge immunity test".

- IEC 61000-4-6: "Electromagnetic compatibility (EMC) - Part 4-6: Testing and measurement techniques - Immunity to conducted disturbances, induced by radio frequency fields".

- IEC 61000-4-11: "Electromagnetic compatibility (EMC) - Part 4-11: Testing and measurement techniques - Voltage dips, short interruptions and voltage variations immunity tests".

- IEC 61000-4-21: "Electromagnetic compatibility (EMC) - Part 4-21: Testing and measurement techniques - Reverberation chamber test methods".

- ITU-R SM.329: "Unwanted emissions in the spurious domain".

[20]	IEC 60050-161: "International Electrotechnical Vocabulary - Chapter 161: Electromagnetic compatibility".

[21]	ETSI EN 301 489-1: "Electromagnetic Compatibility (EMC) standard for radio equipment and services - Part 1: Common technical requirements - Harmonised Standard for Electromagnetic Compatibility ".

[22]	3GPP TS 38.101-4

------------------------------ Next modified section ------------------------------

## 4.4	Exclusion bands

### 4.4.1	Transmitter exclusion band

The transmitter exclusion band for NR repeater is the frequency range over which no tests of radiated immunity of a transmitter are made. The transmitter exclusion band is defined as:

FDL,low – ΔfOBUE<f < FDL,high + ΔfOBUE

Where:

Values of FDL,low and FDL,high are defined for each operating band in TS 38.104 [2], clause 5.2.

The value of ΔfOBUE is derived considering the width of the operating band, and is defined in TS 38.106 [2], clause 6.5.1 for repeater type 1-C, and in in TS 38.106 [2], clause 7.5.1 for repeater type 2-O.

For NR repeater capable of multi-band operation, the total transmitter exclusion band is a combination of the exclusion bands for each operating band supported by NR repeater.

### 4.4.2	Receiver exclusion band

The receiver exclusion band for NR repeater is the frequency range over which no tests of radiated immunity of a receiver are made. The receiver exclusion band is defined as:

FUL,low – ΔfOOB<f < FUL,high + ΔfOOB

Where:

Values of FUL,low and FUL,high are defined for each operating band in TS 38.106 [2], clause 5.2.The ΔfOOB values for Repeater type 1-C and Repeater type 2-O is defined in table 4.4.2-1.

Table 4.4.2-1: ΔfOOB offset values for NR repeater 1-C and repeater 2-O

| NR repeater type | Operating band characteristics | ΔfOOB (MHz) |
| --- | --- | --- |
| Repeater type 1-C | FUL,high – FUL,low < 100 MHz | 20 |
|  | 100 MHz ≤ FUL,high – FUL,low ≤ 900 MHz | 60 |
| Repeater type 2-O | FUL_high – FUL_low ≤ 4000 MHz | 1500 |

In case the spatial exclusion zone (as depicted in figure 9.2.2-1) is not used during the EMC RI testing, the receiver exclusion band for NR repeater type 1-O is defined as:

FUL,low – ΔfRIexclusion < f < FUL,high + ΔfRIexclusion

Where the values of ΔfRIexclusion are defined in table 4.4.2-2.

Table 4.4.2-2: Maximum ΔfRIexclusion offset outside the uplink operating band

| Operating band characteristics | ΔfRIexclusion (MHz) |
| --- | --- |
| 100 MHz ≥ FUL,high – FUL,low | 60 |
| 100 MHz < FUL,high – FUL,low | 200 |

For NR repeater capable of multi-band operation, the total receiver exclusion band is a combination of the exclusion bands for each operating band supported by NR repeater.

------------------------------ Next modified section ------------------------------

# 5	Performance assessment

## 5.1	General

The following information shall be recorded in or annexed to the test report:

-	The primary functions of the radio equipment to be tested during and after the EMC testing;

-	The intended functions of the radio equipment which shall be in accordance with the documentation accompanying the equipment;

-	The method to be used to verify that a communications link is established and maintained;

-	The user-control functions and stored data that are required for normal operation and the method to be used to assess whether these have been lost after EMC stress;

-	The ancillary equipment to be combined with the radio equipment for testing (where applicable);

-	The information about ancillary equipment intended to be used with the radio equipment;

-	Information about the common and/or band-specific active RF components and other hardware blocks for a communication link in EUT capable of multi-band operation;

-	An exhaustive list of ports (or RIBs), classified as either power or signal/control. Power ports shall further be classified as AC or DC power.

Performance assessment of a NR repeater with multiple enclosures may be done separately, according to the manufacturer's choice.

A communication link used by more than one operating band, shall be assessed on all operating bands. Communication link(s) and/or radio performance parameters for the operating bands can during the test be assessed simultaneously or separately for each band, depending on the test environment capability.

## 5.2	NR repeaters

The parameter used for assessment of performance of a NR repeater is the [gain] within the operating band.

## 5.3	Ancillary equipment

At the manufacturer's discretion the test may be performed on the ancillary equipment separately or on a representative configuration of the combination of radio and ancillary equipment. In each case EUT is tested against all applicable immunity and emission clauses of the present document and in each case, compliance enables the ancillary equipment to be used with different radio equipment.

------------------------------ Next modified section ------------------------------

# 6	Performance criteria

## 6.1	Performance criteria for continuous phenomena for NR repeaters

The gain of the EUT shall be measured throughout the period of exposure of the phenomenon. The gain measured during the test shall not change from the gain measured before the test by more than ± 1 dB. At the conclusion of the test the EUT shall operate as intended with no loss of user control functions or stored data.

## 6.2	Performance criteria for transient phenomena for NR repeaters

The gain of the EUT shall be measured before the test and after each exposure. At the conclusion of each exposure the gain of the EUT shall not have changed by more than ± 1 dB. At the conclusion of the total test comprising the series of individual exposures, the EUT shall operate as intended with no loss of user control functions or stored data, as declared by the manufacturer, and the gain of the EUT shall not have changed by more than ± 1 dB.

------------------------------ End of modified section ------------------------------