3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, August 15 - 26, 2022

Title:	Reply LS on UL Tx switching across 3 or 4 bands

Response to:	R1-2205502, LS on UL Tx switching across 3 or 4 bands in Rel-18

Release:		Release 18

Work Item:	NR_MC_enh-Core

Source:	RAN4

To:	RAN1

CC:                         	RAN2

Contact person:	Shan YANG

yangshan@chinatelecom.cn

Attachments:	none

- 1 Overall description

RAN4 would like to thank RAN1 for their LS in R1-2202927. For UL Tx switching across 3 or 4 bands in single TAG scenario, RAN4 has reached the following agreements regarding the RAN1 questions:

RAN1 Question:

RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands.

RAN4 Answer:

On the length of switching period:

- For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL.

- The length of switching period is applied per band pair for each band combination.

- For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17.

  - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

Moderator’s note (will be removed from the final LS): This bullet is in general agreeable in round 1, and one sub-bullet is added for clarification based on Nokia comment and the following text from 38.214 and 38.331: 6.1.6 Uplink switching (38.214 Rel-17) The UE may omit uplink transmission during the uplink switching gap if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching. The switching gap is indicated by UE capability uplinkTxSwitchingPeriod2T2T if uplinkTxSwitching-2T-Mode is configured, and uplinkTxSwitchingPeriod otherwise 6.3.2 Radio resource control information elements (38.331 Rel-17) uplinkTxSwitching-2T-Mode Indicates 2Tx-2Tx switching mode is configured for inter-band UL CA or SUL, in which the switching gap duration for a triggered uplink switching (as specified in TS 38.214 [19]) is equal to the switching time capability value reported for the switching mode. If this field is absent and uplinkTxSwitching is configured, it is interpreted that 1Tx-2Tx UL Tx switching is configured as specified in TS 38.214 [19].…

- While the set of switching periods is the same as in Rel-16/17, RAN4 has not concluded on whether the same or a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands.

On the UE complexity:

RAN4 has not identified any complexity issue or technical difficulty for UE to realize Tx switching across 3 or 4 bands. Meanwhile, to keep the UE complexity from UE memory perspective, it was raised by one company that the sharing of UE memory across bands and the corresponding flushing & reloading time need to be taken into account. It has not been concluded in RAN4 on whether or not the UE memory sharing is a common implementation for different UEs as well as whether or not it has impact on the preparation time. Considering that the UE preparation time is specified in RAN1 specification, RAN4 would like to recommend the UE memory sharing issue to be further discussed in RAN1.

RAN1 Question:

RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

- “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

RAN4 Answer:

RAN4 has discussed the UE assumption/behavior considering two cases:

- Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B.

For Case 1, RAN4 agreed that the other Tx chain is not expected to be used for transmission during the switching period.

- Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).

For Case 2, RAN4 agreed that, as baseline UE assumption, the other Tx chain is not expected to be used for transmission on band C during the switching period. RAN4 will further discuss the feasibility that the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

RAN4 also discussed other related issues for UL Tx switching across 3 or 4 bands in single TAG scenario, and reached the following agreements:

For concurrent UL transmission on 2 bands:

For UL Tx switching across 3 and 4 bands, the support of concurrent UL transmission on 2 (out of 3 or 4) bands at least requires UL CA support on the corresponding band pair(s) by the UE.

For the number of bands supporting 2Tx:

From RAN4 perspective, it was recommended that at least 1 band out of 3 or 4 bands shall support 2Tx. In RAN4 understanding, the final decision on the minimum and maximum numbers of bands that a UE shall support 2Tx across the bands is up to RAN1.

- 2 Actions

To 3GPP RAN1

ACTION: RAN4 respectfully asks RAN1 to take into account the above information in their work.

- 3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #104-e-bis		10-19 October, 2022		 E-meeting

TSG-RAN WG4 Meeting #105			11-14 November, 2022		 TBD