3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, August 15 - 26, 2022

Title:	Reply LS on UL Tx switching across 3 or 4 bands

Response to:	R1-2205502, LS on UL Tx switching across 3 or 4 bands in Rel-18

Release:		Release 18

Work Item:	NR_MC_enh-Core

Source:	RAN4

To:	RAN1

CC:                         	RAN2

Contact person:	Shan YANG

yangshan@chinatelecom.cn

Attachments:	none

- 1 Overall description

RAN4 would like to thank RAN1 for their LS in R1-2202927. For UL Tx switching across 3 or 4 bands in single TAG scenario, RAN4 has reached the following agreements regarding the RAN1 questions:

RAN1 Question:

RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on potential increase of switching period and UE’s complexity in case of UL Tx switching across 3 or 4 bands in comparison to 2 bands.

RAN4 Answer:

Agreement:

On the length of switching period:

- For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL.

- The length of switching period is applied per band pair for each band combination.

- For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17.

  - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

- For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

On the UE complexity:

RAN4 has not identified any technical difficulty for UE to prevent realizing Tx switching across 3 or 4 bands.

RAN4 would like to recommend the UE memory sharing issue to be further discussed in RAN1 if necessary.

Agreement: the capture the following sentence in the other way forward document.

- The complexity related aspects would be discussed in RAN4 in future meetings.

RAN1 Question:

RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

- “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

RAN4 Answer:

RAN4 has discussed the UE assumption/behavior considering two cases:

- Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B.

For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period.

- Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).

For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

Agreement: capture the following bullet in the way forward

- RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

RAN4 also discussed other related issues for UL Tx switching across 3 or 4 bands in single TAG scenario, and reached the following agreements:

For concurrent UL transmission on 2 bands:

For UL Tx switching across 3 and 4 bands, the support of concurrent UL transmission on 2 (out of 3 or 4) bands at least requires UL CA support on the corresponding band pair(s) by the UE.

For the number of bands supporting 2Tx:

From RAN4 perspective, it was recommended that at least 1 band out of 3 or 4 bands can support 2Tx. In RAN4 understanding, the final decision on the minimum and maximum numbers of bands that a UE shall support 2Tx across the bands is up to RAN1.

FFS:

- Add the action in the LS to ask RAN1 to inform the progress related to the number of bands supporting 2Tx.

- 2 Actions

To 3GPP RAN1

ACTION: RAN4 respectfully asks RAN1 to take into account the above information in their work.

- 3	Dates of next TSG RAN WG4 meetings

TSG-RAN WG4 Meeting #104-e-bis		10-19 October, 2022		 E-meeting

TSG-RAN WG4 Meeting #105			11-14 November, 2022		 TBD