3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, Aug 15 – Aug 26, 2022

Title: 	WF on NR FR1 8Rx for CPE/FWA/vehicle/industrial devices

Agenda Item:	11.6.5

Source: 	NTT DOCOMO, INC.

Document for:	Approval

# Companies’ comments will be transferred to e-mail discussion summary when formal version of WF is prepared.

# RF requirements for 8Rx supporting band

## How to determine the ΔRIB, 8R?

<Way forward >: Open issue needs further discussion

Summary of round 1 discussion

< Candidate options >

  - Option 1:  Take the ΔRIB, 8R defined for LTE as baseline, e.g., consider -4dB for band n77/n78, -4.5dB could be considered as starting point for band n7 (R4-2212014/R4-2212561).

  - Option 2: Apply ΔRIB, 8R = -4dB for all example bands (R4-2213312).

  - Option 3: ΔRIB, 8R has 0 ~ 3 dB improvement compared with ΔRIB, 4R. The exact value needs further discussion (R4-2213969).

  - Option 4: Need more discussion/evaluation (R4-2212804/R4-2213193/R4-2213727).

<Companies’ views on each option>

Option 1: OPPO

Option 2:

Option 3: OPPO, Nokia

Option 4: Qualcomm, vivo, Xiaomi, Skyworks, Nokia, Huawei, Intel, DOCOMO, Meta, AT&T, T-Mobile USA, Sony, Ericsson

Majority views support option 4. Need more discussion and evaluation. During 1st round, several companies commented that RAN4 should study the possible enhanced performance in NR compared to LTE, and take into account the fact that LTE 8Rx aims for any types of UE but this work for NR 8Rx aims for CPE/FWA/vehicle/industrial devices. Also, as impacted factors, ILs, couplings among Rx paths, and antenna correlation are suggested.

- Recommended WF

- Study possible [enhanced] performance of ΔRIB, 8R in NR devices compared to LTE

  - Considering the fact NR 8Rx aims for CPE/FWA/vehicle/industrial devices while LTE 8Rx aims for any types of UE

- As impacted factors, ILs, couplings among Rx paths, and antenna correlation should be studied. Other factors are not precluded.

| Company | Comments |
| --- | --- |
| Huawei | OK with the recommended WF. |
| Nokia | Support the WF |
| Meta | Support recommend WF |
| SoftBank | We are fine with the recommended WF. |
| OPPO | Ok with WF. |
| Qualcomm | First line should be modified to “Study possible enhanced performance difference of ΔRIB, 8R in NR devices compared to LTE. While LTE and NR are different systems, there should be no pre-assumption of enhanced performance. |
| ZTE | Fine with the WF. |
| NTT DOCOMO | Thank you for the comments. We put [] into “enhanced” in recommended WF based on Qualcomm comments. We will discuss in GTW. |

## How to determine the ΔTRxSRS for 1T8R/2T8R AS-SRS?

<Way forward >: Open issue needs further discussion

Summary of round 1 discussion

< Candidate options >

  - Option 1: Reuse 3dB for 8Rx (R4-2212561).

  - Option 2: Need more analysis/evaluation (R4-2212804/R4-2213193/R4-2213727).

  - Option 3: Adopt the following table for additional ILs for different SRS transmission capabilities (R4-2213312).

| UE capability | Additional IL @ .5GHz | Additional IL @ 4.9GHz |
| --- | --- | --- |
| t1r8 | 4 dB | 5 dB |
| t2r8 | 3 dB | 3.5 dB |
| t2r8 and t1r8 | 5 dB | 6 dB |

<Companies’ views on each option>

Option 1:

Option 2: Qualcomm, Xiaomi, Skyworks. Nokia, Huawei, DOCOMO, Meta, AT&T, T-Mobile USA, Sony, Ericsson.

Option 3: OPPO, (Huawei)

Majority views support option 2. Need more analysis and evaluation. During 1st round, as impacted factors, Ils, form factor and the number of antennas are suggested. There are comments that CPE/FWA specific assumption should be further studied, and that the study of the impact of larger SRS IL on the NW performance is needed. Question on the assumption (zero delta is applied for the transmission power calculation for the first SRS resource (or even PUSCH)) is also raised.

- Recommended WF

- Further study ΔTRxSRS for 1T8R/2T8R AS-SRS considering the following aspects:

  - Ils, form factor and the number of Rx/Tx antennas as impacted factors

  - CPE/FWA/vehicle/industrial devices specific assumption

  - The impact of larger SRS IL on the NW performance

- [Clarify if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource ] still valid]

- ΔTRxSRS for 4T8R AS-SRS will be discussed after 4Tx requirement is completed.

| Company | Comments |
| --- | --- |
| Huawei | In general, we are fine with the recommended WF. Typo in the first sub-bullet? “Ls”  “Ils”? |
| Skyworks | Since the same bands and also intended for 4Tx should we have 4T/8R added? So may be we should add number of Tx antennas as a factor. |
| NTT DOCOMO | Thank you for the comments To Huawei, this is a typo, we will revise. To Skyworks, yes, 4T8R should be discussed after 4Tx requirements is completed. Suggestion on modified recommended WF is: - Further study ΔTRxSRS for 1T8R/2T8R AS-SRS considering the following aspects: - Ils, form factor and the number of Rx/Tx antennas as impacted factors - CPE/FWA/vehicle/industrial devices specific assumption - The impact of larger SRS IL on the NW performance - Clarify if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource (or even PUSCH)) still valid - ΔTRxSRS for 4T8R AS-SRS will be discussed after 4Tx requirement is completed. |
| Nokia | We think “− Clarify if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource (or even PUSCH)) still valid” should be removed. It’s covered by WF in topic 1.3 and we don’t understand what the sub-bullet mean. - Why is PUSCH related to this discussion? - What is zero delta(we guess this is notΔTRxSRS but ΔPPowerClass from Huawei’s comment)? We are ok to study revisit of applicability of ΔPPowerClass to PCMAX_H,f,c and/or possibility to with consideration of network impact of DL CSI acquisition via SRS antenna switching. And our understanding is that not only t1ry, but also t2ry should be taken into account for this discussion if the declared power class is PC1.5. This is covered topic 1.3 |
| Meta | Principle OK to us. The ΔTRxSRS,will be studied according to the 1T8R/2T8R/4T8R. ΔPPowerClass is discussed in the configured Tx power. |
| OPPO | Principle is ok, in this meeting we have given one reference architecture for FWA device type from product implementation including 1T8R/2T8R and also 2T8R fallback 1T8R, we encourage companies to check the values in the option 3. For the 4T8R, it should be considered after the 4T is introduced in the spec, we not prefer to start the 4T8R discussion in this early stage. |
| Huawei | OK with the revision from DOCOMO, thanks. To Nokia: Thanks for the comments, please find our further explanations. First we think our proposal, i.e. the second bullet, is related to issue 1.2 and not covered by issue 1.3. Thus we could only focus on ΔTRxSRS under issue 1.2. Second, the reason for our proposal is that with the valuable inputs from OPPO, more Ils are needed for each Tx/Rx chain. Then it is reasonable to further study whether the original “zero delta” for the first SRS resource is still valid or not because the non-zero ΔTRxSRS is applied for the SRS resources other than the first one according to the definition in TS 38.101-1: But we understand the concern from Nokia, since only SRS is under discussion for this topic. So we would like to try the following revision based on DOCOMO’s proposal: - Further study ΔTRxSRS for 1T8R/2T8R AS-SRS considering the following aspects: - Ils, form factor and the number of Rx/Tx antennas as impacted factors - CPE/FWA/vehicle/industrial devices specific assumption - The impact of larger SRS IL on the NW performance - Clarify if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource (or even PUSCH)) still valid - ΔTRxSRS for 4T8R AS-SRS will be discussed after 4Tx requirement is completed. |
| Qualcomm | OK with the principle. However, the sentence “Clarify if the assumption” is not necessarily needed and should be removed or modified to erase PUSCH as that is not in the scope. |
| ZTE | Ok to further study. |
| NTT DOCOMO | Thank you for the comments. We add clarification text and collect typo as suggested in out comments since there are no objections. Regarding “Clarify if ~” part, we put [ ] since this is a controversial part. We will discuss it in GTW. |

## Methods to improve the SRS output power given the large delta SRS value

<Way forward >: Open issue needs further discussion

Summary of round 1 discussion

< Candidate options >

  - Option 1: For a UE indicating the support of TxD and 1T8R AS-SRS, the ΔPPowerClass applied for PCMAX_H,f,c should be reviewed (R4-2213727).

  - Option 2: Others

<Companies’ views on each option>

Option 1:

Option 2 Qualcomm, Skyworks, Nokia (Need more analysis), OPPO (different view), AT&T, T-Mobile USA,

Need more discussion. There is a question for clarification, and proponent answer that this proposes to study a way to improve the SRS output power, e.g., by not applying ΔPPowerClass for PCMAX_H,f,c, while considering the severe power imbalance foreseen for the routing implementation with 8Rx, and possibility of supporting more complex front end architecture for 8R AS-SRS. On the other hands, there is a comment thatΔPPowerClass should be applied to both Pcmax,L and Pcmax,H.

- Recommended WF

For a UE that supports 2Tx and 1T8R SRS AS, further study whether 3dB power back off at main antenna defined for TxD (ΔPPowerClass) is applied for PCMAX_H,f,c or not

For a UE that supports 4Tx and 1T8R and/or 2T8R SRS AS, further study after 4Tx requirement is completed.

| Company | Comments |
| --- | --- |
| Huawei | Appreciate for the discussion so far. As explained during first round, we think 3dB power reduction introduced by ΔPPowerClass for the upper bound ΔPCMAX_H, f, c is not necessary, which unnecessarily low down the possible higher SRS output power, and we are open to discuss whether such solution could be extended for other xT8R AS-SRS. Just to clarify our proposal for the improving SRS output power issue: Since SRS is important for channel estimation, the larger the SRS transmission power, the better the system performance, however, some potential issues on SRS transmission power become even more serious with 8Rx, e.g. the rationality of 0dB IL assumption for the first SRS resource and enlarged IL imbalance between RF chains as mentioned in issue 1.2, we should find all possibilities to increase the SRS transmission power rather than stick to the current spec. More methods could be studied in the next meeting while removing the ΔPPowerClass for Pcamx,h is one of a possible solution. |
| Skyworks | This also may depend on the number of Tx antennas that we consider for TxD. In our view TxD should be restricted to 2Tx as the gain for 4Tx using CDD needs to be better understood. |
| NTT DOCOMO | Thank you for the comments. To Huawei, we understand that the motivation is to improve SRS output power, and removing ΔPPowerClass for Pcamx,h is one of a possible solution. We can add “Other solutions to improve SRS output power are not precluded” if other companies are also OK. To Skyworks, for question for our understanding, your comments on “TxD should be restricted to 2Tx” is for SRS AS, or for general 4Tx TxD? |
| Nokia | We don’t agree with using a text “a UE indicating the support of TxD” at this moment. We don’t know what kind of TxD is assumed here. We don’t know why we need to apply the conclusion of 1T8R to that of 2T8R? They are different. Alternative is below. For a UE that supports 4Tx and 1T8R and/or 2T8R AS-SRS, further study whether ΔPPowerClass is applied for PCMAX_H,f,c or not. We don’t think we need to add “Other solutions to improve SRS output power are not precluded” to the WF before we understand issues correctly and which WG(s) should address the issues. |
| OPPO | Generally we are ok with this study, but not clear how UE with TxD can only lower the Pcmax,L but no change of Pcmax,H, isn’t TxD will cause 3dB degradation to both Pcmax,L and H in the antenna 0 transmission? Nokia alternative is clearer maybe small update is needed since the ΔPPowerClass in the spec is for several purposes and the discussion here is for the 3dB power back off due to TxD feature. Maybe we can just say the 3dB power back off defined for TxD in the main antenna? Alternative 2: For a UE that supports 4Tx and 1T8R and/or 2T8R AS-SRS, further study whether 3dB power back off at main antenna defined for TxD (ΔPPowerClass) is applied for PCMAX_H,f,c or not. |
| Huawei | Thanks for the comments. We are fine with the alternative from Nokia, but we would like to focus on 1t8R and 2t8R at this stage, which is: For a UE that supports 4Tx and 1T8R and/or 2T8R AS-SRS, further study whether ΔPPowerClass is applied for PCMAX_H,f,c or not. |
| Qualcomm | Rationale to coupling TxD into improving 8RX SRS output power is not well elaborated. If discussed, shouldn’t it be discussed under maintenance for all SRS TxSwitch capabilities and not for 8RX only? |
| NTT DOCOMO | Thank you for the comments. We updated the sentence based on companies” comments. The original text in recommended WF is intended for UE supporting 2Tx. So we clarify it. For UE supporting 4Tx, it should be discussed later after 4Tx is completed. |

# Release independence issue

## Release independence issue for 8Rx

<Way forward >: Check if the following WF is agreeable

- Further study if 8Rx can be release independent from Rel-15 after seeing how the requirements of 8Rx looks like.

| Company | Comments |
| --- | --- |
| Huawei | We are fine with further study whether 8Rx could be release independent or not, but we think the sub-bullet may be inappropriate since AS-SRS for >4Rx may not be supported in early release like Rel-16. In general, we suggest to remove the sub-bullet: - Further study if 8Rx can be release independent from Rel-15 after seeing how the requirements of 8Rx looks like. |
| NTT DOCOMO | Given that SRS AS is supported in later release than Rel-15, then we wonder if 8Rx without SRS AS can be release independent from Rel-15, and 8Rx with SRS AS can be from later release. So, we think SRS AS is one of the discussion point, and that’ why we captured it in WF. But we are open to discuss it. Let us see other companies’ comments. |
| Nokia | We don’t need the last sub-bullet. We need to discuss something following aspects in the future after 8Rx requirements become stable to some extent. SRS AS has been introduced since Rel-15. New SRS port switching patterns were introduced in Rel-16, but they are not specific to 8Rx. In Rel-17, SRS Tx port switching patterns were further enhanced and 8Rx relevant patterns were introduced. So, if the definition of 8Rx support includes SRS antenna switching including txr8, Rel-17 spec is needed. But there may be an option that UE supports 8Rx but SRS antenna switching patterns are limited to Rel15 while the device may not be able to achieve what the proponent of 8Rx wanted. In any case, SRS antenna switching is option regardless of releases, the relation b/w 8Rx and required SRS antenna switching patterns depends on if RAN4 impose RAN4 discipline on it or not. |
| Meta | It will be discussed in later based on the status of other WG supporting of 8Rx feature |
| SoftBank | We are fine with continuing the study for release independency. |
| OPPO | For clarification, this is for 8Rx feature not specifically for 8Rx SRS antenna switching, right? |
| Qualcomm | The WF should be simply “Further study 8Rx release independence after seeing how the requirements of 8Rx looks like”. As pointed out by few companies above there are many impacting aspects so mentioning Rel-15 specifically is not useful. |
| NTT DOCOMO | Thank you for the comments. Several companies support to remove the 2nd bullet, then we removed. Qualcomm’s suggestion on wording is fine to us, but in 1st round, several operators support release independence from Rel-15, that’s why we use this wording. We will discuss it in GTW. |

# CA scenario

## The support of 8Rx for a band combination.

<Way forward >: Check if the following WF is agreeable

- Further study the support of 8Rx for a band combination [with example band combination(s)] after seeing how the requirements of 8Rx for single carrier looks like.

| Company | Comments |
| --- | --- |
| Huawei | OK to focus on the single band requirements with 8Rx at this stage. We are fine with the current recommended WF. |
| Skyworks | We should cover one CC first and then check if the framework is applicable to DL CA. At this point in time we are not sure it is useful to elaborate on the options. In any case we will need a example combination to start the work (hopefully from an operator request) |
| NTT DOCOMO | Thank you for the comments. To Skyworks, the intention was that this issue will be discussed later after 8Rx for single carrier is competed, so, as a reference for the future, we just captured possible options. But we are open to discuss it. Let us see other companies’ comments. |
| Nokia | We agree with Skyworks. Not sure why we keep discussing this now. Note that we don’t say we don’t need to discuss this, but we don’t need to discuss this now. |
| Meta | Support WF and example band combinations will be studied first by operator request. |
| SoftBank | We are fine with the recommended WF and focusing on the discussion for the single carrier requirements at this timing. |
| OPPO | Agree with Skyworks and Nokia. Now we are focusing on the single band 8Rx, and no discussion for band combinations, suggest to remove the options in this WF. |
| Qualcomm | Agree with Skyworks |
| ZTE | Agree with Skyworks. |
| NTT DOCOMO | Thank you for the comments. Several companies support to remove the possible options, then we removed it. Several companies support to consider example band combination(s), so we add it with []. |

# SRS guard period

## Whether guard period between xTyR (y>=4) AS-SRS resources within one resource set needs to be kept.

<Way forward >: Open issue needs further discussion

Summary of round 1 discussion

< Candidate options >

- Proposal (R4-2213969): Remove the requirement for a guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage. That will allow the base station to have an accurate and timely CSI based on SRS sounding, which is especially important for a UE configured with xTyR where y≥4.

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others

<Companies’ views on each option>

Option 1: Ericsson

Option 2: Huawei

Option 3: Qualcomm, Skyworks (Need more analysis), OPPO(Question)

1 company supports option1, and 1 company supports option 2. 2 companies think need more analysis.

- Recommended WF

- Further study if remove the requirement for a guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage.

  - Check if this has RAN1 spec impact.

| Company | Comments |
| --- | --- |
| Huawei | As also questioned by OPPO, obviously there are spec impacts for both RAN1 and RAN4 due to the fact that the guard period is defined in TS 38.214 and referred in TS 38.101-1. Actually, during the late stage of the discussion for Rel-17 FeMIMO WI, this issue received some interests since RAN1 was discussing on less SRS scheduling restriction under the background of 6Rx SRS pattern design. Regarding the RAN4 conclusion which could be found in RAN4 LS R4-2202413 & R4-2211226, it has been clarified that there is NO RAN4 spec impacts for this issue, i.e. the Y symbols guard period which had been specified since Rel-15 shall not be removed. Thus we still support Option 2. |
| Qualcomm | Preference is not to change the existing specification. |