3GPP TSG-RAN WG4 Meeting #104-e	R4-2214459

Electronic Meeting, 15– 26 August 2022

Title: 	WF on co-existence evaluation for ATG

Agenda Item:	11.12.5

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

In RAN#95e meeting, a new work item [1] on Air-to-ground network for NR was established. In this RAN4 meeting, companies provided a lot of contributions [2~9] to discuss the co-existence study for ATG. In this way forward, we’d like to collect the outcome based on the 1st round discussion.

# General issues

<Way forward 1>: The following assumption can be acceptable.

- “The ATG network should be distinguished from TN network to enable/disable specific type of UE access”

<Way forward 2>: Discuss whether ATG BS can be co-located with TN BS

- Proposal: that the scenario of ATG BS non-co-located with TN BS is included. Whether the scenario of ATG BS co-located with synchronized TN BS should be included will be decided in the next meeting.

  - Note: working group can clarify if “the TN BS with ATG BS are synchronized then co-location” is a valid scenario.

Discussions:

<Way forward 3>: Clarify an assumption on the density of aircraft within the simulation area.

- Option 1: It’s assumed single UE per ATG BS as a starting point.

- Option 2: FFS

<Way forward 4>: How can RAN4 determine the ATG BS ISD and number of beams per BS?

- It’s encouraged to provide the ATG ISD evaluation based on both capacity and coverage.

<Way forward 5>: For commercial aircraft flight, what is the range of UE altitude?

- The following upper and lower boundary can be considered.

  - UE altitude (upper boundary): range: 10~15km

  - UE altitude (lower boundary): range: 3~7km

Discussion:

Huawei: we can choose one value for lower and upper boundary separately.

Qualcomm: for UE altitude, we prefer to use one value for each simulation. We do not want to use random value. We can just select higher bound and lower bound.

ZTE: for lower boundary, choose 7km. In that altitude there is no additional service.

Huawei: We have concern on the simulation workload. We just drop aircraft into range.

China Telecom: for specific number, we should check worst case. (3 for lower and 10 for upper)

Apple: to lower boundary, I wonder if 7 km is too high. We need consider helicopter.

Ericsson: for values, we would like prefer to 3km. This is co-existence simulation. We should make sure it is robust.

CMCC: we also support to use lower value to evaluate the worst case, 3 and 10.

ZTE: during the aircraft taking-off and landing the UE needs be turned off. We believe 3km is not the use case.

LGE: we have similar view as CTC and CMCC.

ZTE: what is the link budget for 3 km?

China Telecom: service can be provided at 3km.

Ericsson: regarding 3 or 7km, it is not obvious what regulation is used. Aircraft may not land at 3km. For link budget, if it does not work, we

Agreement:

For UE altitude of commercial aircraft flight,

- The following upper and lower boundary can be considered.

  - UE altitude (upper boundary): range: 10km

  - UE altitude (lower boundary): range: 3~7km

- Choose one fixed value for one simulation.

<Way forward 6>: Discuss whether to consider other scenarios with lower altitude and speed (in addition to commercial aircraft) when setting requirements, e.g. helicopters

- Only aircraft is considered in this WI.

# The outline for ATG co-existence study

<Way forward 7>: The following outline can be a starting point for ATG co-existence study.

- Co-existence simulation scenario

- Co-existence simulation assumption

  - Network layout model

    - Co-existence between ATG and NR terrestrial network

    - Co-existence between ATG and ATG (Moderator’s note: It depends on whether ATG co-existence with ATG will be simulated)

  - System parameters

    - ATG parameters

    - ATG UE parameters

    - TN BS and UE parameters

  - Antenna and beamforming pattern modelling

    - ATG BS antenna model

    - ATG UE antenna model

    - TN BS antenna model

    - TN UE antenna model

  - ACLR and ACS modelling

  - Propagation model

    - Propagation model between ATG BS and ATG UE

    - Propagation model between TN BS and TN UE

    - Propagation model between ATG BS and TN BS

    - Propagation model between ATG BS and TN UE

    - Propagation model between TN BS and ATG UE

    - Propagation model between TN UE and ATG UE

  - Transmission power control model

    - TN UL TPC

    - TN DL TPC

    - ATG UL TPC

    - ATG DL TPC

  - Received power model

  - Performance metric

  - Link level performance for NR ATG coexistence

- Co-existence simulation methodology

- Co-existence simulation results

<Way forward 8>: Is the co-existence scenario adjacent channel only or both adjacent channel and co-channel?

- Only adjacent channel co-existence scenario is considered by RAN4.

Discussions:

China Telecom: we think it is true that co-channel simulation is not done by RAN4. It is better to have co-channel simulation to decide the power class.

ZTE: Power class is just parameters to do the simulation. RAN4 just defines the requirement for adjacent channel. RAN decision for other similar issue is do the simulation for adjacent channel in RAN4.

Apple: we share the similar view as ZTE.

Huawei: it is more relevant to RAN1 to do the co-channel study.

Qualcomm: share the similar view as ZTE and Apple. `

Agreement:

- Focus on adjacent channel co-existence scenario in RAN4.

# Co-existence simulation scenario

<Way forward 9>:

- The following scenarios can be assumed as starting point.

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |
| 15 | ATG with ATG | ATG DL | 100/20M TDD/FDD | ATG DL | 100/20M TDD/FDD | 3.5/2 GHz |  | FFS |
| 16 | ATG with ATG | ATG UL | 100/20M TDD/FDD | ATG UL | 100/20M TDD/FDD | 3.5/2 GHz |  | FFS |

Huawei: remove 15 and 16.

ZTE: support to remove 15 and 16. In NTN we do not consider interference bwt NTN. 7 and 8 are deprioritized.

CMCC: OK to remove 15 and 16. Regarding 7 and 8, for example band we also have n79. For n79 we will have different CP length.

ZTE: 7 corresponds gNB-gNB interference.

# Co-existence network layout

<Way forward 10>: Potential assumption for network layout of ATG.

- Option 1: ATG network can be deployed on the airline routes.

- Option 2: ATG network can be deployed in larger area

Huawei: how many ATG sites should be put in simulation?

ZTE: most company to support option 1. ATG BS is equipped with three sectors to serve the large area.

Ericsson: Option 1 is for aircraft directly. The aircraft goes in different directions as observed in Radar24. We should not assume all the aircraft go in one direction. ZTE comment is part of simulation assumption discussion.

ZTE: single ATC can provide the service for large area.

Verizon: it is better to have regulation study on this one. If you touch countries, we should have some regulation study.

Huawei: if we just put single ATG, option 1 and 2 have no difference.

China Telecom: agree with ZTE.

Ericsson: 1) do we expect all the aircrafts flights in the same difference; 2) antenna; 3) ISD. Discuss those things separately.

<Way forward 11>: The assumption for network layout of NR terrestrial network

- 19-sites 57 sectors with wrap-around can be assumed.

- FFS how to drop the TN cluster.

  - Option 1: drop TN clusters close to ATG BS

  - Option 2: Dropping clusters of TN cells randomly

<Way forward 12>: The assumption for network layout between ATG and ATG.

- Before addressing this issue, it’s better to conclude that the scenario (ATG to ATG) will be considered by working group firstly.

# Co-existence system parameters

<Way forward 13>:

- The initial system parameters outline for ATG BS is assumed as below.

| ATG BS altitude | FFS |
| --- | --- |
| Carrier frequency | 2GHz, 3.5GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@3.5GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@3.5GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@3.5GHz |
| Number of cells | TBC |
| Environment1 | To be removed? |
| UE distribution | TBC |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | FFS |
| Number of UL active UEs per cell (NOTE 2) | FFS |
| DL scheduled bandwidth per UE | FFS |
| UL scheduled bandwidth per UE | FFS |
| Traffic model | Full buffer |
| ATG BS maximum output power | FFS |
| ATG BS noise figure | 5dB |
| Handover margin | FFS |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s); NOTE 3: ATG BS max TX power is defined per polarization |  |

<Way forward 14>:

- The initial system parameters outline for ATG UE is assumed as below.

| ATG UE altitude | FFS based on outcome of WF 5 |
| --- | --- |
| Carrier frequency | 2, 3.5GHz |
| ATG UE max TX power in dBm | FFS |
| ATG UE min TX power in dBm | FFS |
| ATG UE noise figure | FFS |

<Way forward 15>:

- The initial system parameters outline for TN BS and TN UE is assumed as below.

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Carrier frequency |  |  |  |
| Channel bandwidth |  |  |  |
| Scheduled channel bandwidth per UE (DL) |  |  |  |
| Scheduled channel bandwidth per UE (UL) |  |  |  |
| The number of active UE (DL) (Note 1) |  |  |  |
| The number of active UE (UL) (Note 1) |  |  |  |
| Traffic model |  |  |  |
| DL power control |  |  |  |
| UL power control |  |  |  |
| TN BS-UE min distance in meters |  |  |  |
| TN BS max TX power in dBm |  |  |  |
| TN UE max TX power in dBm |  |  |  |
| TN UE min TX power in dBm |  |  |  |
| TN BS Noise figure in dB |  |  |  |
| TN UE Noise figure in dB |  |  |  |
| Handover margin |  |  |  |
| Note 1 Same as the number of BS beam(s); Note 2: TN BS max TX power is defined per polarization |  |  |  |

# Antenna and beamforming pattern modelling

<Way forward 16>: Please discuss the initial Antenna and beamforming pattern modelling outline for ATG BS.

- Option 1: It’s listed as below

|  |  | ATG |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern |  |
| 1.2 | Element gain (dBi) (Note 2) |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |
| 1.5 | Antenna polarization |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) |  |
| 1.7 | Number of supported polarizations, P |  |
| 1.8 | Horizontal/Vertical radiating element spacing |  |
| 1.9 | Array Ohmic loss (dB) (Note 2) |  |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) |  |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) |  |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) |  |
| 1.13 | Mechanical uptilt (degrees) |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

- Option 2: Date parameterized model in 38.803 section 5.2.3.2.4 should be used (apart from that the pre-set downtilt should be adjusted for ATG)

<Way forward 17>: Please discuss the initial Antenna and beamforming pattern modelling outline for ATG UE

- Option 1: Omni antenna assumption.

- Option 2: ATG UE has beam steering capability.

- Option 3: antenna mounted on the aircraft are directional without beam steering capability.

<Way forward 18>: Please discuss whether we need to consider both AAS BS and non-AAS BS

- Only consider AAS BS

<Way forward 19>: Please discuss the initial Antenna and beamforming pattern modelling outline for TN AAS BS.

- Option 1: It’s listed as below

|  |  | Rural/sub-urban….. |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern |  |
| 1.2 | Element gain (dBi) (Note 2) |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |
| 1.5 | Antenna polarization |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) |  |
| 1.7 | Number of supported polarizations, P |  |
| 1.8 | Horizontal/Vertical radiating element spacing |  |
| 1.9 | Array Ohmic loss (dB) (Note 2) |  |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) |  |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) |  |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) |  |
| 1.13 | Mechanical downtilt (degrees) |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

- Option 2: The TN BS use the model of 38.803 section 5.2.3.2.4 using sub-arrays.

<Way forward 20>: Please discuss the initial Antenna and beamforming pattern modelling outline for TN UE

- The following assumption for TN UE antenna is agreeable.

| Characteristics | Handheld |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Tx/Rx Antenna gain | 0 dBi per element |
| the number of Tx and Rx | 1T2R |

# Reference

[1] RP-220962, New WID: Air-to-ground network for NR, CMCC

[2] R4-2211916, General consideration on ATG, Apple

[3] R4-2212315, ATG simulation assumption, CMCC

[4] R4-2212383, Discussion on ATG FR1 co-existence evaluation for ATG network, LG Electronics UK

[5] R4-2212616, Discussion on scenarios and deployment assumptions for ATG, Ericsson

[6] R4-2213158, General discussion on coexistence scenario and assumption for ATG scenario, Huawei, HiSilicon

[7] R4-2213186, Coexistence scenarios of Air-to-ground network for NR, Qualcomm Incorporated

[8] R4-2213686, Discussion on coexistence evaluation for ATG network, ZTE Corporation

[9] R4-2211712, Discussion of the ATG co-existence simulation assumption, CATT