3GPP TSG-RAN WG4 Meeting #104-e 	R4-220xxxx

Online meeting, August 15 – August 26, 2022

Title: 	WF on UL 256QAM

Source: 	Xiaomi

Agenda item:	11.7.1

Document for:	Approval

# WF

- Issue 1-1-1: Link level simulation assumption

In first round discussion, most of companies agree the simulation assumption, at the same time some modifications are proposed.

- Consider 48GHz(n262), 5 companies agree(vivo, Xiaomi, Sony, AT&T, LG), Nokia prefer to focus on 29GHz first;

- PTRS configuration: for DFT-s-OFDM waveform, the mapping parameter (,Nsampgroup)could be choose from the group (2,2) (2,4) (4,2) (4,4) (8,4), two companies prefer to use  (4,4) as baseline;

- Other more parameters need consider refer to R4-2212591.

- Further align the MIMO channel for 2x2 AWGN, whether just use the diagonal matrix.

Agreement:

Modified simulation assumption: (new added as yellow highlight part)

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. 256QAM: CP-OFDM/DFT-s-OFDM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-s-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-s-OFDM: (,Nsampgroup)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 2%, 3%, 3.5%, 4%, rxEVM: 2%, 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

- Issue 1-1-2: Supporting power classes

Agreement:

First focus on PC1, PC2, PC5.

- Discussion on PC3 is not precluded and handled as second priority.

- Issue 1-1-3: EVM test

Agreement:

- Option 1: Configuring PTRS for 256QAM EVM testing to compensate CPE.

- The DMRS based channel estimate in the PTRS-ready EVM calculator shall utilize CPE-corrected DMRS symbols.

- The PTRS extraction and correction stage in the PTRS-ready EVM calculator is the final refinement of the received signal.

- FFS on the compensation for Inter Carrier Interference (ICI).

- Issue 1-1-4: PTRS configuration

Options:

Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration.

Option 2: Use a fixed PTRS configuration for all devices

Discussion:

Moderator: keep two options for further discussion. Try to align on option 1. We can decided whether to send LS to RAN5 based on the discussion in the next meeting.

Qualcomm: Option 2 is added from testability consideration.

R&S: we are fine to keep both options. We want to check what the impact of option 1 is

Oppo: we share the similar view as R&S. There would be testability issue in RAN5. When we define the requirement, we should think about how to test it.

Apple: we also think option 1 and option 2 should be left for further discussions. It is benefit of option 1 over option 2. If the benefit is identified, we can determine it.

Qualcomm: we have delegate of TE in RAN4. Do we need LS to RAN5?

OPPO: it is better to check with RAN5. Some time it is not only TE issue but also verification issue.

Agreement:

- Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration.

- Option 2: Use a fixed PTRS configuration for all devices

- Issue 1-2-1: MPR simulation assumption

The following MPR simulation parameters can be used as the start point, the detail values for the parameters are FFS.

- Transceiver noise

- Modulator I/Q imbalance

- Modulator CIM3

- Carrier suppression

- Phase noise

Discussions:

Nokia: this is not starting point. We would like to keep the values. Otherwise, people use different values in the simulation. We suggest to keep them and if the problem is found we can fix it.

Apple: I see the issue discussed by Nokia. FR1 parameters is quite harsh.

Huawei: the parameters come from FR1 which is not suitable for FR2.

Qualcomm: we want to further discuss the phase noise.

Chair=> further discuss those parameters in the future meeting.

- Issue 1-2-2: EVM budget in MPR simulation

Agreement:

Option 3: Discuss it after EVM is defined.

- Issue 1-3: minimum EIRP

Agreement:

Option 3: Discuss it after EVM and operating SNR are defined.

# Comments

- Issue 1-1-1: Link level simulation assumption

Modified simulation assumption: (new added as yellow highlight part)

| Parameter | Value |
| --- | --- |
| Carrier frequency | 29 GHz (n257), 39 GHz (n260) and 48GHz (n262) |
| CBW | 50 MHz, 100MHz |
| SCS | 120 kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 30ns delay spread, 35Hz Doppler frequency TDL-D 30ns delay spread, 35Hz Doppler frequency Static (AWGN) |
| MCS | 64QAM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. 256QAM: MCS 21, 23 in TS 38.214 Table 5.1.3.1-2, other MCSs are not precluded. Baseline: fixed MCSs |
| Symbol type | CP-OFDM; DFT-S-OFDM |
| HARQ | 8, None |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 (using the diagonal matrix) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PUSCH configuration | Type A mapping, Start symbol 0, Duration 14 |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | CP-OFDM: KPTRS : 2 (every 2 RBs), LPTRS : 1 (every 1 symbol) DFT-S-OFDM: (,Nsampgroup)=(4, 4), LPTRS : 1 (every 1 symbol) |
| Phase noise compensation | Practical based on PTRS |
| Phase noise model | TR 38.803 model (in section 6.1.10 and section 6.1.11) modelled Phase noise for TX and RX Option a): example1 (UE) + example1(BS) Option b): example2 (UE) + example2(BS) Option d): example1 (UE) + example2(BS) |
| txEVM + rxEVM excluding phase noise for 256QAM | txEVM: 2%, 3%, 3.5%, 4%, rxEVM: 2%, 3%, 3.5%, 4% Option 1: txEVM >= rxEVM; |
| Other parameters | follow assumptions in TS38.104 Section 11.2.2 . |

Companies are encouraged to further check the modified simulation assumption and provide comments in below table:

| Company | Comments: |
| --- | --- |
| Huawei | Issue 1-1-1: Link level simulation assumption UL 64QAM MCS should be corrected to “Table 6.1.4.1-1”, Table 5.1.3.1-1 is for DL. 64QAM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1 Issue 1-2-1: MPR simulation assumption Since it states that the detail values for the parameters are FFS, we prefer to remove all the parameters to avoid confusion. |
| Apple | Issue 1-1-2: Supporting power classes We propose to update the wording of the agreement to clarify that the second priority only refers to PC3. Issue 1-2-1: MPR simulation assumption Agree with Huawei. It would be preferable to have the values as FFS. |
| Qualcomm | On Issue 1-2-1: prefer we not retain values if FFS |
| vivo | Issue 1-1-1: Link level simulation assumption In fact, the 64QAM MCS table for DFT-s-OFDM and CP-OFDM is different in TS 38.214, it is better to be further clarified as: 64QAM: CP-OFDM: MCS 23, 24 in TS 38.214 Table 5.1.3.1-1, other MCSs are not precluded. DFT-s-OFDM: MCS 22, 23 in TS 38.214 Table 6.1.4.1-1, other MCSs are not precluded. MCS index is different but they have same code rate. Issue 1-2-1: MPR simulation assumption We are ok to remove all parameter only retain FFS. |
| OPPO | - Issue 1-1-2: Supporting power classes Same view as Apple. PC3 shall be second priority according to WID. - Issue 1-1-4: PTRS configuration Same view as R&S it needs to check RAN5 testability issue before make agreement. As commented in 1st round, conformance testing should fix the configurations to align test conditions for all UEs rather than different configurations. If needed, we think an LS to check RAN5 feasibility is important, though we understand why company want to let UE indicate which PTRS they want. |
| Nokia | Issue 1-1-1: Link level simulation assumption OK with moderator proposal and Huawei correction. Issue 1-2-1: MPR simulation assumption OK with moderator proposal. We should keep the simulation parameters as starting point as moderator suggested, we can revise them in next meeting if needed after we have more simulation results from companies, otherwise each company may start with different set of parameters, and it will be difficult to compare the simulation results in next meeting. |
| LGE | Issue 1-1-1: Link level simulation assumption -We are Ok about revised simulation assumption. Issue 1-1-2: Supporting power classes -We have same view with Apple. Issue 1-2-1: MPR simulation assumption -We think these parameters are good as starting point. However, it may be necessary to modify the parameter values through the further study. |
| Xiaomi | As moderator, we would like to update the following informations based on the comments: Issue 1-1-1: To HW and vivo, you are correct, the MCS configuration information has been updated in WF part according to vivo’s suggestion. Issue 1-2-1: We think it is better to use the MPR simulation parameters in Option1 as the starting point, if other parameters are allowed to add if needed, the detail values for these parameter can keep FFS Issue 1-1-4: We think this issue can further discuss and further align the companies’ understanding of Option 1, then decide whether need send LS to RAN5 to check the test feasibility in next meeting. |
| Sony | We agree PC3 has second priority but not precluded (Thanks Apple to improved wording). Regarding MPR simulation assumption parameters, we are OK to remove. |
| ZTE | Issue 1-1-1: Link level simulation assumption Same comments as Vivo, the MCS table is different for CP-OFDM and DFT-s-OFDM. In addition to calibration purpose, the link level simulation results with different carrier frequencies should be consolidated into demodulation performance requirements. Issue 1-1-2: Supporting power classes Ok with Apple’s proposal. Issue 1-2-1: MPR simulation assumption Ok with Moderator’s proposal to set a starting point and further to make adjustment if necessary. |
| Ericsson | We are OK with the updated WF by moderator, where for Issue 1-2-1 we prefer to remove the exact values of parameters for the moment. |
| AT&T | Issue 1-1-1: Link level simulation assumption OK with moderator proposal and suggested correction from Huawei. |

# Reference

[1] R4-2214111, Email discussion summary for [104-e][133] FR2_enh_req_Ph3, Moderator (Nokia, Xiaomi)