3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, Aug 15 – Aug 26, 2022

Title: 	WF on study for lower MSD

Agenda Item:	11.6.5

Source: 	Huawei, HiSilicon

Document for:	Approval

# Example band combinations for MSD analysis

## Candidate example band combinations

<Agreement in GTW>:

Use the following example band combinations to study the feasibility of MSD improvement for different MSD types:

- CA_n28-n40 (harmonic mixing)

- CA_n41-n77 (cross band isolation)

- CA_n1-n3-n78 and fallback combinations (IMD on the 3rd band, cross band isolation on CA_n1-n3 using 50MHz channel bandwidth, IMD2/4 and 2nd harmonic and harmonic mixing on CA_n3-n78)

Note 1: All supported power classes for the above example band combinations can be analyzed

Note 2: Band combinations with two bands are in the first priority

## power class to be considered for the band combinations in lower MSD study

< Agreement >:

Both PC2 and PC3 band combinations could be considered as example band combination. Other power class is not excluded.

# Study of MSD improvement

## Whether to change the existing MSD requirement

<Agreement>:

The minimum requirements in the specification shall be kept unchanged for the lower MSD study.

## RF component assumptions and methods for lower MSD analysis

< Way forward >:

- Recommended WF

The following assumptions and methods should be considered for the MSD study in next meeting.

- Antenna isolation: 10~20dB

- PCB isolation: 60~[75]dB

- Other parameters, e.g. RF component linearity, diplexer/duplexer/filter rejection, etc. up to the choice of companies

- For the MSD study, the parameters used in the analysis should be provided

- MSD reduction via UL power back-off is FFS, depending on further inputs and clarification in next meeting

Note: It is understood that practical implementations usually make different design trade-offs. The optimal values are unlikely to be achieved for all RF parameters and/or all band combinations simultaneously.

Nokia: In our view, the listed assumptions are not open issues. What we can say is when companies provide analysis, clarify the assumptions.

- GTW agreement:

## How to justify the improved MSD

< Way forward>: Open issues in the recommended WF need further discussion

- Recommended WF

- Any indicated MSD improvement should be verified

- The verification of improved MSD should be based on the same test configurations as for the minimum requirements (MSD test points, UL/DL configurations, etc.)

- No radiated performance requirements are considered to verify MSD improvement

- Detailed testability issues could be further studied in RAN5 [if a corresponding SI was approved]

## How to obtain the threshold(s) of improved MSD

< Way forward>:

How to obtain the threshold(s) can be further considered together with the MSD signaling part in next meeting

Alternative:

The threshold(s) to be reported as useful value(s) can be further considered together with the MSD signaling part in next meeting

- GTW agreement:

# Study of signaling for MSD improvement

## Necessity of signaling improved lower MSD

< Way forward >: Open issue in the recommended WF needs further discussion

- Recommended WF

- Companies can continue discussion on the signaling aspects even no consensus on the necessity of signaling based MSD improvement method. Companies interested in this direction can provide more analysis in next meeting.

Alternative:

- Companies can continue discussion on the signaling aspects even no consensus on the necessity of signaling based MSD improvement method. Companies interested in this direction can provide more analysis in next meeting.

- GTW agreement:

## Clarification of the expected behaviour from NW for the potential lower MSD capability

< Way forward >: Open issues in the recommended WF need further discussion

- Recommended WF

- Basic assumption is that UE with or without lower MSD capabilities should be treated fairly in the NW, but how to schedule UEs with different MSD capabilities is up to NW implementation

- How the lower MSD capability can be utilized by the NW can be further clarified in next meeting

Alternative:

- Basic assumption is that UE with or without lower MSD capabilities should be properly treated according to its performance as well as environmental conditions as much as possible in the NW, but how to schedule UEs with different MSD capabilities is up to NW implementation

- How the lower MSD capability can be utilized by the NW can be further discussed in next meeting

- GTW agreement:

## If lower MSD capability is introduced, should it be an optional capability?

<Agreement>:

If lower MSD capability is introduced, it should be an optional capability.

## How to indicate the MSD is improved for a band combination

< Way forward >:

The following aspects related to lower MSD signaling will be further discussed in next meeting.

- Whether the lower MSD capability is a per BC capability

  - how to handle a band combination with different MSD types

  - how to handle the same BC with different victim bands suffered the same MSD type and order

- Whether lower MSD capability for different interference sources could be reported separately

- Whether lower MSD capability means all MSD types for a band combination have been improved

- Whether delta MSD compared to the minimum requirements or directly improved MSD values to be reported

- Whether a single/unique MSD value or MSD threshold(s) for a band combination to be considered

  - If MSD threshold(s), whether a single MSD threshold value or could be multiple intervals? Exact absolute threshold(s) or relative threshold(s)? Different or same threshold(s) for different interference type?

- Whether MSD reduction via UL power back-off could be dynamically reported

- Applicability of lower MSD capability for different power class

- Signaling overhead for the lower MSD capability

Alternative:

- Relation between MSD reduction and UL power back-off

- GTW agreement:

## Lower MSD capability definition

< Way forward >:

The concept of lower MSD capability definition can be considered, which will be further discussed once the group has a clear view on the lower MSD capability.

Alternative:

The concept of Lower MSD capability definition can be is considered, which will be further discussed once the group has a clear view on the lower MSD capability.

- GTW agreement: