3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, Aug. 15th – 26th, 2022

Title: 	WF on DC location signalling applicability

Agenda Item:	9.4.3

Source: 	Qualcomm Incorporated

Document for:	Approval

# 1-2-1: Discuss which feature variants which method for DC location signalling is applicable.

The intention is to get common understanding what methods can be used to signal DC location for which features. The intent is to enable reporting the location of the carrier leakage.

## Issue 1-2-1-1 Treatment of signalling 3300/3301 vs not supporting applicable signalling method

Two possibilities how to treat the 3300/3301 case in the Ran4 specification:

<Way forward/Agreement>:

- Option 1: Even if the signalling method does not support reporting DC location, mention in the specification the method as one option since 3300/3301 can be used to indicate DC location is outside carrier or in an unknown place in carrier.

- Option 2: Do not consider signalling method a possible method for a feature if it can only be used to signal an unknown location.

| Company | Comments |
| --- | --- |
| Nokia | Option 2: Specifically, 3301, i.e., Undetermined position within the carrier, is a very unfortunate option. It’s useless. Even if a UE always report 3301, the UE is considered as supporting this UL DC location signaling feature… Perhaps, during the test, TE can use this as a criteria to decide to try to find out where the UL DC location is by sweeping the frequency…since at least we know UL DC location somewhere in the carrier..But in the field, it must be useless. 3300 is not the best option since operators may have more than one spectrum blocks. In this case, even if an UL DC of a UE in a spectrum block “A” doesn’t exist, but if it exists in the other spectrum block “B” belonging to the same operator, it’s worth reporting the exact position since other UEs in the other sub-block “B” may avoid impact of the DC from the UE in the block “A” on their UL performance. But still 3300 has some benefit while 3301 doesn’t. |

## Applicable signalling methods

The table from Oppo in the comments as starting point with edits to categorize the issues to help alignment:

Case 1: -> No (Rel-16 scheme only apply to 2UL CA case)

According to the Oppos comments, the method is only when two uplink carriers are configured.

Case 2: -> Yes (3300 reported) and -> Yes (3300 reported for the outside UL CC)

This case depends on issue 1-2-1-1. If it considered reportting an unknow location is meaningful to mention in ran4 specification then this is yes. If it is considered that reportting unknown location results in to the same outcome from ran4 spec, then this is no.

Case 3: -> No (R15 DC location indicated in CA case may be incorrect)

This case the reporting is possible but with restrictions such that there has to be DC on UL carrier.

| Feature/Reporting method |  | R15 | R16 | R17 |
| --- | --- | --- | --- | --- |
| Single CC |  | Yes | No -> No (Rel-16 scheme only apply to 2UL CA case) | Yes |
| DL CA, single UL CC | UL DC on UL CC | Yes | No -> No (Rel-16 scheme only apply to 2UL CA case) | Yes |
|  | UL DC on DL CC | No -> Yes (3300 reported) | No | Yes |
| Contiguous UL CA up to 2 UL CCs | Single LO on UL CC | Yes, but one DC per UL CC -> No (R15 DC location indicated in CA case may be incorrect) | Yes | Yes |
|  | Single LO outside UL CC | No (R15 DC location indicated in CA case may be incorrect) | No -> Yes (3300 reported) | Yes |
|  | Dual LO on UL CC | No -> No (R15 DC location indicated in CA case may be incorrect) | Yes | Yes |
|  | Dual LO on DL CC (N/A in RAN4 specs) | No -> No (R15 DC location indicated in CA case may be incorrect) | No -> Yes (3300 reported) | Yes |
| Contiguous UL CA > 2 UL CCs | Single LO, all cases | No -> No (R15 DC location indicated in CA case may be incorrect) | No | Yes |
| Non-contiguous UL CA | Single LO on UL CC | No -> No (R15 DC location indicated in CA case may be incorrect) | Yes | Yes |
|  | Single LO outside UL CC | No (R15 DC location indicated in CA case may be incorrect) | No -> Yes (3300 reported) | Yes |
|  | Dual LO on UL CC | No -> No (R15 DC location indicated in CA case may be incorrect) | Yes | Yes |
|  | Dual LO, at least one outside UL CC | No (R15 DC location indicated in CA case may be incorrect) | No -> Yes (3300 reported for the outside UL CC) | Yes |

- <Way forward/Agreement>: All the case above will set to “No” (changes to the original in R4-2214039 are in red) since they either can not be used to declare DC location or they can only support limited cases where it can be reported.

Companies are encouraged to provide comments on the above cases and if there are comments or concerns on non highlighted cases, the please indicate where.

| Company | Comments |
| --- | --- |
| Nokia | We support this clarification activity. But we think that it’s more appropriate to ask RAN2 for what each release UL DC location method can do via LS or we capture the table in the WF and collect each company’s view in the next meeting. Because it is likely that RAN4 delegates consult with RAN2 delegates internally and check what is feasible. We don’t think it is the right way for RAN4 to speculate what is feasible in terms of signaling. If we send an LS to RAN2, the questions and associated languages must be clarified. For instance, - What is the exact criteria of “Yes” and “No”? 3300 and/or 3301 counted as Yes? - Single CC on the 2nd row is an appropriate expression? May be replaced with Non-CA state, etc…Since up to Rel-16, the report is conducted per cell basis, but from Rel-17, it must not be the case because the number of CCs can be larger than the number of UL DC. So this must not be interpreted as reporting per CC basis, though we may be overthinking… - Etc… |

# <Topic Z> Issue 1-2-2: Whether change RAN4 specifications to allow exceptions for carrier leakage and IQ image only if UE declares support for an appropriate method for signalling the DC location?

Issue comments had some questions and as proponent, we clarify that the intention from Rel-17 onwards is that the exception for emission requirements such as SEM in FR2 case and IBE in both FR1 and FR2 case would be granted only if UE reports the DC location by using one of the possible methods. For example table Table 6.4A.2.1.2-2 text right most column would say:

"The frequencies of the up to 2 non-allocated RBs are indicated by the UE with IE [uplinkTxDirectCurrent…. -r17] . The frequency raster of the RBs is derived when this component carrier is allocated with RBs”

It may further make sense to clean up the whole treatment and concentrate the carrier leakage indications to the section 6.4A.2.1.3.

<proposed Way forward>:

- From Rel-17 onwards, any RF correction or exception to emission or such IBE for carrier leakage and IQ image will be granted only if UE supports signalling method that enables signalling the carrier leakage for the applicable feature.

| Company | Comments |
| --- | --- |
| Nokia | For the first exception mentioned by the proponent, there was a discussion on if we allow this exception only for FR2 or not. If the same requirement in FR1 applies to FR2, we don’t need to discuss this exception anymore… In any case, the concept that exceptions are allowed for only UEs that indicate where UL DC location(s) is are very reasonable approach. But there are following texts. 3300 or 3301 is reported, requirements are waived since the position is unknown…So, the proponent are thinking that some specific requirements should allow some exceptions for UEs to indicate UL DC location while some other specific requirements should allow some exceptions for UEs not to indicate UL DC location? In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation-r16 or txDirectCurrentLocation (as defined in TS 38.331 [7]) or UE does not indicate the DC location parameters, carrier leakage measurement requirement in clause 6.4A.2.4.2 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality. |