3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, August 15-26, 2022

Title: 	WF on the scope of a coexistence study for HPHT 5G broadcast

Agenda Item:	12.4.5

Source: 	Ericsson

Document for:	Approval

# Topic #1: System parameter

## Sub-Topic 1-1: Coexistence

Agreement:

- Coexistence study is needed for HPHT scenario to at least verify the applicability of UE and BS coexistence parameters (UE ACS and BS ACLR) against regulations and acceptable performance.

Way forward:

- Further investigate HPHT deployment scenario and DTT regulations (see WI RP-220518):

- For ITU region 1, requirements defined in ETSI EN 302 296 and GE06 Agreement.

- For ITU region 2:

  - In United States, Title 47 CFR 73.622.

  - In Brazil, ABNT 15601.

- For ITU region 3, in China, GB20600-2006.

- Review existing coexistence studies (IMT adjacent to DTT) and their considered scenarios:

- TR 36.776 Study on LTE-based 5G terrestrial broadcast.

- TR 36.976 Overall description of LTE-based 5G broadcast.

- ETSI TS 103 720 Broadcast System for linear TV and radio services; LTE-based 5G terrestrial broadcast system.

- Based on the above inputs (note: other inputs are not precluded):

  - Assess if 3GPP BS and UE requirements (e.g. ACLR, ACS, blocking, …) would still be relevant for the foreseen scenarios and adopted Regulations.

  - If not, evaluate potential impacts on 3GPP BS/UE requirements.