3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, August 15-26, 2022

Title: 	WF on the scope of a coexistence study for HPHT 5G broadcast

Agenda Item:	12.4.5

Source: 	Ericsson

Document for:	Approval

# Topic #1: System parameter

## Sub-Topic 1-1: Coexistence

Agreement:

- Coexistence study is needed for HPHT scenario to at least verify the applicability of UE and BS coexistence parameters (UE ACS/blocking and BS ACLR) against regulations and acceptable performance.

Way forward:

- Further investigate HPHT deployment scenario and DTT regulations (see WI RP-220518):

- For ITU region 1, requirements defined in ETSI EN 302 296, EN 303 340 and GE06 Agreement.

- For ITU region 2:

  - In United States, Title 47 CFR 73.622.

  - In Brazil, ABNT 15601.

- For ITU region 3, in China, GB20600-2006.

- Review the simulation assumptions and their considered scenarios:

- TR 36.776 Study on LTE-based 5G terrestrial broadcast.

- TR 36.976 Overall description of LTE-based 5G broadcast.

- ETSI TS 103 720 Broadcast System for linear TV and radio services; LTE-based 5G terrestrial broadcast system.

- EBU TR 054

- The following scenarios are to be studied for coexistence:

  - Coexistence of 5G broadcast BS and 5G broadcast UE in adjacent channels within the same band

    - Shall include in the study coordinated deployments with co-located 5G broadcast BSs and necessary guard bands

    - Non-coordinated deployments can also be studied to understand the limitations but the WID indicates coordination is expected

  - UE blocking for the coexistence of IMT BS and 5G broadcast in adjacent bands

    - Assumption on UE Rx filter capability should be indicated. Guard bands between adjacent 5G broadcast and IMT should be evaluated and quantified.

- Based on the above inputs (note: other inputs are not precluded):

  - Assess if 3GPP BS and UE requirements (e.g. ACLR, ACS, blocking, …) would still be relevant for the foreseen scenarios and adopted Regulations.

  - If not, evaluate potential impacts on 3GPP BS/UE requirements.