3GPP TSG-RAN WG4 Meeting #104-e	R4-2214459

Electronic Meeting, 15– 26 August 2022

Title: 	WF on co-existence evaluation for ATG

Agenda Item:	11.12.5

Source: 	Huawei, HiSilicon

Document for:	Approval

# Background

In RAN#95e meeting, a new work item [1] on Air-to-ground network for NR was established. In this RAN4 meeting, companies provided a lot of contributions [2~9] to discuss the co-existence study for ATG. In this way forward, we’d like to collect the outcome based on the 1st round discussion.

# General issues

<Way forward 1>: The following assumption can be acceptable.

- “The ATG network should be distinguished from TN network to enable/disable specific type of UE access”

<Way forward 2>: Discuss whether ATG BS can be co-located with TN BS

- Proposal: that the scenario of ATG BS non-co-located with TN BS is included. Whether the scenario of ATG BS co-located with synchronized TN BS should be included will be decided in the next meeting.

  - Note: working group can clarify if “the TN BS with ATG BS are synchronized then co-location” is a valid scenario.

Ericsson: Our understanding is that some ATG vendors co-locate TN and ATG BS (with synchronization). It is good if the requirements are in the end suitable for all relevant scenarios. We are not convinced to agree a prioritization right now; if both are relevant then both should be done. We can check on the co-located until the next meeting.

Huawei: To Ericsson, I’m OK with your revision.

<Way forward 3>: Clarify an assumption on the density of aircraft within the simulation area.

- Option 1: It’s assumed single UE per ATG BS as a starting point.

- Option 2: FFS

Ericsson: This is OK, but then we assume that the capacity/density would be taken care of by more dense BS rather than more beams or sectors per BS.

Qualcomm: it is OK with Option 1 as the starting point. We’d like to encourage companies to do more analysis on the link budget to decide the density of aircraft and ISD.

<Way forward 4>: How can RAN4 determine the ATG BS ISD and number of beams per BS?

- It’s encouraged to provide the ATG ISD evaluation based on both capacity and coverage.

<Way forward 5>: For commercial aircraft flight, what is the range of UE altitude?

- The following upper and lower boundary can be considered.

  - UE altitude (upper boundary): range: 10~15km

  - UE altitude (lower boundary): range: 3~7km

Ericsson: It is OK, but it would be good to get to a single number for the UE altitude, or possibly 2 scenarios (high altitude, low altitude). Is the intention to achieve that in the next meeting ?

Huawei: To Ericsson, we can check this issue in today’s GTW meeting. My original intention is to achieve an range (x ~ y km) for aircraft cruising. X can be derived from 3~7 km. Y can be derived from 10~15 km. And aircraft can be dropped between X and Y randomly.

China Telecom: Agree with Ericsson. Specific numbers for two scenarios are needed. We prefer 3km for low altitude and 10km for high, which is the low boundary for both scenarios. It makes sense to evaluate worst-case performance for both cases.

Qualcomm: Agree with Ericsson. We should just select one UE altitude in one simulation case (we can have two cases as suggested by CTC). Otherwise, the results would not be converged.

<Way forward 6>: Discuss whether to consider other scenarios with lower altitude and speed (in addition to commercial aircraft) when setting requirements, e.g. helicopters

- Only aircraft is considered in this WI.

# The outline for ATG co-existence study

<Way forward 7>: The following outline can be a starting point for ATG co-existence study.

- Co-existence simulation scenario

- Co-existence simulation assumption

  - Network layout model

    - Co-existence between ATG and NR terrestrial network

    - Co-existence between ATG and ATG (Moderator’s note: It depends on whether ATG co-existence with ATG will be simulated)

  - System parameters

    - ATG parameters

    - ATG UE parameters

    - TN BS and UE parameters

  - Antenna and beamforming pattern modelling

    - ATG BS antenna model

    - ATG UE antenna model

    - TN BS antenna model

    - TN UE antenna model

  - ACLR and ACS modelling

  - Propagation model

    - Propagation model between ATG BS and ATG UE

    - Propagation model between TN BS and TN UE

    - Propagation model between ATG BS and TN BS

    - Propagation model between ATG BS and TN UE

    - Propagation model between TN BS and ATG UE

    - Propagation model between TN UE and ATG UE

  - Transmission power control model

    - TN UL TPC

    - TN DL TPC

    - ATG UL TPC

    - ATG DL TPC

  - Received power model

  - Performance metric

  - Link level performance for NR ATG coexistence

- Co-existence simulation methodology

- Co-existence simulation results

<Way forward 8>: Is the co-existence scenario adjacent channel only or both adjacent channel and co-channel ?

- Only adjacent channel co-existence scenario is considered by RAN4.

- China Telecom: Based on our understanding, it also might be necessary to consider the co-channel scenario since the transmitting power of the ATG UE can be affected. Co-channel simulation is usually not done by RAN4. However, for the ATG scenario, the power class of ATG UE hasn't been decided, we think it’s beneficial to do co-channel simulation to help define ATG UE's power class.

# Co-existence simulation scenario

<Way forward 9>:

- The following scenarios can be assumed as starting point.

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |
| 15 | ATG with ATG | ATG DL | 100/20M TDD/FDD | ATG DL | 100/20M TDD/FDD | 3.5/2 GHz |  | FFS |
| 16 | ATG with ATG | ATG UL | 100/20M TDD/FDD | ATG UL | 100/20M TDD/FDD | 3.5/2 GHz |  | FFS |

Ericsson: To clarify; is this table proposing to simulate all of these scenarios, or is it listing all of the possible scenarios with a view to potentially downprioritizing some in the next meeting (e.g. ATG-ATG ?) (We assume the latter; i.e. potential downprioritizing)

Huawei: To Ericsson, we are OK to downprioritize ATG-ATG scenario. That’s why I highlight them as FFS for study phase. Anyway, I can agree some of these scenarios in this meeting.

China Telecom: Some scenarios can be prioritized, like ATG-TN coexistence scenario.

CATT: Why there’s DL-UL, UL-DL interference for TDD. Does it mean different DL/UL configuration? It may need some clarification if ATG and NR use the same SCS, DL/UL configuration, if they’re synchronized.

# Co-existence network layout

<Way forward 10>: Potential assumption for network layout of ATG.

- Option 1: ATG network can be deployed on the airline routes.

- Option 2: ATG network can be deployed in larger area

Ericsson: Regarding deploying along airline routes, if we would have that option then there is a need to consider that there may be multiple airline routes in different directions and crossing one another in dense areas. Since there can be a lot of routes crossing, it seems easier to consider simply a large area for co-existence simulation purposes.

Huawei: To Ericsson, even if we go option 2, we can only put one ATG BS site due to the hundred kilometres coverage. From this point of view, it seems that option 1 and option 2 has no big difference.

China Telecom: It's true that multiple airline routes in different directions will cross one another in dense areas. However, in most cases, flight routes pass through rural areas and suburban areas, where only a few courses exist. In dense areas, option 1 may look similar to option 2 considering the denser routes. However, it's a big difference for other scenarios. Therefore, it's suggested that the simulation should be carried out based on the option 1 with ATG network density varying according to different scenarios.

Qualcomm: We prefer option 1 which is typical scenario we need to consider in the simulation. The network layout of option 1/2 should be provided.

CATT: We slightly prefer option 2. In our understanding option 2 requirement can cover option 1, but not visa verse. For option 1, as commented by other companies, how many routes and what’s the relationship of the routes may need further discussion.

<Way forward 11>: The assumption for network layout of NR terrestrial network

- 19-sites 57 sectors with wrap-around can be assumed.

- FFS how to drop the TN cluster.

  - Option 1: drop TN clusters close to ATG BS

  - Option 2: Dropping clusters of TN cells randomly

Ericsson: We would be OK to narrow to option 1, since TN being close to ATG is the worst case for which requirements should be set (no good if ATG would not cause loss on average, but would cause loss in networks that are deployed close to the ATG…)

Huawei: We are also OK to go option 1.

China Telecom: In dense areas, TN BS may be close to ATG BS. However, this is not the case for rural and other scenarios.

Qualcomm: We are OK with both option 1 and option 2.

CATT: We’re not sure if option 1 is the practical deployment plan from operators. It will cause more stringent requirement. If it’s the majority view, we’re ok.

<Way forward 12>: The assumption for network layout between ATG and ATG.

- Before addressing this issue, it’s better to conclude that the scenario (ATG to ATG) will be considered by working group firstly.

# Co-existence system parameters

<Way forward 13>:

- The initial system parameters outline for ATG BS is assumed as below.

| ATG BS altitude | FFS |
| --- | --- |
| Carrier frequency | 2GHz, 3.5GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@3.5GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@3.5GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@3.5GHz |
| Number of cells | TBC |
| Environment1 | To be removed? |
| UE distribution | TBC |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | FFS |
| Number of UL active UEs per cell (NOTE 2) | FFS |
| DL scheduled bandwidth per UE | FFS |
| UL scheduled bandwidth per UE | FFS |
| Traffic model | Full buffer |
| ATG BS maximum output power | FFS |
| ATG BS noise figure | 5dB |
| Handover margin | FFS |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s); NOTE 3: ATG BS max TX power is defined per polarization |  |

CATT: Can we assume some parameters even just for calibration purpose? Same comment to other parameters.

<Way forward 14>:

- The initial system parameters outline for ATG UE is assumed as below.

| ATG UE altitude | FFS based on outcome of WF 5 |
| --- | --- |
| Carrier frequency | 2, 3.5GHz |
| ATG UE max TX power in dBm | FFS |
| ATG UE min TX power in dBm | FFS |
| ATG UE noise figure | FFS |

<Way forward 15>:

- The initial system parameters outline for TN BS and TN UE is assumed as below.

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Carrier frequency |  |  |  |
| Channel bandwidth |  |  |  |
| Scheduled channel bandwidth per UE (DL) |  |  |  |
| Scheduled channel bandwidth per UE (UL) |  |  |  |
| The number of active UE (DL) (Note 1) |  |  |  |
| The number of active UE (UL) (Note 1) |  |  |  |
| Traffic model |  |  |  |
| DL power control |  |  |  |
| UL power control |  |  |  |
| TN BS-UE min distance in meters |  |  |  |
| TN BS max TX power in dBm |  |  |  |
| TN UE max TX power in dBm |  |  |  |
| TN UE min TX power in dBm |  |  |  |
| TN BS Noise figure in dB |  |  |  |
| TN UE Noise figure in dB |  |  |  |
| Handover margin |  |  |  |
| Note 1 Same as the number of BS beam(s); Note 2: TN BS max TX power is defined per polarization |  |  |  |

CATT: The same as above, our understanding is that TN system may not be difficult to be decided. TR 38.803 can be a starting point.

# Antenna and beamforming pattern modelling

<Way forward 16>: Please discuss the initial Antenna and beamforming pattern modelling outline for ATG BS.

- Option 1: It’s listed as below

|  |  | ATG |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern |  |
| 1.2 | Element gain (dBi) (Note 2) |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |
| 1.5 | Antenna polarization |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) |  |
| 1.7 | Number of supported polarizations, P |  |
| 1.8 | Horizontal/Vertical radiating element spacing |  |
| 1.9 | Array Ohmic loss (dB) (Note 2) |  |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) |  |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) |  |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) |  |
| 1.13 | Mechanical uptilt (degrees) |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

- Option 2: Date parameterized model in 38.803 section 5.2.3.2.4 should be used (apart from that the pre-set downtilt should be adjusted for ATG)

Ericsson: We believe that sub-arrays have already proven to be the most effective way to implement BS in this frequency range. The structure of the array impacts sidelobes (including grating lobes). Sidelobes from an ATG can impact the TN, so it is important to take care in deciding the array structure. In TR 38.803 an array antenna model extension is document with support for vertical sub-arrays. Also, parameters relevant for AAS BS operating within the frequency range 1710 to 4990 MHz is defined. The sub-array model extension requires some more additional parameters to be introduced in table above. We support option 2.

Huawei: To Ericsson, Since the sidelobes may have an impact on the TN BS, I’m not sure if the antenna pattern with sub-arrays is the best choice for ATG BS.

<Way forward 17>: Please discuss the initial Antenna and beamforming pattern modelling outline for ATG UE

- Option 1: Omni antenna assumption.

- Option 2: ATG UE has beam steering capability.

- Option 3: antenna mounted on the aircraft are directional without beam steering capability.

Ericsson: Option 1 does not appear to provide a good enough link budget, and option 3 seems unworkable because it is not possible for the aircraft to steer it’s direction towards the BS.

Huawei: We agree that option 1 can be removed. Assuming a width beam, UE don’t need to do the beam steering just like 4T4R 1-H BS. Since we never assume FR1 UE has beam steering capability, I’m not sure how we can specify such kind of UE for FR1.

China Telecom: Option 2 is preferred.

CATT: Our understanding is that aircraft antenna can be designed as directional similar with BS. Still need to cover certain horizontal and vertical range according to the flying height of the aircraft. If only a certain altitude scenario is simulated, then the range is not very large.

<Way forward 18>: Please discuss whether we need to consider both AAS BS and non-AAS BS

- Only consider AAS BS

<Way forward 19>: Please discuss the initial Antenna and beamforming pattern modelling outline for TN AAS BS.

- Option 1: It’s listed as below

|  |  | Rural/sub-urban….. |
| --- | --- | --- |
| 1 | Base Station Antenna Characteristics |  |
| 1.1 | Antenna pattern |  |
| 1.2 | Element gain (dBi) (Note 2) |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |
| 1.5 | Antenna polarization |  |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) |  |
| 1.7 | Number of supported polarizations, P |  |
| 1.8 | Horizontal/Vertical radiating element spacing |  |
| 1.9 | Array Ohmic loss (dB) (Note 2) |  |
| 1.10 | Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3) |  |
| 1.11 | Base station maximum coverage angle in the horizontal plane (degrees) |  |
| 1.12 | Base station vertical coverage range (degrees) (Note 1) |  |
| 1.13 | Mechanical downtilt (degrees) |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.9.

Note 3:	The conducted power per element assumes Row × Column ×Number of supported polarizations elements (i.e. power per H/V polarized element).

Note 4: Row × Column means there are Row vertical and Column horizontal radiating elements.

- Option 2: The TN BS use the model of 38.803 section 5.2.3.2.4 using sub-arrays.

Ericsson: Similar to ATG, our understanding is that the sub-array implementation is both very likely and important to take into account in order to capture interference patterns realistically. We believe that sub-arrays have already proven to be the most effective way to implement AAS BS in this frequency range. The structure of the array impacts sidelobes including (and grating lobes). Sidelobes from an TN can impact the ATG, so it is important to take care in deciding the array structure. In TR 38.803 an array antenna model extension is document with support for vertical sub-arrays. Also, parameters relevant for AAS BS operating within the frequency range 1710 to 4990 MHz is defined. The sub-array model extension requires some more additional parameters to be introduced in table above. For ATG where a new deployment scenario is created it is essential to model TN BS and ATG BS correctly since side lobe level now will matter. Previously, when all BS was placed on a flat ground the side-lobe performance didn’t matter for RAN4 co-ex evaluation. But for ATG relevant parameters is essential. We prefer option 2.

Huawei: To Ericsson, in R17 NTN, we just assumed the antenna pattern without sub-array implementation. I think ATG scenario can follow what we have assumed in R17.

CATT: Why the model in TR 38,863 can’t be used?

<Way forward 20>: Please discuss the initial Antenna and beamforming pattern modelling outline for TN UE

- The following assumption for TN UE antenna is agreeable.

| Characteristics | Handheld |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Tx/Rx Antenna gain | 0 dBi per element |
| the number of Tx and Rx | 1T2R |

# Reference

[1] RP-220962, New WID: Air-to-ground network for NR, CMCC

[2] R4-2211916, General consideration on ATG, Apple

[3] R4-2212315, ATG simulation assumption, CMCC

[4] R4-2212383, Discussion on ATG FR1 co-existence evaluation for ATG network, LG Electronics UK

[5] R4-2212616, Discussion on scenarios and deployment assumptions for ATG, Ericsson

[6] R4-2213158, General discussion on coexistence scenario and assumption for ATG scenario, Huawei, HiSilicon

[7] R4-2213186, Coexistence scenarios of Air-to-ground network for NR, Qualcomm Incorporated

[8] R4-2213686, Discussion on coexistence evaluation for ATG network, ZTE Corporation

[9] R4-2211712, Discussion of the ATG co-existence simulation assumption, CATT