3GPP TSG-RAN WG4 Meeting #104-e	R4-22xxxxx

Electronic Meeting, August 15 – August 26, 2022

Title: 	WF on UE requirements for ATG

Agenda Item:	11.12.3

Source: 	Apple

Document for:	Approval

# ATG UE

## Issue 3-1-1: Assumption for ATG operating mode

Only consider single cell operation in Rel-18.

## Issue 3-1-2: GNSS assumption

It is proposed that ATG UE in R18 is GNSS capable.

## Issue 3-1-2: Doppler/timing pre-compensation assumption

Pending 2nd round discussion

  - Option1: NTN functionality is re-used together with GNSS (explain how)

  - Option2: FFS

Ericsson: It is OK to re-use NTN functionality, but the “explain how” part is important. There are some differences to Satellites (e.g. ephemeris information consisting of BS positions) and it is important that we check that the NTN functionality can actually work. We should add a “revisit if any issue is found” to the agreement.

## Issue 3-1-3: subclause to capture ATG UE requirement

Pending 2nd round discussion

  - Option1: It is proposed to define UE requirement for ATG UE in separate subclause with suffix J in 38.101-1.

  - Option2:  other, please specify

## Issue 3-1-4: Power class for ATG UE

Pending 2nd round discussion

  - Option1: A separate UE power class is defined for ATG UE

  - Option3: not to define the power class for ATG UE similar as IAB-MT

  - Option2: Consider further whether to set a power level in the requirement, or set a requirement on power accuracy with a declared power (subject to a maximum limit)

Ericsson: Option 2 and option 3 are similar; IAB has a declared power, possibly subject to a limit. IAB has two deployment classes for the IAB-MT though

## Issue 3-1-5: UE requirement type

Pending 2nd round discussion

  - Option1: discuss whether 1-O and 1-H are appropriate for an AAS like UE equipment.

  - Option2: still use UE requirement structure and investigate what is specific aspects for ATG UE.

## Issue 3-1-6: power control

Pending 2nd round discussion

  - Option1: RAN4 to examine whether power control requirement is needed, or whether the dynamic range of power control can be simplified

  - Option 2: Reduce the power control dynamic range according to the ATG scenario

## Issue 3-1-7: power control accuracy

Pending 2nd round discussion

  - Power control accuracy should be investigated, e.g. whether it can be tightened to reduce the interference.

  - Option 2: reuse the current requirement

Ericsson: It is not captured in the WF yet, but probably some requirements like in-band blocking and minimum input level could be relaxed for an ATG UE (not close to strong in-band blockers, not close to serving BS…)