3GPP TSG-RAN WG4 Meeting #104-e	R4-2214333

<Electronic Meeting>, Aug 15 ‒ Aug 26, 2022

Title: 	WF on R17 NR MG enhancements – multiple concurrent MGs

Agenda Item:	9.9.3

Source: 	MediaTek inc.

Document for:	Approval

# Topic #1: General (AI 9.9.1)

No issues in this topic

# Topic #2: Core requirement maintenance (AI 9.9.1.2)

## Issue 2-1:  Proximity condition for overlapping in FR2-2

< Agreement >:

- For proximity condition for overlapping in FR2-2, X=4ms.

## Issue 2-2:  Relation between legacy (classic) MG and concurrent MG

< Way forward/Agreement >:

- Option 1: Send LS to RAN2 asking RAN2 to introduce priority for legacy gaps

- Option 2: No requirement applies when legacy and new gap collides.

- Option 3: Assign a default priority to legacy gap

- Option 4: No requirement when legacy gap (gapConfig) and new gap (gapConfig-r17) are both configured.

| Company | Comments |
| --- | --- |
| Moderator | Please note that we have a draft LS discussion in parallel. The conclusion in this discussion would impact the LS. |
| Ericsson | We prefer Option 3 We suggest to define a default priority in RAN4 instead of further LS to trigger the discussion in RAN2. Option 2 and 4 will result in no requirement when NW configures legacy gap and new gap which is allowed from RAN2. |
| Apple | Considering forward compatibility, option 1 is preferred. In future more and more gaps can be configured simultaneously. It is clearer for UE to know the priority of each gap pattern. Assigning a default priority may result in unpredictable constraint. From RAN4 requirements point of view, if RAN2 introduces priority for legacy gaps, existing RAN4 requirements for concurrent gaps can still apply. |

## Issue 2-3:  How to define the overhead cap when concurrent MGs are configured

< Agreement in Aug 19 GTW session >:

- Regarding the overhead cap on concurrent gaps in Rel-17, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR.

# Topic #3: Performance requirement (AI 9.9.2.2)

## Issue 3-1:  Whether to introduce test cases for PRS measurement

< Agreement in Aug 19 GTW session >:

  - Confirm that RAN4 is to introduce test cases for PRS measurement with concurrent gaps.

  - PRS RSRP test cases will be introduced.

## Issue 3-2:  Whether to introduce test cases for EUTRAN measurement

< Agreement in Aug 19 GTW session >:

- Confirm that RAN4 is to introduce the test case for EUTRAN measurement with concurrent gaps.

## Issue 3-3:  Which test case to add SBI reporting

< Agreement in Aug 19 GTW session >:

- Skip SBI reporting test case for concurrent gaps.

# Reference

[1] R4-2104096, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98e, Feb’21

[2] R4-2105856, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98bis-e, Apr’21

[3] R4-2108346, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#99e, May’21

[4] R4-2115342, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#100e, Aug’21

[5] R4-2120414, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101e, Nov’21

[6] R4-2202603, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101-bis-e, Jan’22

[7] R4-2206874, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#102-e, Feb’22

[8] R4-2210585, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#103, May’22