3GPP TSG-RAN WG4 Meeting #104-e	R4-2214333

<Electronic Meeting>, Aug 15 ‒ Aug 26, 2022

Title: 	WF on R17 NR MG enhancements – multiple concurrent MGs

Agenda Item:	9.9.3

Source: 	MediaTek inc.

Document for:	Approval

# Topic #1: General (AI 9.9.1)

No issues in this topic

# Topic #2: Core requirement maintenance (AI 9.9.1.2)

## Issue 2-1:  Proximity condition for overlapping in FR2-2

< Agreement >:

- For proximity condition for overlapping in FR2-2, X=4ms.

## Issue 2-2:  Relation between legacy (classic) MG and concurrent MG

< Way forward/Agreement >:

- Option 1: Send LS to RAN2 asking RAN2 to introduce priority for legacy gaps

- Option 2: No requirement applies when legacy and new gap collides.

- Option 3: Assign a default priority to legacy gap

- Option 4: No requirement when legacy gap (gapConfig) and new gap (gapConfig-r17) are both configured.

| Company | Comments |
| --- | --- |
| Moderator | Please note that we have a draft LS discussion in parallel. The conclusion in this discussion would impact the LS. |
| Ericsson | We prefer Option 3 We suggest to define a default priority in RAN4 instead of further LS to trigger the discussion in RAN2. Option 2 and 4 will result in no requirement when NW configures legacy gap and new gap which is allowed from RAN2. |
| Apple | Considering forward compatibility, option 1 is preferred. In future more and more gaps can be configured simultaneously. It is clearer for UE to know the priority of each gap pattern. Assigning a default priority may result in unpredictable constraint. From RAN4 requirements point of view, if RAN2 introduces priority for legacy gaps, existing RAN4 requirements for concurrent gaps can still apply. |
| Xiaomi | We prefer option 2 or option 4. In reply LS to RAN2 (R4-2202604), RAN4 common understanding is that the association of frequency layers or dedicated use cases to measurement gaps shall be clearly understood by both UE and Network for all configured measurements. In such late stage, we prefer not to consider the case of legacy gap configured together with concurrent gaps. |

## Issue 2-3:  How to define the overhead cap when concurrent MGs are configured

< Agreement in Aug 19 GTW session >:

- Regarding the overhead cap on concurrent gaps in Rel-17, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR.

# Topic #3: Performance requirement (AI 9.9.2.2)

## Issue 3-1:  Whether to introduce test cases for PRS measurement

< Agreement in Aug 19 GTW session >:

  - Confirm that RAN4 is to introduce test cases for PRS measurement with concurrent gaps.

  - PRS RSRP test cases will be introduced.

## Issue 3-2:  Whether to introduce test cases for EUTRAN measurement

< Agreement in Aug 19 GTW session >:

- Confirm that RAN4 is to introduce the test case for EUTRAN measurement with concurrent gaps.

## Issue 3-3:  Which test case to add SBI reporting

< Agreement in Aug 19 GTW session >:

- Skip SBI reporting test case for concurrent gaps.

# Reference

[1] R4-2104096, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98e, Feb’21

[2] R4-2105856, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98bis-e, Apr’21

[3] R4-2108346, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#99e, May’21

[4] R4-2115342, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#100e, Aug’21

[5] R4-2120414, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101e, Nov’21

[6] R4-2202603, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101-bis-e, Jan’22

[7] R4-2206874, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#102-e, Feb’22

[8] R4-2210585, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#103, May’22