3GPP TSG-RAN WG4 Meeting #104-e	R4-2214333

<Electronic Meeting>, Aug 15 ‒ Aug 26, 2022

Title: 	WF on R17 NR MG enhancements – multiple concurrent MGs

Agenda Item:	9.9.3

Source: 	MediaTek inc.

Document for:	Approval

# Topic #1: General (AI 9.9.1)

No issues in this topic

# Topic #2: Core requirement maintenance (AI 9.9.1.2)

## Issue 2-1:  Proximity condition for overlapping in FR2-2

< Agreement >:

- For proximity condition for overlapping in FR2-2, X=4ms.

## Issue 2-2:  Relation between legacy (classic) MG and concurrent MG

< Way forward/Agreement >:

- Option 1: Send LS to RAN2 asking RAN2 to introduce priority for legacy gaps

- Option 2: No requirement applies when legacy and new gap collides.

- Option 3: Assign a default priority to legacy gap

- Option 4: No requirement when legacy gap (gapConfig) and new gap (gapConfig-r17) are both configured.

| Company | Comments |
| --- | --- |
| Moderator | Please note that we have a draft LS discussion in parallel. The conclusion in this discussion would impact the LS. |
| Ericsson | We prefer Option 3 We suggest to define a default priority in RAN4 instead of further LS to trigger the discussion in RAN2. Option 2 and 4 will result in no requirement when NW configures legacy gap and new gap which is allowed from RAN2. |
| Apple | Considering forward compatibility, option 1 is preferred. In future more and more gaps can be configured simultaneously. It is clearer for UE to know the priority of each gap pattern. Assigning a default priority may result in unpredictable constraint. From RAN4 requirements point of view, if RAN2 introduces priority for legacy gaps, existing RAN4 requirements for concurrent gaps can still apply. |
| Xiaomi | We prefer option 2 or option 4. In reply LS to RAN2 (R4-2202604), RAN4 common understanding is that the association of frequency layers or dedicated use cases to measurement gaps shall be clearly understood by both UE and Network for all configured measurements. In such late stage, we prefer not to consider the case of legacy gap configured together with concurrent gaps. |
| LGE | For clear understanding, Option 1 is preferred |
| ZTE | Prefer Option 1. Regarding to Option 3, RAN4 had some discussion in previous meeting but not any conclusion achieved. If we want to address the obscureness through some default rule, except for default priority, maybe default association is also needed. |
| vivo | Prefer option 2 and 4. Based on option 2/4, from RAN4 perspective, it clearly indicates that the configuration of a legacy gap (without priority) with Rel-17 concurrent gap is no benefit. It avoid the RAN2 impact at the last minute. |
| MTK | We understand a similar issue is also discussed in RAN2 right now. We disagree with Option 3. A default priority could be problematic when later we consider other gaps (e.g., MUSIM). As far as we know, it is hard to decide at this moment whether legacy gap should always have a higher (or lower) priority than MUSIM gap. We prefer Option 2 or Option 4, which aligns with the previous RAN4 agreements that RAN4 does not expect legacy gap and Rel-17 new gap to be configured together. In our understanding, even if RAN2 allows to do something from signalling perspective, it does not mean RAN4 needs to define requirements correspondingly. RAN4 can has its own focus. |
| OPPO | We think this is to seek efficient solutions or requirements for R17 concurrent gaps. As RAN4 agreed, the coexistence of concurrent gap and other type of gaps (e.g., MUSIM gap) is not considered in R17. No requirement applies in case of gap collision, or a default priority to legacy gap, are feasible solutions from our side. There seems no harm to do so in this release as companies also point out a legacy gap and Rel-17 concurrent gaps are not expected to be configured together. In case that network may configure both, we slightly prefer option 3 to define a default priority of legacy gap in RAN4 instead of further LS to trigger the discussion in RAN2. Besides, the joint gap discussion has been started in R18. Considering the compatibility, we think it is also proper to introduce signalling for priority of legacy gaps in a new release, but no additional RAN2 impact in R17. |
| Intel | For Option 1, we don’t think it is a good way to ask RAN2 introduce the signaling so far. So, from RAN4 itself, Option 2/4 are more preferable to us. The other solution (e.g. Option 1 and 3) can be more feasible in Rel18 MGE or other Rel18 WI in which more gaps will be configured jointely. |
| Nokia | Option 1. Our understanding is that RAN2 did not introduce priority for legacy gaps because this was not included as a request in the LS and RAN4 would now need to discuss and find agreements on the remaining details related to the interworking between any legacy/classic MG. We believe the simplest and most forward compatible and future proof approach (also considering the Rel18 WI) is option 1. We do not see option 4 as agreeable. It is obvious that legacy gaps and new gaps (with priority) can be configured and be active simultaneously if they do not overlap. The discussion is also ongoing in RAN2 but let us make some comments and questions to companies views submitted so far: For MTK comment ‘previous RAN4 agreements that RAN4 does not expect legacy gap and Rel-17 new gap to be configured together’ – maybe MTK can refer to when this was agreed to help the discussion. We agree with several companies that it may be problematic in the future if we assign a default priority (high or low) as the future use case scenarios are very difficult to predict. Hence, such default priority could bring problems. We agree with companies views that asking RAN2 to introduce priority for legacy gaps will be a far more future proof solution. We do not see that there should be a problem in asking this from RAN2. |
| CATT | Option 1 or option 4. Within R17 scope, we think option 4 is acceptable. In previous agreement, RAN4 except the association between concurrent gaps and MO is always configured, so we don’t see the necessity to configure legacy gap and concurrent together. But considering that it has been agreed in RAN2 that they can be configured together from signalling perspective, we are also fine with option 1 to introduce the priority. We understand RAN2 is also working on this issue and option 1 may facilitate the discussion is RAN2. For option 2, one clarification is needed what does the collision mean. Does it mean the physically colliding or the colliding with proximity condition as we defined for concurrent gap? |
| Ericsson | Based on the latest information, it seems RAN2 will have some conclusions on this issue soon. Thus, we suggest to postpone the discussion and further check whether RAN2 will solve it by themselves. We agree some comments about option 3. Then we can compromise to option 2. |

## Issue 2-3:  How to define the overhead cap when concurrent MGs are configured

< Agreement in Aug 19 GTW session >:

- Regarding the overhead cap on concurrent gaps in Rel-17, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR.

# Topic #3: Performance requirement (AI 9.9.2.2)

## Issue 3-1:  Whether to introduce test cases for PRS measurement

< Agreement in Aug 19 GTW session >:

  - Confirm that RAN4 is to introduce test cases for PRS measurement with concurrent gaps.

  - PRS RSRP test cases will be introduced.

## Issue 3-2:  Whether to introduce test cases for EUTRAN measurement

< Agreement in Aug 19 GTW session >:

- Confirm that RAN4 is to introduce the test case for EUTRAN measurement with concurrent gaps.

## Issue 3-3:  Which test case to add SBI reporting

< Agreement in Aug 19 GTW session >:

- Skip SBI reporting test case for concurrent gaps.

# Reference

[1] R4-2104096, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98e, Feb’21

[2] R4-2105856, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#98bis-e, Apr’21

[3] R4-2108346, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#99e, May’21

[4] R4-2115342, “WF on R17 NR MG enhancements – Multiple concurrent and independent MG patterns”, MediaTek Inc., RAN4#100e, Aug’21

[5] R4-2120414, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101e, Nov’21

[6] R4-2202603, “WF on R17 NR MG enhancements – General issues and multiple concurrent MGs”, MediaTek Inc., RAN4#101-bis-e, Jan’22

[7] R4-2206874, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#102-e, Feb’22

[8] R4-2210585, “WF on R17 NR MG enhancements – multiple concurrent MGs”, MediaTek Inc., RAN4#103, May’22