3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214348

Electronic Meeting, 15– 26 August 2022

Title: 	WF on Further NR Mobility Enhancements

Agenda Item:	11.16

Source: 	MediaTek Inc.

Document for:	Approval

# Topic #1: General and work plan

## Sub-topic 1-1: Work plan

<Way forward/Agreement>: Issue 1-1: Work plan proposals

In first round discussion, some companies have some concern on the work plan in R4-2211550.

For 2nd round: comment on the revised work plan in the 2nd round.

Note: please comment on the dedicated Email thread for work plan.

# Topic #2: Study of improvement on FR2 Scell/SCG setup/resume

## Sub-topic 2-1: Clarification and potential direction

<Way forward/Agreement>: Issue 2-1-1: Carification on time point of “when UE has initiated access”

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1(Ericsson, Nokia): RAN4 to clarify that the “when UE has initiated access” is the point in time when RRC procedures in clauses 5.3.2.3, 5.3.3.2 or 5.3.13.2 (38.331) are initiated.

    - Paging

    - Initiation (UE receives SIB1 and apply default MAC cell group configuration)

    - RRC connection resume

  - Option 2(MTK, CMCC, Apple, xiaomi, HW, vivo, CATT): Discuss the starting point for improved measurement directly.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-1-2: Potential direction for further study: measurement enhancement when UE is about to enter connected mode

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume.

    - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest.

    - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest.

  - Option 2: UE perform enhanced measurement during RRC connection setup/resume.

    - Option 2a (Nokia): perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume)

    - Option 2b (MTK, vivo, Ericsson ): after receiving paging for MT originating call

    - Option 2c (MTK, HW, vivo): after first RACH preamble transmission, i.e. Msg1

    - Option 2d (Ericsson): Initiation (UE receives SIB1 and apply default MAC cell group configuration) for MO originating call

    - Option 2e (Apple, xiaomi, QC): FFS

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-1-3: Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode

According to the 1st round discussion, the most divergence on this issue is whether further enhancement on R16 EMR (measurement during green part) is in the scope. Moderator suggests to align the understanding on this at first before go to more details.

  - Option 1 (MTK, Apple, xiaomi, HW, vivo): further enhancement on Rel-16 EMR (measurement during green part) is out of scope

  - Option 2 (CMCC, QC, Ericsson, Nokia, LGE, CATT): further enhancement on Rel-16 EMR (measurement during green part) is in the scope

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-1-4: Applicable scenarios

Tentative agreements in the 1st round: The enhanced measurement (if feasible) at least applies to scenarios that the EMR measurement results are unavailable or invalid.

Please provide further comments on the following option

  - Revised Option 2 (QC, vivo, LGE, Nokia):  Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and valid

    - e.g. On the cells that have been detected/measured in early measurement

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

## Sub-topic 2-2: Assumptions for feasibility study

<Way forward/Agreement>: Issue 2-2-1: Assumption for feasibility study: RF chain status when performing enhanced measurement

In the 1st round, majority companies support option 2. Two companies point out that one RF chain shall also be allowed for UE only capable of intra-band CA. One company proposes multiple (≥ 2) RF Chains may also be feasible.

Please provide further comments on the following options

  - Option 1 (Apple, HW): One active RF chain

  - Option 2 (CMCC, xiaomi, MTK, QC, Ericsson, vivo): Two active RF chains

  - Option 3 (Nokia): more than two active RF chains

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-2-2: Assumption for feasibility study: number of frequency layers

In the 1st round, majority companies think “Reduce the number of EMR carriers to be measured for improved measurement” is a potential direction. Some companies point out that there are some issues for further discussion, i.e. how to select the frequency layers for improved measurement, how many frequency layers to measure, which scenario that is applicable.

As option 2 in issue 2-2-6 “UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes” is about how to select the frequency layers for improved measurement. Moderator suggests to merge it in this issue.

Please provide further comments on the following options

  - Revised Option 1: Reduce the number of EMR carriers to be measured for improved measurement.

    - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable.

      - Option 1a (Nokia): UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-2-3: Assumption for feasibility study: Reduced number of samples

Majority companies have concern on measurement accuracy with reduced number of samples. Almost all the companies emphasize that measurement accuracy should be guaranteed.

Please provide further comments on the following options

  - Option 1: Yes

  - Option 2 (MTK, Apple, HW, CATT ): No

  - Option 3 (CMCC, xiaomi, QC, Ericsson, vivo, Nokia): FFS how to reduce number of samples without measurement accuracy degradation.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-2-4: Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1(CATT, Nokia): Reduce the scaling factor of Rx beam sweeping

    - Option 1a (CMCC): use Rx beam sweeping factor of R17 HST FR2 or R17 positioning

      - For FR2 HST, the value of scaling factor is 2 or 6 pending on the different deployment.

      - In Rel-17 positioning WI, the candidate Rx beam sweep numbers for reduced Rx beam sweeping factor (<8) UE capability are {1, 2, 4, 6}.

    - Option 1b (xiaomi, Ericsson, vivo): Use prior information on the UE Rx beam to reduce the scaling factor of Rx beam sweeping

  - Option 2 (MTK, Apple, HW): Not to reduce the scaling factor of Rx beam sweeping

  - Option 3 (QC): Further discussion

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-2-5: Assumption for feasibility study: Configuration assumption

All companies support that DRX is not in use during RRC connection setup/resume procedure for enhanced measurement.

Tentative agreements in the 1st round: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement.

Regarding not using SMTC, there is no tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (vivo): use SMTC when specifying the requirements

  - Option 2 (Ericsson ): use SSB period when specifying the requirements

  - Option 3 (CMCC, MTK, HW): Further discussion

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-2-6: Assumption for feasibility study: others

As option 1 is covered by issue 2-2-2 and 2-1-3 based on Ericsson’s clarification. Moderator suggests to discuss in issue 2-2-2 and 2-1-3 and not to discuss in this issue in the 2nd round.

Clarification from Ericsson on Option 1 For Option 1 as we discuss in Issue 2-2-2: Assumption for feasibility study: number of frequency layers, potential priority of configuration can be provided from network side. (Moderator: discussed in issue 2-2-2) Also, as EMR is associate with T331 timer, as time adaptive enhanced measurement requirement can be introduced to guarantee network can use the measurement results at least of UE measurement effort. (Moderator: discussed in issue 2-1-3)

Regarding Option 2, there is no tentative agreements. In my understanding, option 2“UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes” is about how to select the frequency layers for improved measurement. Moderator suggests to merge it in issue 2-2-2.

No further discussion.

## Sub-topic 2-3: Feasibility discussion

<Way forward/Agreement>: Issue 2-3-1: Whether RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay

In the 1st round, all companies agree that RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay. Two companies propose to further study short and accuracy measurement during RRC connection setup/resume.

Tentative agreements in the 1st round: RRC connection setup/resume delay is very short for improvement on FR2 Scell/SCG setup delay.

Please provide further comments on the following option

  - Option 1 (Ericsson, Nokia): Study short and accuracy measurement during RRC connection setup/resume.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-3-2: Impact on RACH due to measurement during RRC connection setup/resume

In the 1st round, it seems no company opposes that MSG2/MSG4 during RACH procedure may be lost due to Rx beam sweeping for measurement on FR2 intra-band inter-frequency during RRC connection setup/resume. Two companies also point out that FR2 intra-band CA is not the target scenario of enhanced measurement.

Some company proposes “Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning”. In my understanding, the latter is mainly focus on inter-band inter-frequency measurement.

Some company also propose to study potential mitigation schemes.

Tentative agreements in the 1st round: MSG2/MSG4 during RACH procedure may be lost due to measurement on FR2 intra-band inter-frequency during RRC connection setup/resume.

In the 2nd round, moderator recommends to further discuss on the new options.

  - Option 1 (MTK): For inter-band inter-frequency measurement, Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning.

  - Option 2 (vivo, Nokia): FR2 intra-band CA is not the target scenario of enhanced measurement.

  - Option 3 (Ericsson): Study potential mitigation schemes to avoid/mitigate the impact on RACH due to measurement during RRC connection setup/resume.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 2-3-3: Feasibility of improvement in FR2 SCell/SCG setup delay

Tentative agreements in the 1st round: Further discuss the feasibility of improvement in FR2 SCell/SCG setup delay.

No further discussion.

# Topic #3: L1/L2 based inter-cell mobility

## Sub-topic 3-1: RRM requirements to specify

<Way forward/Agreement>: Issue 3-1-1: L1/L2 inter-cell mobility delay requirements

Tentative agreements in the 1st round:

Specify L1/L2 inter-cell mobility delay and analyse each component of L1/L2 inter-cell mobility delay.

No further discussion.

<Way forward/Agreement>: Issue 3-1-2: inter-cell mobility interruption requirements

No companies oppose Tinterrupt in mobility delay requirements is considered in issue 3-1-1. Ericsson clarifies interruption due to MG for inter-frequency measurement may need to be considered. Since interruption due to MG is well captured in spec, Moderator recommends not to discuss this issue in 2nd round.

No further discussion.

<Way forward/Agreement>: Issue 3-1-3: L1-RSRP measurement delay requirements

In the 1st round, majority companies propose to further discuss this issue when the procedure and supported scenario is clear. Moderator recommends to come back to this issue after the procedure and supported scenario is clear.

No further discussion.

<Way forward/Agreement>: Issue 3-1-4: Timing requirements

Tentative agreements in the 1st round: Wait for RAN1/2 input and then discuss whether and how to define timing management requirements.

No further discussion.

<Way forward/Agreement>: Issue 3-1-5: Timing management requirements

Clarification from Ericsson in the 1st round In inter-cell BM, as only intra-frequency carrier was considered, we assumed that RTD between TRP are within the CP. When the inter-frequency measurements are considered, we are not sure if it can be within CP or can be more than CP. IF it can be more than CP, we need to specify MRTD applicability for measurements.

Please provide further comments on the following option

  - Revised Option 1 (Ericsson): Specify MRTD of serving cell and neighbour cell if covering the scenario that time offset of serving cell and neighbour cell is more than CP.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

## Sub-topic 3-2: Scenarios

<Way forward/Agreement>: Issue 3-2-1: Whether to consider simultaneous Rx/Tx with both source cell and target cell

In the 1st round, majority companies support Option 1. One company asks for more clarification on Option 1.

Please provide further comments on the following option

  - Option 1 (MTK, Apple, vivo, QC, HW, Ericsson, LGE, CATT): Not consider simultaneous Rx/Tx with both source cell and target cell.

Note: Proponents of option 1 should clarify more on “simultaneous Rx/Tx with both source cell and target cell” in the 2nd round.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-2-2: Whether to consider simultaneous multi-panel in FR2

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (MTK, Apple, vivo, QC, HW, LGE): Not consider simultaneous multi-panel in FR2

  - Option 2 (Ericsson, Nokia): Consider simultaneous multi-panel in FR2

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-2-3: Definition of intra-frequency/inter-frequency in inter-cell operation

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (vivo, Ericsson, CATT, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation

    - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-2-4: Whether to cover inter-frequency

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (MTK, vivo, QC, apple): Further discuss the necessity, feasibility, and pros/cons of specifying inter-frequency L1-RSRP measurement.

  - Option 2 (CMCC, HW, Ericsson, Nokia): Yes

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-2-5: Whether to cover non-synchronous scenarios

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1: Not consider non-synchronous scenarios.

  - Option 2 (Ericsson, Nokia): Consider non-synchronous scenarios.

  - Option 3 (HW, CATT): FFS

    - Option 3a (MTK, vivo): discuss the definition of synchronous and non-synchronous

    - Option 3b (Apple, QC): wait for RAN2’s progress

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-2-6: Whether to support L1 measurement on multiple cells with PCI different from serving cell

Tentative agreements in the 1st round: Further discuss the number of supported cells with PCI different from serving cell.

Please provide further comments on the following options

  - Option 1(Apple, HW, vivo, Ericsson, CATT, Nokia): to discuss the number of supported cells with PCI different from serving cell.

    - Option 1a (MTK): To discuss the number of supported cells with PCI different from serving cell on a FR2 band

    - Option 1b (QC): To discuss the total number of cells to be monitored on all bands

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-3-1: Intra-frequency L1-RSRP measurement accuracy requirements

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (Ericsson, Nokia, vivo): Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility

  - Option 2 (MTK, Apple, QC, CATT): reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

<Way forward/Agreement>: Issue 3-3-2: Inter-frequency L1-RSRP measurement accuracy requirements

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (Ericsson, Nokia): Define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell

    - Option 1a (QC, CATT): reuse the legacy intra-frequency L1-RSRP measurement accuracy requirements.

  - Option 2 (Apple, vivo): Further discuss whether and how to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell in performance part.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |

# Topic #4: CHO/CPAC Enhancement

## Sub-topic 4-1: subsequent CPAC

<Way forward/Agreement>: Issue 4-1-1: RRM requirements for subsequent CPAC

In the 1st round, all companies are fine to wait for RAN2 input to further discuss RAN4 impact.

Tentative agreements: wait for RAN2 input to further discuss RRM requirements for subsequent CPAC.

No further discussion.

## Sub-topic 4-2: CHO with CPAC

<Way forward/Agreement>: Issue 4-2-1: RRM requirements for CHO with CPAC

In the 1st round, all companies are fine to wait for RAN2 input to further discuss RAN4 impact.

Tentative agreements: wait for RAN2 input to further discuss RRM requirements for CHO with CPAC.

No further discussion.