3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214348

Electronic Meeting, 15– 26 August 2022

Title: 	WF on Further NR Mobility Enhancements

Agenda Item:	11.16

Source: 	MediaTek Inc.

Document for:	Approval

# Topic #1: General and work plan

## Sub-topic 1-1: Work plan

<Way forward/Agreement>: Issue 1-1: Work plan proposals

In first round discussion, some companies have some concern on the work plan in R4-2211550.

For 2nd round: comment on the revised work plan in the 2nd round.

Note: please comment on the dedicated Email thread for work plan.

# Topic #2: Study of improvement on FR2 Scell/SCG setup/resume

## Sub-topic 2-1: Clarification and potential direction

<Way forward/Agreement>: Issue 2-1-1: Carification on time point of “when UE has initiated access”

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1(Ericsson, Nokia): RAN4 to clarify that the “when UE has initiated access” is the point in time when RRC procedures in clauses 5.3.2.3, 5.3.3.2 or 5.3.13.2 (38.331) are initiated.

    - Paging

    - Initiation (UE receives SIB1 and apply default MAC cell group configuration)

    - RRC connection resume

  - Option 2(MTK, CMCC, Apple, xiaomi, HW, vivo, CATT): Discuss the starting point for improved measurement directly.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Prefer option 2. Option 1 is overlapped with issue 2-1-2 as different options under issue 2-1-2 capture different part of option 1 here. It could be simpler for us to focus on one issue. |
| Qualcomm | Although we understand the point of Option 1, we don’t think such a clarification is essential. |

<Way forward/Agreement>: Issue 2-1-2: Potential direction for further study: measurement enhancement when UE is about to enter connected mode

No tentative agreements in 1st round.

Please provide further comments on the following options

  - Option 1(CMCC): UE perform enhanced measurement closely before RRC connection setup/resume.

    - For MT originating call, the measurement is performed during the period between paging reception and UE send RRCResumeRequest/ RRCSetupRequest.

    - For MO originating call, the measurement is performed after upper layers request establishment of an RRC connection and before UE send RRCResumeRequest/ RRCSetupRequest.

  - Option 2: UE perform enhanced measurement during RRC connection setup/resume.

    - Option 2a (Nokia): perform FR2 cell measurements during connection setup (i.e. paging, RRC connection establishment, RRC connection resume)

    - Option 2b (MTK, vivo, Ericsson ): after receiving paging for MT originating call

    - Option 2c (MTK, HW, vivo): after first RACH preamble transmission, i.e. Msg1

    - Option 2d (Ericsson): Initiation (UE receives SIB1 and apply default MAC cell group configuration) for MO originating call

    - Option 2e (Apple, xiaomi, QC): FFS

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | We think RAN4 may need more time to identify/compare the gain of different options. At current stage option 2 is preferred. Option 2b could be the earliest point for MT call and option 2c could be the earliest point which can be verified for MO call. In option 1 the measurement time is limited to the window between paging (upper layers request establishment) and RRC request. We think the window may be too short for accurate measurement even for a single carrier, not to mention the case wherein UE needs to measure multiple carriers as commented by some other companies. |
| Qualcomm | We do not disagree with any particular option at the moment. However, Option 1 seems to be limiting the way RAN4 can discuss for the enhancement too much just based on the wording in the option. |

<Way forward/Agreement>: Issue 2-1-3: Potential direction for further study: enhancement on R16 EMR, i.e. measurement enhancement in idle/inactive mode

According to the 1st round discussion, the most divergence on this issue is whether further enhancement on R16 EMR (measurement during green part) is in the scope. Moderator suggests to align the understanding on this at first before go to more details.

  - Option 1 (MTK, Apple, xiaomi, HW, vivo): further enhancement on Rel-16 EMR (measurement during green part) is out of scope

  - Option 2 (CMCC, QC, Ericsson, Nokia, LGE, CATT): further enhancement on Rel-16 EMR (measurement during green part) is in the scope

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | In our understanding the objective in this WI is mainly for the case wherein EMR measurement becomes inaccurate after T331 expires. Otherwise, if the T331 is still running, NW can use EMR result and thereby no need introduce new measurement. We are open to study if RAN4 can borrow some outcome of EMR procedure. However, it doesn’t mean we need to optimize EMR procedure. That is out of scope of this WI. |
| Qualcomm | Echo Apple’s comment. And we are not sure if companies are on the same page about the definition of “enhancement on Rel-16 EMR.” If Option 1 precludes a possibility of UE utilizing the measurement results obtained during EMR during RRC connection procedure, we oppose Option 1. |

<Way forward/Agreement>: Issue 2-1-4: Applicable scenarios

Tentative agreements in the 1st round: The enhanced measurement (if feasible) at least applies to scenarios that the EMR measurement results are unavailable or invalid.

Please provide further comments on the following option

  - Revised Option 2 (QC, vivo, LGE, Nokia):  Further study whether enhanced measurement is applicable to scenarios that EMR measurement results are available and valid

    - e.g. On the cells that have been detected/measured in early measurement

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | More justification is needed on option 2. In our view, when EMR result is still available, NW can directly use EMR result for CA/DC configuration. |
| Qualcomm | Support Option 2. Regarding the wording “available,” our interpretation is that UE has managed to finish measurements on EMR carriers and has the results in a memory. The question would be about whether the result is “valid” or not from UE perspective and NW perspective, e.g. the result may or may not be valid by the time from NW perspective when UE goes into Connected mode. The details can be further discussed in the next meeting. |

## Sub-topic 2-2: Assumptions for feasibility study

<Way forward/Agreement>: Issue 2-2-1: Assumption for feasibility study: RF chain status when performing enhanced measurement

In the 1st round, majority companies support option 2. Two companies point out that one RF chain shall also be allowed for UE only capable of intra-band CA. One company proposes multiple (≥ 2) RF Chains may also be feasible.

Please provide further comments on the following options

  - Option 1 (Apple, HW): One active RF chain

  - Option 2 (CMCC, xiaomi, MTK, QC, Ericsson, vivo): Two active RF chains

  - Option 3 (Nokia): more than two active RF chains

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Option 1 shall be the baseline assumption. Using multiple RF chains for simultaneous measurement on multiple carriers means result in even tightened RRM requirement than connected mode. We fail to understand the necessity of this. As we mentioned in the 1st round, the new measurement is only for potential CA/DC operation, it is likely that NW isn’t urgent to use CA/DC after RRC connection setup. The measurement is configured when UE leaves connected mode. However, network cannot predict there will be huge traffic data after UE sleep in idle. We consider this as “less ungent” than UE in connected mode. In connected mode, NW shall have better idea on whether and when UE needs to be configured with CA/DC. |
| Qualcomm | Our comment in the first round doesn’t seem to be captured correctly. We are in favor of Option 1 as a starting point. |

<Way forward/Agreement>: Issue 2-2-2: Assumption for feasibility study: number of frequency layers

In the 1st round, majority companies think “Reduce the number of EMR carriers to be measured for improved measurement” is a potential direction. Some companies point out that there are some issues for further discussion, i.e. how to select the frequency layers for improved measurement, how many frequency layers to measure, which scenario that is applicable.

As option 2 in issue 2-2-6 “UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes” is about how to select the frequency layers for improved measurement. Moderator suggests to merge it in this issue.

Please provide further comments on the following options

  - Revised Option 1: Reduce the number of EMR carriers to be measured for improved measurement.

    - FFS: how to select the frequency layers for improved measurement, how many frequency layers to measure and which scenario that is applicable.

      - Option 1a (Nokia): UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | The revised option 1 could be a good starting point when RAN4 discuss corresponding measurement capability, i.e. number of carriers to measure in the new measurement requirements. Regarding option 1a, it is not supported considering UE power consumption. To us it effectively extends T331. |
| Qualcomm | In principle, Option 1 is okay. Option 1a is not yet clear to us in terms of UE measurement behavior and benefits because it only tells “measurement configuration” |

<Way forward/Agreement>: Issue 2-2-3: Assumption for feasibility study: Reduced number of samples

Majority companies have concern on measurement accuracy with reduced number of samples. Almost all the companies emphasize that measurement accuracy should be guaranteed.

Please provide further comments on the following options

  - Option 1: Yes

  - Option 2 (MTK, Apple, HW, CATT ): No

  - Option 3 (CMCC, xiaomi, QC, Ericsson, vivo, Nokia): FFS how to reduce number of samples without measurement accuracy degradation.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Support option 2. Option 3 is interesting. We are open to it. |
| Qualcomm | To us, Option 3 is more or less a question rather than something that we support. What is the domain when we say “number of samples”? Is it the number of SSB bursts or UE Rx beam sweeping or something else? And RAN4 spec only specifies the worst latency, not the number of SSB bursts for the measurement. UE can measure more samples as needed within the latency. |

<Way forward/Agreement>: Issue 2-2-4: Assumption for feasibility study: Reduce the scaling factor of Rx beam sweeping

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1(CATT, Nokia): Reduce the scaling factor of Rx beam sweeping

    - Option 1a (CMCC): use Rx beam sweeping factor of R17 HST FR2 or R17 positioning

      - For FR2 HST, the value of scaling factor is 2 or 6 pending on the different deployment.

      - In Rel-17 positioning WI, the candidate Rx beam sweep numbers for reduced Rx beam sweeping factor (<8) UE capability are {1, 2, 4, 6}.

    - Option 1b (xiaomi, Ericsson, vivo): Use prior information on the UE Rx beam to reduce the scaling factor of Rx beam sweeping

  - Option 2 (MTK, Apple, HW): Not to reduce the scaling factor of Rx beam sweeping

  - Option 3 (QC): Further discussion

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Support option 2 and 3. We are not clear at current stage based on what we can reduce the scaling factor of Rx beam sweeping compared to the connected mode. In other word, if we can reduce it in this procedure, could we also reduce it in connected mode in legacy requirements? that sounds more like an enhancement on FR2 measurement, rather than an enhancement particular during RRC connection setup. |
| Qualcomm | Okay with Option 1b as a candidate mechanism. But it is too early to agree on it without “FFS” or “may” in it. |

<Way forward/Agreement>: Issue 2-2-5: Assumption for feasibility study: Configuration assumption

All companies support that DRX is not in use during RRC connection setup/resume procedure for enhanced measurement.

Tentative agreements in the 1st round: DRX is not in use during RRC connection setup/resume procedure for enhanced measurement.

Regarding not using SMTC, there is no tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (vivo): use SMTC when specifying the requirements

  - Option 2 (Ericsson ): use SSB period when specifying the requirements

  - Option 3 (CMCC, MTK, HW): Further discussion

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Further discussion is needed, especially for the potential impact on RRC connection setup due to measurement on other carriers. |
| Qualcomm | For now, not comfortable with Option 2. Can we assume UE will know actual SSB periodicity of non-serving cell during RRC connection procedure? |

<Way forward/Agreement>: Issue 2-2-6: Assumption for feasibility study: others

As option 1 is covered by issue 2-2-2 and 2-1-3 based on Ericsson’s clarification. Moderator suggests to discuss in issue 2-2-2 and 2-1-3 and not to discuss in this issue in the 2nd round.

Clarification from Ericsson on Option 1 For Option 1 as we discuss in Issue 2-2-2: Assumption for feasibility study: number of frequency layers, potential priority of configuration can be provided from network side. (Moderator: discussed in issue 2-2-2) Also, as EMR is associate with T331 timer, as time adaptive enhanced measurement requirement can be introduced to guarantee network can use the measurement results at least of UE measurement effort. (Moderator: discussed in issue 2-1-3)

Regarding Option 2, there is no tentative agreements. In my understanding, option 2“UE can be configured to maintain measurement configuration of previous serving cells for EMR purposes” is about how to select the frequency layers for improved measurement. Moderator suggests to merge it in issue 2-2-2.

No further discussion.

## Sub-topic 2-3: Feasibility discussion

<Way forward/Agreement>: Issue 2-3-1: Whether RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay

In the 1st round, all companies agree that RRC connection setup delay is very short for improvement on FR2 Scell/SCG setup delay. Two companies propose to further study short and accuracy measurement during RRC connection setup/resume.

Tentative agreements in the 1st round: RRC connection setup/resume delay is very short for improvement on FR2 Scell/SCG setup delay.

Please provide further comments on the following option

  - Option 1 (Ericsson, Nokia): Study short and accuracy measurement during RRC connection setup/resume.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Open for further study. It is challenging to achieve accurate measurement during such a short time in FR2. |
| Qualcomm | It is unclear whether Option 1 aims for a quick cold start measurement or not, i.e. no previous measurement results or side information to be utilized at all? |

<Way forward/Agreement>: Issue 2-3-2: Impact on RACH due to measurement during RRC connection setup/resume

In the 1st round, it seems no company opposes that MSG2/MSG4 during RACH procedure may be lost due to Rx beam sweeping for measurement on FR2 intra-band inter-frequency during RRC connection setup/resume. Two companies also point out that FR2 intra-band CA is not the target scenario of enhanced measurement.

Some company proposes “Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning”. In my understanding, the latter is mainly focus on inter-band inter-frequency measurement.

Some company also propose to study potential mitigation schemes.

Tentative agreements in the 1st round: MSG2/MSG4 during RACH procedure may be lost due to measurement on FR2 intra-band inter-frequency during RRC connection setup/resume.

In the 2nd round, moderator recommends to further discuss on the new options.

  - Option 1 (MTK): For inter-band inter-frequency measurement, Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning.

  - Option 2 (vivo, Nokia): FR2 intra-band CA is not the target scenario of enhanced measurement.

  - Option 3 (Ericsson): Study potential mitigation schemes to avoid/mitigate the impact on RACH due to measurement during RRC connection setup/resume.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Option 1 is more like an observation, which is valid. We are open to discuss how to avoid/mitigate the impact as included in option 3. |
| Qualcomm | Do not disagree with Option 1 and 2. Option 3 is too much open ended to agree it in this meeting. |

<Way forward/Agreement>: Issue 2-3-3: Feasibility of improvement in FR2 SCell/SCG setup delay

Tentative agreements in the 1st round: Further discuss the feasibility of improvement in FR2 SCell/SCG setup delay.

No further discussion.

# Topic #3: L1/L2 based inter-cell mobility

## Sub-topic 3-1: RRM requirements to specify

<Way forward/Agreement>: Issue 3-1-1: L1/L2 inter-cell mobility delay requirements

Tentative agreements in the 1st round:

Specify L1/L2 inter-cell mobility delay and analyse each component of L1/L2 inter-cell mobility delay.

No further discussion.

<Way forward/Agreement>: Issue 3-1-2: inter-cell mobility interruption requirements

No companies oppose Tinterrupt in mobility delay requirements is considered in issue 3-1-1. Ericsson clarifies interruption due to MG for inter-frequency measurement may need to be considered. Since interruption due to MG is well captured in spec, Moderator recommends not to discuss this issue in 2nd round.

No further discussion.

<Way forward/Agreement>: Issue 3-1-3: L1-RSRP measurement delay requirements

In the 1st round, majority companies propose to further discuss this issue when the procedure and supported scenario is clear. Moderator recommends to come back to this issue after the procedure and supported scenario is clear.

No further discussion.

<Way forward/Agreement>: Issue 3-1-4: Timing requirements

Tentative agreements in the 1st round: Wait for RAN1/2 input and then discuss whether and how to define timing management requirements.

No further discussion.

<Way forward/Agreement>: Issue 3-1-5: Timing management requirements

Clarification from Ericsson in the 1st round In inter-cell BM, as only intra-frequency carrier was considered, we assumed that RTD between TRP are within the CP. When the inter-frequency measurements are considered, we are not sure if it can be within CP or can be more than CP. IF it can be more than CP, we need to specify MRTD applicability for measurements.

Please provide further comments on the following option

  - Revised Option 1 (Ericsson): Specify MRTD of serving cell and neighbour cell if covering the scenario that time offset of serving cell and neighbour cell is more than CP.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Further discussion is needed. currently MRTD requirements only apply for serving cells. Even with the clarification in the 1st round, it is still unclear to us why RAN4 needs to define MRTD between serving cell and neighbour cell, as most companies support not to consider simultaneous Rx/Tx during this procedure. |
| Qualcomm | Not clear yet. Whether serving and the candidate cells for L1/L2 mobility are in the same carrier, band, FR, etc is not clear. |

## Sub-topic 3-2: Scenarios

<Way forward/Agreement>: Issue 3-2-1: Whether to consider simultaneous Rx/Tx with both source cell and target cell

In the 1st round, majority companies support Option 1. One company asks for more clarification on Option 1.

Please provide further comments on the following option

  - Option 1 (MTK, Apple, vivo, QC, HW, Ericsson, LGE, CATT): Not consider simultaneous Rx/Tx with both source cell and target cell.

Note: Proponents of option 1 should clarify more on “simultaneous Rx/Tx with both source cell and target cell” in the 2nd round.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Support option 1. We are not sure which part needs to be clarified. In our view, option 1 means UE is not expected to conduct simultaneous Rx/Tx with both source cell and target cell during the mobility procedure. In other word, UE only needs to conduct Rx/Tx with one single cell. |
| Qualcomm | Not clear the definition of “simultaneous Rx/Tx.” While UE receives signals/channels or transmits signals/channels from/to the current PCell, if the UE can measure SSB from the candidate cell, is this considered “simultaneous Rx/Tx”? What about the case one of the candidate cells is actually the UE’s current SCell? This is also one of scenarios in the scope in RAN2. |

<Way forward/Agreement>: Issue 3-2-2: Whether to consider simultaneous multi-panel in FR2

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (MTK, Apple, vivo, QC, HW, LGE): Not consider simultaneous multi-panel in FR2

  - Option 2 (Ericsson, Nokia): Consider simultaneous multi-panel in FR2

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Support option 1 as for baseline UE. Multi-panel is being discussed in other R18 WIs. It is premature to conclude at current stage that we will include the outcome of other ongoing WI. |
| Qualcomm | Support Option 1. |

<Way forward/Agreement>: Issue 3-2-3: Definition of intra-frequency/inter-frequency in inter-cell operation

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (vivo, Ericsson, CATT, Nokia): RAN4 to discuss the definition of intra-frequency/inter-frequency in inter-cell operation

    - Option 1a (MTK): For SSB L1-RSRP measurement, follow the definition of L3 measurement.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Open for further discussion. |
| Qualcomm | No view on Options yet. To us, this is a second level of details that can be discussed later. |

<Way forward/Agreement>: Issue 3-2-4: Whether to cover inter-frequency

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (MTK, vivo, QC, apple): Further discuss the necessity, feasibility, and pros/cons of specifying inter-frequency L1-RSRP measurement.

  - Option 2 (CMCC, HW, Ericsson, Nokia): Yes

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Open for further discussion. |
| Qualcomm | We don’t think this is “yes” or “no” question at least for now. |

<Way forward/Agreement>: Issue 3-2-5: Whether to cover non-synchronous scenarios

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1: Not consider non-synchronous scenarios.

  - Option 2 (Ericsson, Nokia): Consider non-synchronous scenarios.

  - Option 3 (HW, CATT): FFS

    - Option 3a (MTK, vivo): discuss the definition of synchronous and non-synchronous

    - Option 3b (Apple, QC): wait for RAN2’s progress

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Support option 3a and 3b. This can be revised after the procedure becomes stable and clear. For instance, if the procedure requires tight synchronization, probably async would be out of the scope. But of course the definition of sync and async needs to be discussed. |
| Qualcomm | Option 3b |

<Way forward/Agreement>: Issue 3-2-6: Whether to support L1 measurement on multiple cells with PCI different from serving cell

Tentative agreements in the 1st round: Further discuss the number of supported cells with PCI different from serving cell.

Please provide further comments on the following options

  - Option 1(Apple, HW, vivo, Ericsson, CATT, Nokia): to discuss the number of supported cells with PCI different from serving cell.

    - Option 1a (MTK): To discuss the number of supported cells with PCI different from serving cell on a FR2 band

    - Option 1b (QC): To discuss the total number of cells to be monitored on all bands

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Open to further discussion. Existing requirement for L3 measurement is per layer. Maybe we can start from that. |
| Qualcomm | Do want to discuss this level of details for now. |

<Way forward/Agreement>: Issue 3-3-1: Intra-frequency L1-RSRP measurement accuracy requirements

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (Ericsson, Nokia, vivo): Discuss whether intra-frequency L1-RSRP measurement accuracy can be improved for L1/L2 mobility

  - Option 2 (MTK, Apple, QC, CATT): reuse the legacy intra-frequency L1-RSRP measurement accuracy for L1/L2 mobility.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | This shall be discussed in performance part. Anyway, if side condition is same a legacy, we need to apply the legacy accuracy requirement. |
| Qualcomm | The issue can be discussed later. If the group agrees to consider a better side condition in terms of SNR just in L1/L2 mobility context, the accuracy may be improved. Otherwise, Option 1 doesn’t seem to be the one that the group needs to discuss. This WI is not about L1-RSRP measurement improvement. |

<Way forward/Agreement>: Issue 3-3-2: Inter-frequency L1-RSRP measurement accuracy requirements

No tentative agreements in the 1st round.

Please provide further comments on the following options

  - Option 1 (Ericsson, Nokia): Define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell

    - Option 1a (QC, CATT): reuse the legacy intra-frequency L1-RSRP measurement accuracy requirements.

  - Option 2 (Apple, vivo): Further discuss whether and how to define inter-frequency L1-RSRP measurement accuracy requirements on non-serving cell in performance part.

(The below table is to be moved to 2nd round summary and removed in the formal WF)

| Company | Comments |
| --- | --- |
| Apple | Option 2. This shall be discussed in performance part. So far we don’t know whether the L1-RSRP measurement on inter-frequency is expected to be same as intra-frequency, e.g. side condition, measurement latency and etc. |
| Qualcomm | Let’s skip this sort of discussion. We do not want to spend time/effort discussing such a high level topic without any detailed context in terms of many. |

# Topic #4: CHO/CPAC Enhancement

## Sub-topic 4-1: subsequent CPAC

<Way forward/Agreement>: Issue 4-1-1: RRM requirements for subsequent CPAC

In the 1st round, all companies are fine to wait for RAN2 input to further discuss RAN4 impact.

Tentative agreements: wait for RAN2 input to further discuss RRM requirements for subsequent CPAC.

No further discussion.

## Sub-topic 4-2: CHO with CPAC

<Way forward/Agreement>: Issue 4-2-1: RRM requirements for CHO with CPAC

In the 1st round, all companies are fine to wait for RAN2 input to further discuss RAN4 impact.

Tentative agreements: wait for RAN2 input to further discuss RRM requirements for CHO with CPAC.

No further discussion.