3GPP TSG-RAN WG4 Meeting #104-e 											  R4-22xxxxx

Electronic Meeting, Aug. 15 – 26, 2022

Title: 	WF on Feature Group 6-1a

Agenda Item:	13.4

Source: 	Apple

Document for:	Approval

# Wayforward

Issue 1: whether it is a valid scenario in the standard to support the operation of BWP without SSB where the UE does not perform BM/RLM/BFD due to the lack of necessary reference signal (SSB and CSI-RS) in the active BWP

Agreement:

From RAN4 specification point of view, it is not a valid scenario

Issue 2: how should the UE perform BM/RLM/BFD when the active BWP does not contain SSB

Tentative Agreement:

RAN4 has examined the Rel-15, Rel-16 and Rel-17 specs.  The following possible solutions for the issue are identified.

- Perform BM/RLM/BFD based on CSI-RS within active BWP

  - RAN4 has requirements to support BM/RLM/BFD based on CSI-RS within active BWP and no spec change is needed

- Other than the CSI-RS based solution, in order to support the concerning scenario, RAN4 discussed the following potential independent implementations/features which require existing RAN4 requirements to be updated and developed.

  - UE’s capability to operate using larger BW, or a UE that is equipped with a separate RF chain

    - Depending on UE implementation, UE may or may not cause interruption on other active serving cell(s) due to BM/RLM/BFD based on SSB outside active BWP. For those UEs not causing interruptions on an any active serving cell due to BM/RLM/BFD based on SSB outside active BWP, minor RAN4 requirement updates are expected. However, the power consumption of this implementation can be higher and the power saving gain from BWP operation may be compromised.

  - BM/RLM/BFD on SSB outside BWP are performed with shared MG or NCSG for L3 measurement, or dedicated MG or NCSG for RLM/BFD/BM measurements.

  - NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

  - Note: RAN4 observation is that the above listed independent implementations/features do not have proper placeholder as part of RAN4 work in Rel-17 since RAN4 requirements may need to be developed to support the features

Issue 3: in which release and how to introduce enhanced RRM requirements to support Feature Group 6-1a “bwp-WithoutRestriction”?

Tentative Agreement:

- Option 1: continue discussion in Rel-17 under TEI17

- Option 2: In Rel-18 under the umbrella WI “Rel-18 RRM enhancement”

  - The support of Feature Group 6-1a “bwp-WithoutRestriction” in Rel-17 is left to implementation.

- Option 3: Feature Group 6-1a “bwp-WithoutRestriction” with mandatory supporting FG 1-7 and /or 2-31, and the corresponding requirements can already be supported from Rel-15.

- Option 3a: Feature Group 6-1a “bwp-WithoutRestriction” with mandatory supporting FG 1-7 and /or 2-31 and/or FG-24, and the corresponding requirements can already be supported from Rel-15.

  - For FR2: the CSI-RS repetition is on (FFS).

- Option 4: Leave it to RAN decision.

Issue 4: scope of the RAN4 discussion

Agreement:

- Only consider non-RedCap UEs where RedCap UEs is out of scope.

Issue 5: LS to other working groups

Agreement:

- RAN4 will inform RAN2 and RANP with agreements under issue 1 and 2.

# Reference

- R4-2207617, LS on BWP operation without bandwidth restriction, RAN2

- RP-221870, NCD-SSB for all devices: proposed Way Forward, RANP