3GPP TSG-RAN WG4 Meeting # 104-e	R4-2214360

Electronic Meeting, 15– 26 August 2022

Title: 	WF on NR MIMO OTA

Agenda Item:	9.1.4

Source: 	CAICT, vivo

Document for:	Approval

# Topic #1: General and Testing methodology maintenance

## Sub-topic 1-1 LS on NR MIMO OTA progress

Issue 1-1: LS on NR MIMO OTA progress

Agreements:

- Send an LS on NR MIMO OTA progress to RAN5, CTIA MOSG and CCSA TC9 WG1.

- The LS in R4-22XXXX is approved.

## Sub-topic 1-3 Pass/fail limits for FR2 channel model validation

Issue 1-3-1: PDP pass/fail limits for FR2 channel model validation

Agreement:

- Remove the square brackets in Table D.4.2-1, i.e., the PDP pass/fail limits for FR2 CDL-C UMi channel model validation are specified as below.

Table D.4.2-1: PDP pass/fail limits for FR2 CDL-C UMi channel model validation

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

Issue 1-3-2: Wording of the Temporal Correlation pass/fail limits for FR2

Agreements:

- Adopt the following wording of the Temporal Correlation pass/fail limits for FR1:

- The pass/fail limits for theoretical temporal correlation defined in Clause C.3.3 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

- Adopt the following wording of the Temporal Correlation pass/fail limits for FR2:

- The pass/fail limits for theoretical temporal correlation defined in Clause D.3.3 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end.

Issue 1-3-3: Tighten the Temporal Correlation pass/fail limits for FR2

Agreements:

- Keep the temporal correlation pass/fail limits the same for FR1 and FR2, i.e., not to tighten the temporal correlation pass/fail limits for FR2.

- It is not excluded to further tighten this value (if necessary) when more validation results are obtained.

Issue 1-3-4: Cross-polarization (V/H) pass/fail limits for FR2 channel model validation

Agreement:

- Remove the square brackets, i.e., the cross-polarization ratio pass/fail limit is specified as ±1.5 dB.

Issue 1-3-5: PSP pass/fail limits for FR2 channel model validation

Agreement:

- Remove the square brackets, i.e., the PSP pass/fail limit is specified as 84%.

# Topic #2: FR1 Performance requirements

## Sub-topic 2-1 Figure of Merits for FR1 MIMO OTA

Issue 2-1-1: Maximum downlink power and additional criterion for FR1 MIMO OTA

Agreements:

- Remove the square brackets and confirm the maximum downlink RS-EPRE as -80dBm/15kHz (or equivalent -77dBm/30kHz) for FR1 MIMO OTA.

- Proposal 2: Remove the square brackets in additional criterion. The EUT must meet 90% throughput in 10 of total 12 azimuthal orientations.

- Define the same criterion on 90%TP for bands ≥3GHz and bands <3GHz.

## Sub-topic 2-2 FR1 MIMO OTA lab alignment

Issue 2-2-1: Pass/fail limit for FR1 MIMO OTA lab alignment

Agreement:

- Keep the current FR1 lab alignment pass/fail limit unchanged as +/- 0.75MU, i.e., +/- 2.25 dB for band <3GHz and +/- 2.55 dB for band >3GHz.

Issue 2-2-2: FR1 MIMO OTA lab alignment outcome

Agreements:

- 3GPP FR1 MIMO OTA lab alignment among the 6 labs, i.e., CAICT, CMCC&BUPT, Huawei, MediaTek, Xiaomi, and Apple is confirmed.

- 3GPP FR1 MIMO OTA lab alignment activity is closed in RAN4#104-e meeting.

## Sub-topic 2-3 FR1 MIMO OTA performance test campaign

Issue 2-3: TRMS measurement data for defining FR1 MIMO OTA performance requirements

Agreement:

- The TRMS measurement results in R4-2209330 (CAICT), R4-2209513 (Xiaomi), R4-2208413 (CMCC&BUPT), R4-2213204 (Xiaomi), R4-2211996 (Huawei), and R4-2212407 (Apple) are included in FR1 MIMO OTA data pool for defining performance requirements.

## Sub-topic 2-4 FR1 MIMO OTA performance requirements

Issue 2-4-2: Final values of TRMS requirements

GTW agreements:

- Further discuss TRMS requirements in Rel-17 MIMO OTA for n78 within below range:

  - n78: [-94.5 ~-95.5] dBm/30kHz

- Further discuss TRMS requirements in Rel-17 MIMO OTA for n41 with below range:

  - n41: [-93.0 ~ -93.5] dBm/30kHz

Tentative agreement: (compromised values based on the GTW agreements)

- Define FR1 MIMO OTA TRMS requirements as below:

    - n41: -93.3 dBm/30kHz

    - n78: -95.2 dBm/30kHz

## Sub-topic 2-5 Test Tolerance for FR1 MIMO OTA

Issue 2-5-1: Test Tolerance (TT) and MU assessment work in RAN4 and RAN5

Agreement:

- RAN4 should discuss recommended TT values for FR1 MIMO OTA, and provide the recommended TT values to RAN5.

Tentative agreement:

- The final MU and TT will be decided by RAN5, but it is suggested not to change the TT values for FR1 MIMO OTA TRMS in RAN5.

Issue 2-5-2: TT values for FR1 MIMO OTA

GTW agreement:

- RAN4 recommended TT values as:

  - Option 3b: Define TT values as 1.8dB for n41 and 2dB for n78.

Tentative agreement: (GTW agreement)

- Define recommended TT values for FR1 MIMO OTA as 0.6MU, i.e., 1.8dB for n41 and 2dB for n78.

# Topic #3: FR2 Performance requirements

## Sub-topic 3-1 General views on FR2 MIMO OTA requirements development

Issue 3-1: General views on FR2 MIMO OTA requirements development

Proposals from companies can be summarized as:

- Proposal 1a (Qualcomm, Keysight): Define FR2 MIMO OTA requirements based on simulation approach.

- Proposal 1b (Qualcomm, 2nd round): To specify the MASC requirements for 70% maximum throughput for n258 as [-138.6]dBm/Hz. Meanwhile to capture the following statement in the chairman notes: “The FR2 MIMO OTA requirements can be further revised if simulation results and/or measurement results are submitted by companies”.

- Proposal 2 (Huawei, Apple, Xiaomi, CAICT, OPPO, Samsung): Suggest to prioritize FR1 performance part work in Rel-17 timeline, and continue FR2 MIMO OTA requirements development in the future.

To conclude the Rel-17 NR MIMO OTA WI, companies are encouraged to reach high-level consensus on how to handle FR2 performance requirements development issues.

Tentative agreement:

- No agreements in RAN4 on FR2 MIMO OTA requirements. How to conclude the performance part of FR2 MIMO OTA depending on the RAN-P decision.