3GPP TSG-RAN WG4 Meeting # 104-Bis-e											    R4-2215343

Electronic Meeting, 10-19 October 2022

Source: 	Murata Manufacturing Co Ltd.

Title: 	On Triple Beat Detection Equation for Intra-band Contiguous 2 CCs

Agenda Item:	5.1.1

Document for:	Approval

# Background

In RAN4#98-bis-e, a Triple Beat detection mechanism was proposed by R4-2107345, for detecting 3rd order non-linearity that is caused by α (TX22TX1), the proposal was approved in a WF R4-2105335, and later was formalized in R4-2202279 in RAN4 #101-Bis-e, as copied below:

- The detection mechanism for the 1st order triple beat is shown below from [2]

    - Duplex Offset is the duplex offset of the victim FDD band

    - ULCAMBW is the non-contiguous 2 tone allocation spacing

    - TXMBW and RXBW is the FDD victim band TX transmission measured BW and the RX DL channel BW respectively

Duplex Offset<ULCAMBW+TXMBW2 +RXBW2														(1)

Figure 1  Existing TB Detection Mechanism is only for victim bands that have Positive Duplex Offsets

We found the above equation (1), strictly speaking, to be partially incomplete, because it does not include victim bands with Negative Duplex Offsets.  In other words, the existing TB detection equation (1) only applies to victim bands that have Positive Duplex Offsets, ie. bands whose DL frequencies are higher than their UL frequencies, e.g. Band 2 and Band 3.  There are bands that have Negative Duplex Offsets, ie. those bands whose UL frequencies are higher than its DL frequencies, e.g. Band 71 and Band 13.  Those bands will require the detection equation to be modified slightly.

Therefore, we propose a slight modification for the detection equation, so it will apply to both kinds of victim bands (with either positive or negative duplex offsets).

# Discussions

## The More Complete TB Picture

## When Triple Beats are generated, they appear in pairs, ie. they appear on both sides of the victim band’s UL spectrum, because of the f1 ± (f3 – f2) terms that’s caused by the trigonometrical expansion of the α (TX22TX1) product (f1 is TX1, f3-f2 is generated by TX22).  For victim bands that have positive duplex offsets, the high side triple beat may overlap with its DL band and causes de-sense, as illustrated in Figure 2 below:

Figure 2  TB always Appear in Pairs (shown with Victim Band with Positive Duplex Offsets)

For victim bands that have negative duplex offsets, the low side triple beat may overlap with its DL band and cause de-sense, as illustrated in Figure 3 below:

Figure 3  TB Mechanism for victim bands that have Negative Duplex Offsets

## Detection Equation for Victim Bands with Negative Duplex Offsets

For intra-band contiguous CA ULs, for victim bands that have negative Duplex Offsets, when the victim band is transmitting at its lower freq edge, we redraw the boundary condition (“Hit Condition”) illustration by shifting the “2 tones” to between victim band’s UL and DL for ease of visualizing frequency stacking.  Thru a few steps of arithmetic manipulation (please see details in Appendix 1) we obtain its Triple Beat Detection Equation as in Equation (2).

- Duplex Offset<ULCAMBW+TXMBW2 +RXBW2													(2)

Figure 4  Illustration for Deriving TB Detection Equation for victim bands that have Negative Duplex Offsets (The 2 CCs are shifted in frequency for visualizing frequency stacking)

## Combining Two Detection Equations

We can combine detection equations (1) and (2) into a single universal equation that applies to victim bands that have either positive or negative duplex offsets:

Duplex Offset<ULCAMBW+TXMBW2 +RXBW2								 					(3)

Proposal 1: Modify the existing TB Detection Equation as Equation (3) for Intra-band Contiguous 2 CC case.

# Summary

To cover victim bands that have negative duplex offset, we propose to replace the existing TB detection equation with the equation (3) below:

Proposal 1: Adopt Equation (3) as the Triple Beat Detection Equation for Intra-band Contiguous 2 CC case.

Duplex Offset<ULCAMBW+TXMBW2 +RXBW2								 					(3)

# References

[1] R4-2105335 Way forward on analysis and framework of triple beat issue of 3CC UL with contiguous intra band UL CA, Qualcomm Incorporated, RAN4#98-bis-e

[2] R4-2111016 MSD Due to NR Intra-band ULCA IMD within Inter-band Combinations, Skyworks, R4#99-e

[3] R4-2107345, Inter-band ULCA with more than 2CCs, Qualcomm, RAN4#98bis-e

[4] R4-2202279, WF on triple beat evaluation and specification framework, Qualcomm, Skyworks, RAN4#101-bis-e

[5] R4-2210557 WF on criteria for triple beat MSD analysis, Qualcomm, Skyworks, ZTE, RAN4#103-e

# Appendix

## Derivation of Detection Equation for Victim Bands with Negative Duplex Offsets

Figure 2 TB Illustration for victim bands that have Positive Duplex Offsets, with Victim Band Transmitting at its Higher Band Edge

Let’s name the center freqs of the victim band Tx, CC1 & CC2 as fc_tx, f2c & f3c, respectively, and their allocated BWs as TXallocBW, BW2 & BW3 respectively.  We have the following relationships for 1st order triple beat, ie. the α (TX22TX1) product:

fc_TB = fc_tx – TXMBW /2 + TXallocBW/2 – (ULCAMBW – BW3/2 – BW2/2)

BW_TB = TXallocBW + BW2 + BW3

f_TB_loweredge = fc_TB – BW_TB/2

= fc_tx – TXMBW /2 + TXallocBW/2 – (ULCAMBW – BW3/2 – BW2/2) – (TXallocBW + BW2 + BW3)/2

= fc_tx – TXMBW/2 – ULCAMBW

where TXMBW is TX Maximum transmission bandwidth, and TXallocBW is TX allocated BW of the victim band.

Detection criteria is when TB starts to overlap the victim’s Rx band, ie. TB_loweredge goes below Rx_upperedge:

f_TB_ loweredge < fc_rx + RXBW/2

i.e.		 fc_tx – TXMBW/2 – ULCAMBW) < fc_rx + RXBW/2

Re-arranged as:

– (fc_rx – fc_tx) < ULCAMBW + TXMBW /2 + RXBW/2

Notice   Duplex_Offset = fc_rx – fc_tx

Substituting it in and we obtain a TB Detection Equation for Victim Bands with Negative Duplex Offsets:

– Duplex Offset < ULCAMBW + TXMBW /2 + RXBW/2													(2)