3GPP TSG-RAN4 Meeting #104bis-e	                                                            R4-2215427

Electronic Meeting, October 10 – October 19, 2022

Title:	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR

Source:	CATT

Agenda Item:	6.10.3.1

Document for:	Discussion

1 Introduction

In the previous RAN4 meeting, RRM requirements for UEs reporting NeedForGapsInfoNR IE was discussed and some WF were agreed in [1]. This contribution further discusses the open issues on the topic and presents our views.

2 Discussion

2.1 Whether interruption is expected when UE reports ’no-gap’

This is the issue 3-1 as listed in [1].

The IE NeedForGapsInfoNR introduced in Rel-16, as per the TS 38.331 [2], indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.

As can be seen from the ASN.1 below, within NeedForGapsInfoNR the UE could indicate “gap” or “no-gap” for each entry of NeedForGapsIntraFreqList and NeedForGapsBandListNR.

-- ASN1START -- TAG-NeedForGapsInfoNR-START NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqList-r16, interFreq-needForGap-r16 NeedForGapsBandListNR-r16 } NeedForGapsIntraFreqList-r16 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 NeedForGapsBandListNR-r16 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 NeedForGapsIntraFreq-r16 ::= SEQUENCE { servCellId-r16 ServCellIndex, gapIndicationIntra-r16 ENUMERATED {gap, no-gap} } NeedForGapsNR-r16 ::= SEQUENCE { bandNR-r16 FreqBandIndicatorNR, gapIndication-r16 ENUMERATED {gap, no-gap} } -- TAG-NeedForGapsInfoNR-STOP -- ASN1STOP

And the RRC specification states that for intra-frequency case

gapIndicationIntra

Indicates whether measurement gap is required for the UE to perform intra-frequency SSB based measurements on the concerned serving cell. Value gap indicates that a measurement gap is needed if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP. Value no-gap indicates a measurement gap is not needed to measure the SSB associated to the initial DL BWP for all configured BWPs, no matter the SSB is within the configured BWP or not.

And, for inter-frequency case

gapIndication

Indicates whether measurement gap is required for the UE to perform SSB based measurements on the concerned NR target band while NR-DC or NE-DC is not configured. The UE determines this information based on the resultant configuration of the RRCReconfiguration or RRCResume message that triggers this response. Value gap indicates that a measurement gap is needed, value no-gap indicates a measurement gap is not needed.

Therefore our suggestion is not to allow interruption for the case when UE reports ’no-gap’ for the intra- or inter-frequency measurement cases mentioned above. Our reasoning is as the following.

- From the previous discussions, we observe that there are different views regarding whether UE needs an interruption period to e.g., re-tune its RF BW to measure the SSB, even if it reports ‘no-gap’ for the target. This seems relates to implementation choices. Nonetheless, in Rel-17 NCSG feature, it is allowed that UE reports‘nogap-ncsg’ in NeedForGapNCSG, which already covers the case when UE does not need gap but may need an interruption. We are not sure whether the need is so strong to introduce another mechanism for NeedForGapsInfoNR in Rel-18, given that Rel-17 NCSG feature already handle this potential case, which might be needed for some implementations.

- In the LTE specification TS36.133 section 8.1.2.1,  it is clearly specified as the following: “Inter-frequency and inter-RAT measurement requirements within this clause rely on the UE being configured with one measurement gap pattern unless the UE has signaled that it is capable according to the capability interFreqNeedForGaps or interRATNeedForGaps of conducting such measurements without gaps and without interruption. UEs shall only support those measurement gap patterns listed in Table 8.1.2.1-1 and table 8.1.2.1.-2 that are relevant to its measurement capabilities. UEs supporting network controlled small gap and which have signaled that they are capable of measurements without gap but requiring NCSG, can be configured with a network controlled small gap pattern in table 8.1.2.1.3-1 on all component carrier(s) to perform inter-frequency and inter-RAT measurement.” So it is reasonable to follow the similar mechanism in the NR requirements.

- Since the measurement is a periodic and frequent procedure, if interruption is allowed, considering the RF retuning time and the impact of asynchronization, the interruption may be pretty long and the system throughput will be negatively affected.

- Gap-less measurement and NCSG are both optional features, and as said in a) the existing Rel-17 NCSG mechanism already covers the potential case where UE needs an interruption period. Allowing even more capability variants for gap-less measurement in Rel-18 is rather late and makes the network configuration and scheduling as per UE capabilities even more complicated.

- With the above bullets, it doesn’t seem to be helpful to define additional UE capability to differentiate whether UE needs interruption either (which is the option 3 given by [1] for the issue 3-1).

With the above analysis we come to the following proposal.

Proposal 1: For both intra-frequency and inter-frequency measurement, reporting ‘no-gap’ through NeedForGapsInfoNR means measurement without gap and without interruption.

2.2 Requirement for intra-freq measurement without gap

Several options have been listed in [1] on this issue 3-4.

Based on the existing RAN4 specification [3], for intra-frequency measurement, the case when UE report ‘no-gap’ is defined as intra-frequency measurement without gap. And the measurement requirements and scheduling restriction have been already defined in 9.2.5 and no other requirements are needed. This has also been confirmed by the justification part of the WID [4], i.e., the texts with highlight as shown below.

The NeedForGaps feature can bring gains for system throughput and/or mobility performance under certain conditions, where gap-less measurements can be applied. The existing RRM requirements for UEs supporting NeedForGaps feature are specified for intra-frequency SSB based measurements without gaps, while requirements for inter-frequency measurements without gaps are missing. Also, RRM requirements for intra-frequency SSB based measurements without gaps do not assume interruptions, which may be required.

Based on the discussions in section 2.1, we believe no interruption is needed in the requirements, so our suggestions is to follow the option 1 as listed in [1] for the Issue 3-4, i.e.,

Option 1: Same as requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap)

Proposal 2: For intra-frequency measurement, the measurement requirements and scheduling restriction for the case when UE report ‘no-gap’ follows the same requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap), and no other requirements are needed.

2.3 Requirement for inter-freq measurement without gap

For inter-frequency measurement, there is no requirement defined for the case when UE report ‘no-gap’ through NeedForGapsInfoNR. Similar to intra-frequency measurement, it is natural to extend the current definition of ‘inter-frequency SSB based measurements without measurement gaps’ in the TS38.133 section 9.3.1 (i.e., see the following texts), so that it covers the case.

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) for UE capable of interFrequencyMeas-NoGap provided

- -	the UE supports interFrequencyMeas-Nogap-r16 [15], and

- -	the SSB is completely contained in the active BWP of the UE.

Then the next question is how to define the RRM requirements for such inter-frequency measurements, for which several options have been listed for the issue 3-5 in [1]. As we discussed in section 2.1, we think interruption is not allowed for the case when UE report ‘no-gap’ through NeedForGapsInfoNR, so the measurement requirements in section 9.3.9 can be taken as baseline.  For the scheduling restriction, the requirements in section 9.3.9 can also be a starting point but to see if there are more cases to be considered.  Besides, the definition of CSSF in section 9.1.5 may need to be updated to include this case.

Proposal 3: For inter-frequency measurement, the case when UE report ‘no-gap’ through NeedForGapsInfoNR should also be defined as inter-frequency measurement without gap, and requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) is taken as a starting point (at least for cell identification and measurement period, scheduling restriction can FFS) for further discussions.

Proposal 4: CSSF in section 9.1.5 need to be updated to include inter-frequency measurement without gap for the case when UE report ‘no-gap’ through NeedForGapsInfoNR.

3 Summary

This contribution discusses the RRM requirements for UEs reporting the NeedForGapsInfoNR IE. The following proposals are made.

Requirement for intra-freq measurement without gap

Proposal 1: For both intra-frequency and inter-frequency measurement, reporting ‘no-gap’ through NeedForGapsInfoNR means measurement without gap and without interruption.

Proposal 2: For intra-frequency measurement, the measurement requirements and scheduling restriction for the case when UE report ‘no-gap’ follows the same requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap), and no other requirements are needed.

Requirement for inter-freq measurement without gap

Proposal 3: For inter-frequency measurement, the case when UE report ‘no-gap’ through NeedForGapsInfoNR should also be defined as inter-frequency measurement without gap, and requirements in Section 9.3.9 of TS38.133 (inter-freq wo/ gap) is taken as a starting point (at least for cell identification and measurement period, scheduling restriction can FFS) for further discussions.

Proposal 4: CSSF in section 9.1.5 need to be updated to include inter-frequency measurement without gap for the case when UE report ‘no-gap’ through NeedForGapsInfoNR.

4 References

- R4-2214346 WF on further enhancements on measurement gaps and measurements without gaps

- TS 38.331 “NR; Radio Resource Control (RRC) protocol specification”

- TS 38.133 “NR; Requirements for support of radio resource management”

[4]	RP-222337 Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps