3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2215443

Online, 10th – 19th Oct. 2022

Agenda item:	5.11.2

Source:	SoftBank Corp.

Title:	TP for TR 38.718-03-01: CA_n3-n41-n79

Document for:	Approval

1	Introduction

NR-CA of 3 Band DL and 2Band UL, CA_n3-n41-n79, was approved in RAN#97 [1]. This TP is to capture the basic aspects for the NR-CA.

2	Text Proposal

[Unchanged Parts Skipped]

5.x	CA_n3-n41-n79

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n41+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A-n79A | CA_n3A-n41A CA_n3A-n79A CA_n41A-n79A | 3DL/1UL has already been specified. |  |  |

5.1.x.3	∆TIB,c and ∆RIB,c values

Since 3DL/1UL has already been specified, ∆TIB,c and ∆RIB,c values have also been specified.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n3-n41, CA_n3-n79 and n41-n79 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings.

- 2nd and 5th order IMD generated by dual uplink of Band n3 + Band n41 may fall into own Rx of Band n79

- 2nd and 5th order IMD generated by dual uplink of Band n3 + Band n79 may fall into own Rx of Band n41

- 2nd and 5th order IMD generated by dual uplink of Band n41 + Band n79 may fall into own Rx of Band n3

5.x.2.2	REFSENS requirements

Table 5.x.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. For the exception by dual uplink of Band n3 + Band n41, the same exception value in DC_3A_n41A-n79A is used. For the exception by dual uplink of Band n3 + Band n79 and Band n41 + Band n79, the same exception values in DC_3A-41A_n79A are used.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n41-n79 | n3 | 1755 | 5 | 25 | 1850 | 29.4 | FDD | IMD21 |
|  | n41 | 2570 | 10 | 50 | 2570 | N/A | TDD | N/A |
|  | n79 | 4420 | 40 | 216 | 442 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | 30.2 | TDD | IMD21 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 30.8 | TDD | IMD21 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]

3	Reference

[1] RP-222092	Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 3 bands DL with x bands UL (x=1,2), ZTE Corporation