3GPP TSG RAN WG4 Meeting #104-bis-e	R4-2215444

Online, 10th – 19th Oct. 2022

Agenda item:	5.11.2

Source:	SoftBank Corp.

Title:	TP for TR 38.718-03-01: CA_n41-n77-n79

Document for:	Approval

1	Introduction

NR-CA of 3 Band DL and 2Band UL, CA_n41A-n77A-n79A, was approved in RAN4#104-e [1]. This TP is to update CA configuration to support n77(2A).

2	Text Proposal

[Unchanged Parts Skipped]

5.2	CA_n41-n77-n79

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1	Operating bands for CA

Table 5.2.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination of band n41+n77+n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n77A-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| CA_n41A-n77(2A)-n79A | CA_n41A-n77A CA_n41A-n79A CA_n77A-n79A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

5.2.1.3	∆TIB,c and ∆RIB,c values

For CA_n41-n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.2.1.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n41-n77-n79 | 0.3 | 0.8 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.2.1.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n41-n77-n79 | 0.5 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.2.2	Specific for 2 bands UL CA

5.2.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations such as CA n41-n77, CA_n41-n79 and n77-n79 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings. Note that there is no IMD impact on CA_n41-n77 to n79 Rx and that on CA_n41-n79 to n77 Rx since CA_n77-n79 is synchronous operation.

- 3rd, 4th and 5th order IMD generated by dual uplink of Band n77 + Band n79 may fall into own Rx of Band n41.

5.2.2.2	REFSENS requirements

Table 5.2.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.2.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77-n79 | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | 10.7 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

[Unchanged Parts Skipped]

3	Reference

[1] R4-2212456			TP for TR 38.xxx-xx-xx: CA_n41-n77-n79, SoftBank Corp., LG Electronics