3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215516

Electronic Meeting, 10 October – 19 October 2022

Source:	Skyworks Solutions, Inc.

Title:	MSD for CA_n7A-n26A

Agenda item:			5.1.1 UE RF Requirements

Document for:	Approval

# 1. Introduction

This contribution proposes an MSD test point for CA_n7A-n26A as defined in WID [1].

# 2. Discussion

The requested uplink configurations for CA_n7A-n26A can be found in Table 1 below. Downlink (DL) band n26 sensitivity may be degraded due to third order receiver (Rx) harmonic mixing with the ACLR1 or ACLR2 of band n7 or uplink (UL) CA_n7B transmissions. This RX harmonic MSD is a near-miss case of type “UL1/DL3” for which MSD evaluation is needed.

Table 1: Uplink Configurations for CA_n7A-n26A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n26A | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7B-n26A | CA_n7A-n26A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |

DC_26A_n41A is a combination with a similar UL frequency range, at the difference that for DC_26A_n41A the band 26 is affected by a direct-hit UL1/DL3 Rx harmonic mixing interference.

## 2.1	Background: DC_26A_n41A Rx Harmonic MSD Test Point

The agreed B26 MSD due to third order Rx harmonic mixing with band n41 UL carrier is reproduced in Table 2.

Table 2: Band 26 MSD due to UL1/DL3 Rx harmonic mixing with UL band n41

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

For 5MHz channel bandwidth (CBW), the downlink band n26 reference maximum sensitivity degradation (MSD) is 24.3dB. This high MSD is due to direct collision of the band n26 Rx third harmonic with the fundamental of band n41 UL transmitter. For CA_n7A-n26A, the n26 MSD is expected to abate significantly since, depending on the band n7 UL CBW, the 3rd order Rx harmonic may mix either with the n7 UL ACLR1 or ACLR2.

## 2.2	Test Point Selection for CA_n7A-n26A

The guidelines for selecting MSD test points due to harmonic interference [2] are:

- UL band: select the smallest UL CBW, applicable to both Tx harmonic MSD and Rx harmonic mixing MSD;

- DL band: select the smallest DL affected band CBW so that worst case MSD is captured.

For CA_n7A-n26A, worst case n26 MSD may not be met if the band n7 is configured to its minimum CBW of 5 MHz because the Rx harmonic would, at worst, only mix with ACLR2. We therefore propose to adopt the smallest n7 UL CBW that ensures Rx harmonic mixing occurs with the highest interference level, i.e., with ACLR1.

The new MSD template facilitates this task since the UL RB allocation start position is specified. The simplified intermodulation distortion (IMD) sketch of Figure 1 shows that these worst-case conditions may be met for:

- Band n26 5MHz CBW, lowest carrier DL frequency, Fc= 861.5MHz. The third Rx harmonic is 2584.5MHz;

- Band n7 25MHz CBW (SCS15kHz), highest UL carrier frequency, Fc = 2557.5MHz; and

- Band n7 UL RB allocation set to Lcrb=25RB104. The IMD3 center frequency of modulated RB with its image is 2584.5MHz, ie. centered on the third order Rx harmonic.

Figure 1: Simplified IMD landscape for CA_n7A-n26A UL1/DL3 Rx harmonic mixing test point.

The proposed n26 MSD test point is captured in Table 3. A new footnote “Note X” is proposed to explain the applicable UL and DL band carrier frequencies configurations for this near-miss Rx harmonic MSD test point.

Table 3: Proposed n26 MSD test point for CA_n7A-n26A

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n26 | 25 | 15 | 25 (RBstart=104) | 5 | [TBD] | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest EARFCN or NR ARFCN of the affected DL (lower) band, and for the highest EARFCN or NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

## 2.3	MSD evaluation

The level of interference that is down-converted via third Rx harmonic mixing can be estimated using either the 3GPP PA linearity calibration assumptions or using the SEM requirements. We propose to use SEM requirements and we assume that, under worst-case conditions, a UE may pass the -13dBm/MHz SEM limit with 1dB MPR for the specified n7 UL RB allocation. This means that, in the worst-case at 0dB MPR, the interference level may be as high as -10dBm/MHz. This corresponds to a total interference level integrated in the band n26 5MHz DL CBW of approximately -3.4dBm/4.5MHz.

Observation: The IMD3 interference level integrated in the band n26 5MHz DL channel can be as high as -3.4dBm/4.5MHz.

Based on this observation, we propose the following MSD requirements.

Proposal: For CA_n7A-n26A, adopt the n26 UL1/DL3 Rx harmonic mixing MSD test point from Table 4.

Table 4: n26 UL1/DL3 Rx harmonic mixing MSD for CA_n7A-n26A

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n26 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest EARFCN or NR ARFCN of the affected DL (lower) band, and for the highest EARFCN or NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

# 3. Conclusion

This contribution brings an MSD test point for n26 MSD due to near miss third order Rx harmonic mixing in CA_n7A-n26A. Based on the following observation:

Observation: The IMD3 interference level integrated in the band n26 5MHz DL channel can be as high as -3.4dBm/4.5MHz.

We make the following proposal:

Proposal: For CA_n7A-n26A, adopt the n26 UL1/DL3 Rx harmonic mixing MSD test point from Table 4.

Table 4: n26 UL1/DL3 Rx harmonic mixing MSD for CA_n7A-n26A

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n26 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest EARFCN or NR ARFCN of the affected DL (lower) band, and for the highest EARFCN or NR ARFCN of the UL (higher) band. |  |  |  |  |  |  |  |  |

# Reference

[1]	RP-221876, New WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE.

[2]	R4-2210565, WF on criteria on Rel-17 enhanced MSD table format, 3GPP TSG-RAN WG4 Meeting #103-e, Skyworks Solutions, Inc.