3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2215583

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.2.7.3.1

Source:	Apple

Title:	Discussion on PDSCH demod requirements for NTN

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-e UE demodulation requirements for NTN were discussed and way forward [1] was agreed. In this contribution we present our views and simulation results for requirements.

# 2. Discussion

In RAN4#104-e many open issues were resolved, and agreements reached as captured in [1]:

General Aspects Issue 1-1: Channel model for NTN-TDLA (NLOS) - DS =100ns, Doppler = 200Hz Issue 1-2: Channel model for NTN-TDLC (LOS) - Assuming elevation angle: 30 degree to decide K_offset and K-factor - DS = [3.5ns], Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots] Issue 1-3: Channel model parameter combination - Use NTN-TDLX <DS>-<Fd> for channel parameter combination for NTN demodulation Issue 1-4: Doppler shift due to satellite motion for DL in service link - Confirm the agreement that do not verify the UE compensation prior to the baseband processing. Issue 1-5: Timing drift and sampling frequency offset - Confirm the agreement that do not define sampling offset model. PDSCH Demod Issue 2-1: PDSCH requirements for GEO and LEO - Define PDSCH Demod requirements for LEO-600. Do not define separate set of requirements for GEO Issue 2-2: Applicability rules for LEO requirements - From RAN4 UE demodulation requirement perspective, the applicability for GSO only is proposed to be the same as ‘nonTerrestrialNetwork-r17’: UE needs to pass TS38.101-4 requirements only Issue 2-3: K-offset value - K_offfset = [8 slots] applied for all HARQ configurations Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel - Option 2: No, there is no need to define AoA

Based on the agreed channel model parameters, we propose to define the channel models for NLOS and LOS scenario as follows –

NTN-TDLA-100

Table 1: Channel model for NTN-TDLA100

| Tap # | Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 105 | -4.7 | Rayleigh |
| 3 | 280 | -6.5 | Rayleigh |

NTN-TDLC-3d5

Table 2: Channel model for NTN-TDLC3d5

| Tap # | Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.39 | LOS path |
|  | 0 | -8.67 | Rayleigh |
| 2 | 45 | -21.42 | Rayleigh |

For LOS channel, the tentatively agreed DS value is 3.5 ns, we also consider DS=5ns for convenience of defining in specification.

NTN-TDLC-5

Table 3: Channel model for NTN-TDLC5

| Tap # | Delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.39 | LOS path |
|  | 0 | -8.67 | Rayleigh |
| 2 | 60 | -21.42 | Rayleigh |

In addition, the baseline simulation parameters and assumptions were agreed in [1]. Based on the agreed simulation parameters we provide simulation results for PDSCH demod in the table below.

Table 4: Simulation results for PDSCH demod for NTN

| Prop. Channel | MCS | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- |
|  |  | Disabled HARQ | 16 HARQ Proc | 32 HARQ Proc |
| NTN-TDLA100-200 | MCS4 | -0.7 | -1.4 | -1.4 |
|  | MCS13 | 7.6 | 6.8 | 6.8 |
| NTN-TDLC3d5-200 | MCS4 | -2.4 | -3.3 | -3.0 |
|  | MCS13 | 6.2 | 5.4 | 5.5 |
| NTN-TDLC5-200 | MCS4 | -2.4 | -3.3 | -3.0 |
|  | MCS13 | 6.2 | 5.4 | 5.5 |

Based on the results we observe that there is no performance delta between using DS of 5ns and 3.5ns for LOS channel.

Observation #1: There is no performance delta for LOS channel with DS 3.5ns and 5 ns.

We propose to define requirements for DS=5ns for LOS channel, since it is cleaner to capture in specification.

Proposal #1: Define requirements for DS=5ns for LOS channel.

Define channel models as Table 1 and Table 3 above in 38.101-5.

Proposal #2: Define channel models for NTN-TDLA-100 and NTN-TDLC-5 in TS 38.101-4

To have coverage for all HARQ configurations for NTN, we agreed to define requirements with disabled HARQ and 32 HARQ processes. Both are optional features for the UE and the baseline HARQ configuration is with 16HARQ processes. We propose to split the HARQ configurations across different tests as proposed in Table 5.

Table 5: HARQ Config for NTN test cases

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS13 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

Proposal #3: Define the PDSCH test cases with the following HARQ configurations:

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS13 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

A couple of open issues related to PDSCH demod requirements are requirements with 64QAM and 30KHz SCS.

Issue 2-5: Modulation order

- FFS whether specify PDSCH requirements for 64QAM in Rel-17, operators’ feedback on the demand are encouraged.

Issue 2-6: SCS/CBW set

- Option 1: Do not specify requirements with additional 30KHz SCS for NTN.

- Option 2: Specify requirements with additional 30KHz SCS for NTN

The operating SNR for SAN-UE for NTN is expected to be low. Also, 64QAM is optional for NTN. Given the low SNR operation and optional capability for 64QAM, we propose not to define requirements for PDSCH demod with 64QAM for NTN.

Observation #2: The operating SNR for SAN-UE is expected to be low in NTN network.

Observation #3: 64QAM is optional feature for NTN.

Proposal #4: Do not define requirements with 64QAM for NTN.

Only FDD operation is supported for NTN and typically 15KHz SCS is used in FDD. We don’t see the need to define requirements with 30kHz SCS for NTN UE demod unless there is a deployment planned.

Observation #4: Do not see the need to define requirements for NTN with 30Khz SCS for FDD unless such deployment is planned.

Hence, we propose to de-prioritize requirements with 30KHz SCS for NTN UE demod.

Proposal #3: De-prioritize requirements with 30KHz SCS for NTN UE demod.

# 3. Conclusion

In this paper, we provide our views NTN UE demod requirements. Our observations and proposals are captured below:

Observation #1: There is no performance delta for LOS channel with DS 3.5ns and 5 ns.

Proposal #1: Define requirements for DS=5ns for LOS channel.

Proposal #2: Define channel models for NTN-TDLA-100 and NTN-TDLC-5 in TS 38.101-4

Proposal #3: Define the PDSCH test cases with the following HARQ configurations:

| Prop. Channel | MCS | HARQ Config |
| --- | --- | --- |
| NTN-TDLA100-200 | MCS4 | Disabled HARQ |
|  | MCS13 | 16 HARQ Proc |
| NTN-TDLC5-200 | MCS4 | 32 HARQ Proc |
|  | MCS13 | 16 HARQ Proc |

Observation #2: The operating SNR for SAN-UE is expected to be low in NTN network.

Observation #3: 64QAM is optional feature for NTN.

Proposal #4: Do not define requirements with 64QAM for NTN.

Observation #4: Do not see the need to define requirements for NTN with 30Khz SCS for FDD unless such deployment is planned.

Proposal #3: De-prioritize requirements with 30KHz SCS for NTN UE demod.

# Reference

- R4-2214386, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm Incorporated.