3GPP TSG-RAN WG4 Meeting #104-bis-e 	R4-2215792

Electronic Meeting, 15.– 26. August 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Feasibility study on amount of MSD improvement

Agenda item:	6.6.4.1

Document for:	Discussion

Introduction

In this contribution we will present Nokia’s point of view concerning expected amount of MSD improvement as well as relation between the amount of MSD improvement and MSD types. It’s noted that the more detailed analysis on UL harmonics and harmonic mixing can be seen in [1].

2nd UL harmonic for CA_3-n78

MSD improvement by single RF component performance improvement

As discussed in [1], there would be multiple ways to improve MSD due to UL harmonics. In order to show a specific example, we assume a following condition captured in Table 1. The condition follows those shown in [2] except for B3 PA H2 performance.

Table 1: MSD calculation based on [2], where PA H2 is 35 dBc.

The derived MSD is almost the same as that of specified MSD, i.e., 23.9 dB. The discussion paper of [2] further proposed improved MSD value by assuming PA H2 of 48 dBc and the MSD becomes 14.3 dB as shown in Table 2, which means 10 dB improvement is possible.

Table 2: MSD calculation based on [2], where PA H2 is 48 dBc.

Observation 1: Feasibility of MSD improvement by PA H2 performance improvement was already proposed by a UE vendor in [2], where it showed 10 dB improvement is possible.

Now the question is what if PA H2 with even better performance is used, whether the amount of MSD is further improved or not. The amount of the MSD improvement by PA H2 performance improvement alone starts to saturate when PA H2 approaches to 50 dBc and the MSD couldn’t be lower than 11.4 dB even if PA H2 was infinite. This is because as the noise in Path 1 becomes lower thanks to PA H2 improvement, the noise from Path 3 appears as the next gating factor since PA H2 doesn’t reduce noise in Path 3 at all. Hence, in order to improve MSD further, the noise from Path 3 must be also reduced.

Next, we’ll take a look at the amount of MSD improvement by assuming that isolations like PCB in Path 2 and Path 3 alone or PA H2 alone is improved in Figure 1 which shows how much MSD improvement can be obtained by improving PA H2 from 35 dBc to 75 dBc or isolation in Path 2 and Path 3 from 70 to 110 dB. It’s noted that we don’t have an intention to discuss if 75 dBc and 110 dB are feasible or not. They are just used to see the trend of MSD improvement by PA H2 or isolation improvement. Regarding MSD improvement vs PA H2 improvement, as mentioned before, the MSD improvement is saturated at around 50 dBc. With respect to MSD improvement vs isolation improvement, the amount of MSD improvement starts to saturate at around 90 dB and even if isolation was infinity, the MSD couldn’t be lower than 9.4 dB.

Observation 2: Around 10 dB MSD improvement for CA_n3-n78 2nd UL harmonics is feasible at least e.g., by PA H2 suppression of 48 dBc or isolation like PCB in Path 2 and Path 3 of 85 dB. Single RF component performance improvement alone, however, cannot achieve even better MSD like 20 dB since other gating factor(s) appears in one or two of the three Paths once noise in two or one of the three Paths is improved.

Figure 1: MSD improvement by H2 PA suppression or isolation improvement by the same amount in Path 2&3

Possible way to achieve 20 dB MSD improvement

With consideration of Observation 2, we take into account multiple components performance improvement impact on combined MSD. Figure 2 shows relation between the combined MSD improvement and improvements due to combinations of PA H2 suppression in Path 1, antenna isolation in DRX Path 1 and an isolation in Path 2 and Path 3.

Figure 2: Impact of antenna isolation, PA H2 suppression and isolation in Path 2/3 on combined MSD

From Figure 2, in order to achieve around 20 dB MSD improvement, in case PA H2 suppression is 48 dBc, around 20 dB isolation improvement for Path 2 and Path 3 is needed. In case PA H2 suppression is 35 dBc, even if antenna isolation is 20 dB, 20 dB MSD is not possible (around 19 dB seems possible) under the assumption of Table 1.

Observation 3: In case of PA H2 suppression of 35 dB, 20 dB MSD improvement is not possible even if antenna isolation is 20 dB and isolation improvement of Path 2&3 was infinity if the other assumptions are the same as those in Table 1.

Observation 4: In order to achieve 20 dB MSD improvement with PA H2 suppression of 48 dBc, around 25 dB isolation improvement, i.e., 95 dB isolation, for Path 2 &3 is required if the other assumptions are the same as those in Table 1.

Next, we discuss the impact of antenna isolation performance improvement on the combined MSD. One of the important aspects of antenna isolation in terms of MSD improvement is that the contribution of it to MSD improvement applies to only MSD for DRX under the UE architecture assumption in [2], which is achieved by reducing the composite H2 level at LNA (and hence MSD) in Path 1 for “DRX”.  It’s noted that even if a UE architecture to enable a UE to obtain antenna isolation for PRX is assumed, the conclusion does not change that much for at least MSD due to UL harmonics since it only improves H2 levels at LNA for PRX and DRX for Path 1 only. Therefore, antenna isolation does not help improve combined MSD if DRX H2 level at LNA for Path 1 is lower than those for Path 2 and 3 and helps improve combined MSD a little if DRX H2 level at LNA for Path 1 is higher than those for Path 2 and 3, though the help can be around 5 dB at maximum with antenna isolation of 20 dB.

Observation 5: Antenna isolation improvement helps improve combined MSD more when DRX H2 levels at LNA for Paths 2 and 3 are even lower than DRX H2 level at LNA for Path 1 while the amount of maximum improvement by antenna isolation is around 5 dB with antenna isolation of 20 dB.

20 dB MSD improvement with less isolation improvement in Path 2 &3

In this sub-section, we discuss a way to mitigate the required isolation improvement in Path 2 and 3 by reducing respective MSDs (composite H2 levels) for PRX and DRX in a similar level from the conditions in Table 1. It is noted that in order to achieve 20 dB improved combined MSD, i.e., MSD = around 4 dB, H2 levels at LNA from each of the paths for both PRX and DRX must be lower than approximately -103 dBm, i.e., values in (j), (l) and (n) must be lower than or equal to -103 dBm except for DRX H2 level at (j) whose value is already -105.2 dBm. A specific example is summarized in Table 3, where combined MSD is 3.67 dB can be obtained.

Table 3: A way to 20 dB MSD improvement with less isolation improvement of Path 2&3 than Figure 2

Observation 6: If harmonic filter rejection, HB switch H2 and Triplexer Rejection towards H2 are improved by 8 dB, 8 dB and 5 dB, respectively, all the LNA H2 levels at the three Paths are almost equally lower than or equal to -103 dBm, as shown in Table 3, the required isolation of Path 2 & 3 can be reduced to 83 dB from 95 dB.

How to achieve MSD = 0 dB

It is not hard to anticipate that achieving MSD improvement of 24 dB, i.e., MSD = 0 dB, even more challenging than achieving that of 20 dB. When MSD improvement of 20 dB is achieved, H2 levels from all the paths for PRX and DRX must be already similar levels or composite H2 level for DRX is even smaller than that for PRX. The former means that even if a component performance is improved and one or some of H2 levels at LNA at Path 1, 2 and/or 3 for PRX and/or DRX are improved, the other H2 levels stay as a gating factor. The latter means that the composite level for PRX must be improved. In any case, multiple components performance must be improved, but cost for MSD improvement becomes higher. For more specific, Figure 3 shows relation between respective H2 levels improvement and combined MSD assuming that H2 levels from the three Paths are improved in the same amount from the condition shown in Table 4. For example, 1 dB in the horizontal line means -116 (-115 dBm - 1 dB) dBm for Path 1 DRX and -106 (-105 dBm - 1 dB) dBm for Path 1/2/3 PRX and Path 2/3 DRX and so on.

Figure 3: Required LNA PRX or DRX H2 levels for all the three path

Figure 3 says that to improve MSD from 1 dB to almost 0 dB, the respective H2 levels must be improved by at least 10 dB, which means that an additional 10 dB isolation improvement for path 2 and 3 is needed and H2 levels in path 1 somehow need to be improved by 10 dB, but the options are very limited since triplexer would have challenges in having more attention than 20 dB so that RF components performance prior to the triplexer must be improved.

Observation 7: Theoretically it is not impossible to achieve MSD = 0dB for CA_n3-n78 for 2nd UL harmonic. However, as MSD approaches 0 dB, it requires more cost, i.e., components performance improvement compared to the cost to improve MSD by 20 dB.

2nd order harmonic mixing for CA_n3-n78

Regarding CA_n3-n77, MSD for this CA band combination has not been specified in 38.101-1 while that for corresponding EN-DC, MSD is defined in 38.101-3. The values for MSD for EN-DC are extracted from Table 7.3B.2.3.2-1 in 38.101-3 as follows.

Table 4: Extract from Table 7.3B.2.3.2-1 in TS38.101-3

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |

A technical analysis to justify MSD = 0 dB for CA_n3-n78 was submitted in [4], where Tx power at LNA input are summarized in Table 5, where Direct Signal Path corresponds to Path 1 in Section 2.1 and PCB Coupling Path corresponds to Path 2 in Section 2.1, respectively. It should be noted that in case of analysis for harmonic mixing, consideration of Path 3 in Section 2.1 is not necessary since the BB output power is always even lower than that PA output power in Path 2 by PA gain as far as the same PCB isolation is assumed. The contribution shared the fact that there is a mixier with the spur rejection of about 66 dBc for mixing with 2LO and it enables UEs to have MSD = 0 dB. Moreover, the contribution of [4] implies that MSD = 0 dB for CA_n3-n78 can be achieved with PCB isolation of 84 (70 dB + 14 dB) dB instead of using the mixer with 66 dBc rejection.

Table 5: Extract of Table 1 from [4]

As can be seen in Table 5, RF components like antenna isolation, filter isolations in Path 1 cannot contribute to improving combined MSD since noise level from Path 2 is higher by 14 dB for PRX and 24 dB for DRX until the noise level in Path 2 approaches close to that in Path 1. 14 dB noise reduction for Path 2 means that PCB isolation must be improved from 70 dB to 84 dB. However, as can be seen in Figure 4, when PCB isolation is 85 dB, combined MSD itself if almost zero dB (Combined MSD is below 1 dB even with mixer spur rejection is 45 dBc) so that there is no room for antenna isolation to contribute to the improvement.

Figure 4: MSD relation b/w PCB isolation and Mixer spur rejection for 2nd harmonic mixing

Figure 4 also shares the information that combined MSD is almost the same if the same amount of PCB isolation improvement or Mixer spur rejection improvement is achieved, though Mixer spur rejection improvement can give a little bit lager MSD improvement by PCB isolation improvement (see e.g., the top red horizontal dashed line.  A point where PCB isolation of 70 dB line has 50 dBc(45+5) mixer spur rejection is lower than a point where PCB isolation of 75 dB(70+5) line has 45 dBc mixer spur rejection). This is theoretically valid since mixer spur rejection applies to the composite noise consisting of Path 1 and Path 2 while PCB isolation only improves noise from Path 2. Finally, although MSD = 0 dB is possible, the cost performance becomes worse as the MSD approaches 0 dB. For instance, in order to achieve 5 dB MSD improvement, i.e., MSD = 2 dB, PCB isolation, mixer spur rejection or the sum of both requires around from 8 dB to 12 dB improvement while to achieve 6 dB MSD improvement, i.e., MSD = 1 dB, PCB isolation, mixer spur rejection or the sum of both requires around 12 to 15 dB improvement.

Observation 8: MSD of 0 dB for CA_n3-n78 harmonic mixing is feasible by PCB isolation improvement, e.g., 84 dB, mixer spur rejection improvement, e.g., 66 dBc or combination of the two RF components performance improvement, e.g., PCB isolation is 75 dB and mixer spur rejection is 60 dBc. Antenna isolation improvement doesn’t help improve MSD for 2nd harmonic mixing for CA_n3-n78.

IMD2 for CA_n3-n78

MSD due to IMD2 for CA_n3-n78 were discussed intensively. One of the contributions as reference could be [5]. Table 6 is a summary of IMD2 analysis from [5], where yellow cells are affected by PCB isolation, aqua cells are affected by antenna isolation, green cells are affected by PCB isolation as well as antenna isolation. Hence, under the conditions in Table 6, it is expected that if only PCB isolation is improved, MSD improvement would start to saturate when the PCB isolation approaches to approximately 80 dB, since IMD2 from antenna switch and diplexer stay at -75 dBm in PRX.  Antenna isolation improvement can contribute to reduce IMD2 in DRX so that the combined MSD can be improved accordingly. Hence, if both PCB isolation and antenna isolation are improved, the MSD can be further improved. However, even if PCB isolation as well as antenna isolation are improved, IMD2 from antenna switch and diplexer stay at -75 dBm in PRX. Hence, in order to reduce the combined MSD significantly, IP2 for these RF components needs to be improved.

Table 6: IMD2 analysis for CA_n3-n78 from [5]

Figure 5: relation b/w IMD, PCB isolation, Ant Iso and IP2 of Antenna switch & Diplexer

From Figure 5, it can be seen that the around 20 dB MSD improvement is possible if IP2 for diplexer and Antenna switch are 125 dBm, PCB isolation is 80 dB and antenna isolation is 20 dB under the conditions of Table 6. Theoretically MSD of 0 dB is possible while the cost is significantly high but the gain = “the amount of MSD improvement”/”the amount of RF component improvement” becomes less and less.

Observation 9: Around 10 dB MSD improvement of IMD2 for CA_n3-n78 is possible if antenna isolation is around 20 dB or PCB isolation is around 80 dB. Around 20 dB improvement is also possible, if IP2 of antenna switch as well as diplexer is around 125 dBm. Further MSD improvement is also possible while the cost and performance = “the amount of MSD improvement”/”the amount of RF component performance improvement” becomes less and less.

MSD improvement and MSD types

The below Figure 6 shows relation between the amount of the MSD improvements and the amount of PCB isolation improvement for different MSD types. It should be noted that assumed UE RF architecture is basically the same across the MSD evaluations for MSD types while some device performance assumptions, e.g., RF front end loss are different across MSD types so that this can be used only as a guide.

Figure 6: The amount of MSD improvement vs PCB isolation improvement

From the Figure 6, all the three MSD types has a similar trend when PCB isolation is roughly up to 75 dB. Then, gradually, 2nd UL harmonic starts to show a different trend than the other 2nd harmonic mixing and IMD2 Moreover, grey line is the case that PCB isolation as well as PA H2 improvement for 2nd UL harmonic are considered. The amount of the MSD improvement is quite different from the other MSD types as well as the same MSD type whose PA H2 performance is 35 dBc. Moreover, given that it is not always the case that UE vendors will always use PCB isolation alone to improve MSD, the amount of MSD improvement can be very different from MSD types to types.

Observation 10: The amount of the MSD improvement is not always the same or similar across MSD types even if one common RF component performance improvement is considered. Provided that it is not always the case that UE vendors will always use one single RF component performance improvement, e.g., only PCB isolation, to improve MSD, the amount of MSD improvement can be very different from MSD types to types.

Conclusion

For 2nd harmonic interference, following observations are obtained.

Observation 1: Feasibility of MSD improvement by PA H2 performance improvement was already proposed by a UE vendor in [2], where it showed 10 dB improvement is possible.

Observation 2: Around 10 dB MSD improvement for CA_n3-n78 2nd UL harmonics is feasible at least e.g., by PA H2 suppression of 48 dBc or isolation like PCB in Path 2 and Path 3 of 85 dB. Single RF component performance improvement alone, however, cannot achieve even better MSD like 20 dB since other gating factor(s) appears in one or two of the three Paths once noise in two or one of the three Paths is improved.

Observation 3: In case of PA H2 suppression of 35 dB, 20 dB MSD improvement is not possible even if antenna isolation is 20 dB and isolation improvement of Path 2&3 was infinity if the other assumptions are the same as those in Table 1.

Observation 4: In order to achieve 20 dB MSD improvement with PA H2 suppression of 48 dBc, around 25 dB isolation improvement, i.e., 95 dB isolation, for Path 2 &3 is required if the other assumptions are the same as those in Table 1.

Observation 5: Antenna isolation improvement helps improve combined MSD more when DRX H2 levels at LNA for Paths 2 and 3 are even lower than DRX H2 level at LNA for Path 1 while the amount of maximum improvement by antenna isolation is around 5 dB with antenna isolation of 20 dB.

Observation 6: If harmonic filter rejection, HB switch H2 and Triplexer Rejection towards H2 are improved by 8 dB, 8 dB and 5 dB, respectively, all the LNA H2 levels at the three Paths are almost equally lower than or equal to -103 dBm, as shown in Table 3, the required isolation of Path 2 & 3 can be reduced to 83 dB from 95 dB.

Observation 7: Theoretically it is not impossible to achieve MSD = 0dB for CA_n3-n78 for 2nd UL harmonic. However, as MSD approaches 0 dB, it requires more cost, i.e., components performance improvement compared to the cost to improve MSD by 20 dB.

For 2nd harmonic mixing, a following observation is obtained.

Observation 8: MSD of 0 dB for CA_n3-n78 harmonic mixing is feasible by PCB isolation improvement, e.g., 84 dB, mixer spur rejection improvement, e.g., 66 dBc or combination of the two RF components performance improvement, e.g., PCB isolation is 75 dB and mixer spur rejection is 60 dBc. Antenna isolation improvement doesn’t help improve MSD for 2nd harmonic mixing for CA_n3-n78.

For IMD2, a following observation is obtained.

Observation 9: Around 10 dB MSD improvement of IMD2 for CA_n3-n78 is possible if antenna isolation is around 20 dB or PCB isolation is around 80 dB. Around 20 dB improvement is also possible, if IP2 of antenna switch as well as diplexer is around 125 dBm. Further MSD improvement is also possible while the cost and performance = “the amount of MSD improvement”/”the amount of RF component performance improvement” becomes less and less.

For relation between the amount of MSD improvement and MSD types, a following observation is obtained.

Observation 10: The amount of the MSD improvement is not always the same or similar across MSD types even if one common RF component performance improvement is considered. Provided that it is not always the case that UE vendors will always use one single RF component performance improvement, e.g., only PCB isolation, to improve MSD, the amount of MSD improvement can be very different from MSD types to types.

From the all the above observations and in order to move forward, we propose a following.

Proposal: RAN4 should confirm that lower MSD(s) than minimum requirements is possible under the condition that the extent of the amount of MSD improvement and measures are different from MSD types to types as well as UE to UE.

- References

- R4-2212093, Possible approaches to improve MSD, Nokia, Nokia Shanghai Bell, TSG-RAN WG4#104-e

- R4-1709559, Discussion on harmonic issue for LTE B3+NR 3.5G, Huawei, HiSilicon, TSG-RAN WG4 NR ad-hoc #3

- R4-2016440, Improving PC2 MSD for EN-DC and UL CA, Qualcomm Incorporated, TSG-RAN WG4#97e

- R4-1707995, On harmonic mixing for LTE NR DC band combinations, Huawei, HiSilicon, TSG-RAN WG4#84

- R4-2212382, Analyses and views on MSD improvement for inter-band CA/DC, Apple, TSG-RAN WG4#104-e