3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2215913

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.3.7.2.2

Source:	Ericsson

Title:	Simulation results for PDCCH and PBCH demodulation in FR2-2

Document for:	Information

# 1	Introduction

In this paper, we present the simulation results on the PDCCH and PBCH demodulation performance in the frequency range 52.6 GHz to 71 GHz. Following the WF [1], we carried out our simulations using the agreed propagation environments during RAN4 meeting #104-e (see Appendix A.1).

For PDCCH Issue 2-1-1: Whether to define requirements for multi-slot PDCCH monitoring - Agreement: RAN4 to follow the mandatory behaviour for multi-slot PDCCH scheduling and schedule PDCCH with (4,1) for 480kHz and (8,1) for 960kHz if agreed Issue 2-1-2: Set of PDCCH Test Cases Agreement: Define PDCCH requirements according to the table in WF [1] Issue 2-1-1: Whether to align propagation conditions with PDSCH - Agreement: Yes For PBCH Issue 2-2-1: SSB index assumption - Agreement: Only with not known SSB index Issue 2-2-2: Test setup for PBCH requirements - Define requirements for the tests below. Propagation condition to be further discussed; Test number; SCS/CBW; Antenna configuration; Propagation condition; PBCH SNR (dB) @ 1% Pm-bch; SS/PBCH block index. 1-1; 120kHz/100MHz; 1x2 Low; [TDLA10-200]; New; unknown. 1-2; 480kHz/400MHz; [TDLA10-200]; New; unknown.

# 2	PDCCH Simulation Results

## 2.1	Simulation setup

As per WF [1], and for the sake of completeness, we carried out simulations using TDLA10/30-200/650 and TDLD10-200 channel models. The simulation parameters are given in the Table below

Test Parameters (reference from TS 38.101-4 Table 7.3.2.2-1)

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  | TBD |  |
| CCE to REG mapping type |  | Interleaved |  |
| REG bundle size |  | 2 for test 1-1 6 for test 1-2 | 2 |
| Interleaver size |  | 3 for test 1-1 2 for test 1-2 | 3 |
| Shift index |  | 0 |  |

Furthermore, we used the following general parameters:

- Numerologies: 120 KHz/100 MHz and 480 KHz/400 MHz.

- Max Doppler frequency/DMRS: f_D = 200 Hz and f_D = 650 Hz, DMRS 1+1

- Pre-loaded Tx EVM: 6%

- Additive Rx EVM: 0%

## 2.2	Simulation results

In this section, we present the PDCCH performance in terms of the probability of miss-detection of the downlink scheduling grant (Pm-dsg) as function of SNR (dB). The PDCCH performance metric is defined as the needed SNR (dB) for a Pm-dsg (BLER) at 1%. In the sequel, we provide the obtained results summarized in the following tables, while we moved the figures in the Appendix A.2.

SCS 120 KHz/CBW 100 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-1 | 3 | 2 | 60 | 1 | 2 | 1x2 ULA Low | 1_0 | 40 | TDLA30-200 | 5.7618 |
|  |  |  |  |  |  |  |  |  | TDLA30-650 | 5.8719 |
|  |  |  |  |  |  |  |  |  | TDLD30-200 | 3.2824 |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA30-200 | 1.9004 |
|  |  |  |  |  |  |  |  |  | TDLA30-650 | 1.842 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -0.4171 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA30-200 | 0.8124 |
|  |  |  |  |  |  |  |  |  | TDLA30-650 | 0.8788 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | 0.4939 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA30-200 | -3.7148 |
|  |  |  |  |  |  |  |  |  | TDLA30-650 | -3.6095 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -5.6132 |

Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [7.7] dB, we can define PDCCH requirements using TDLA30-650.

SCS 480 KHz/CBW 400 MHz

| Test case | Interleaver Size | REG bundle size | CORE SET RB | CORE SET duration | Aggre-gation level | Antenna config | DCI format | Infor Bit | Propagation condition | SNR (dB) @ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1-2 | 2 | 6 | 60 | 1 | 4 | 1x2 ULA Low | 1_1 | 56 | TDLA10-200 | 1.6345 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -0.5096 |
| Test 1-3 | 3 | 2 | 60 | 1 | 8 | 2x2 ULA Low | 1_1 | 56 | TDLA10-200 | 0.7228 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | 0.5403 |
| Test 1-4 | 3 | 2 | 60 | 2 | 16 | 2x2 ULA Low | 1_0 | 40 | TDLA10-200 | -3.9848 |
|  |  |  |  |  |  |  |  |  | TDLD10-200 | -5.5854 |

Observation 2: For SCS 480 KHZ/CBW 400 MHz and referring to the max DL testable SNR at [-0.6] dB, we can define PDCCH requirements with AL16 using TDLD10-200.

# 3	PBCH Simulation Results

## 3.1	Simulation setup

As per WF [1], we carried out simulations under TDLA10/30-200/650 and TDLD10-200 channel models. Furthermore, we considered the following table where it is worth noting that we kept same target coding rate for both numerologies, 120 KHz SCS and 480 KHz SCS.

Table A.3.4.2-1: PBCH Reference Channel

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | R.PBCH.5 | R.PBCH.6 |
| SS/PBCH block subcarrier spacing | KHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

## 3.2    Simulation results

In this section, we present the PBCH performance in terms of the probability of miss-detection of the PBCH (Pm-bch), given by

Pm-bch=1-AB

where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs, as function of SNR (dB). The Pm-bch is derived with the assumption that UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms). Therefore, we have 4 transmissions for each MIB as per the following parameters’ table.

Table (TS 38.101-4 7.4.2.2-1): Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Physical Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | Ms | 20 |
| TDD UL-DL pattern |  | TBD |
| Note 1: as specified in clause 4.1 of TS 38.213 |  |  |

Observation 3: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

The following tables gather the obtained results.

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 120 KHz | 1x2 Low | TDLA30-200 | -6.8866 | unknown |
|  |  |  | TDLA30-650 | -6.9581 |  |

| Test number | SSB SCS | Antenna configuration | Propagation condition | SNR (dB) @ 1% Pm-bch | SS/PBCH block index |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 480 KHz | 1x2 Low | TDLA10-200 | -6.3639 | unknown |
|  |  |  | TDLD10-200 | -10.1525 |  |

Observation 4: The reference SNRs for 120 KHz and 480 KHz SCS, where the SS/PBCH index is unknown, have been provided. These values confirm their conformance with the max DL testable SNR levels.

# 6	Summary

In this paper, we carried out simulations for PDCCH and PBCH in FR2-2. Here we propose some observations that need further discussions:

Observation 1: For SCS 120 KHZ/CBW 100 MHz and referring to the max DL testable SNR at [7.7] dB, we can define PDCCH requirements using TDLA30-650.

Observation 2: For SCS 480 KHZ/CBW 400 MHz and referring to the max DL testable SNR at [-0.6] dB, we can define PDCCH requirements with AL16 using TDLD10-200.

Observation 3: Pm-bch is derived in terms of BLER for every MIB (4 SSB Transmissions).

Observation 4: The reference SNRs for 120 KHz and 480 KHz SCS, where the SS/PBCH index is unknown, have been provided. These values confirm their conformance with the mas DL testable SNR levels.

# References

- R4-2214390, “WF on FR2-2 UE demodulation requirements”, Qualcomm.

# Appendix	Simulation Setup and Results

## A.1	Propagation environments

Channel model tap delay resolution

- Delay resolution: 5ns for below or equal to 200MHz CBW, 2ns for above 200MHz CBW

- [Maximum Number of Taps]: 12 taps for CBW smaller than 200MHz and 16 taps for CBW larger  than 200MHz;

- Adopt channel model according to the channel bandwidth as:

| SCS [kHz] | CBW [MHz] | Applicable channel models | Tap resolution [ns] | [Maximum Number of Taps] |
| --- | --- | --- | --- | --- |
| 120 | 100 | TDLA30-650 TDLD30-200 | 5 | 12 |
| 120 480 [960] | ≥400 | TDLA10-650 TDLD10-200 | 2 | 16 |

Companies can bring simulation results based on following simplified TDL model:

Table 1.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table 1.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table 1.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table 1.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

## A.2	PDCCH Performance Results

(a) AL2/1 OFDM Symbol                      (b) AL4/1 OFDM Symbol

(c) AL8/1 OFDM Symbol                      (d) AL16/2 OFDM Symbols

Fig 1 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (120 KHz, 100 MHz) in {TDLA 30-200, TDLA 30-650, TDLD 30-200} channels, where 60 RBs are allocated for the CORESET.

(a) AL4/1 OFDM Symbol

(b) AL8/1 OFDM Symbol                      (c) AL16/2 OFDM Symbols

Fig 2 – PDCCH performance in terms of BLER as function of SNR using (SCS,CBW) = (480 KHz, 400 MHz) in {TDLA 10-200, TDLD 10-200} channels, where 60 RBs are allocated for the CORESET.

## A.3	PBCH Performance Results

(a) SS/PBCH Block index unknown                  (b) SS/PBCH Block index unKnown

Fig 3 – BLER (4 Transmissions) as function of SNR (dB) for PBCH using 120 KHz SCS (left) and 480 KHz SCS (right) in {TDLA 10-200, TDLD 10-200, TDLA 30-650, TDLA 30-200} channels.