3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215996

# Electronic Meeting, 10th – 19th Oct., 2022

Title: 	Simulation results on BS coverage enhancement demod PUSCH

Source: 	Huawei, HiSilicon

Agenda item:	4.4.2.1

Document for:	Information

Background

During RAN4#104-e meeting, PUSCH demodulation performance of Rel-17 NR coverage enhancement was discussed and the fruitful agreements are noted in [1]. In this contribution, we provide our final updated simulation results about the BS coverage enhancement PUSCH demodulation requirements.

Simulation

PUSCH TB over Multi Slots (TBoMS)

Here we provide the final updated simulation results for PUSCH TBoMS for alignment, as shown in Table 2.1-2, based on simulation assumption as shown in Table 2.1-1.

Table 2.1-1 Simulation assumption for PUSCH TBoMS

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | FDD and TDD |
| SCS |  | 15/30 kHz for FR1 (TDD and FDD) |
|  |  | 60/120 kHz for FR2 (TDD) |
| Antenna |  | 1T2R |
| Channel Model |  | TDLB100-400 Low for FR1 |
|  |  | TDLA30-300 Low for FR2 |
| TDD UL-DL pattern |  | 7D1S2U, S=6D:4G:4U for 30kHz SCS |
|  |  | 3D1S1U, S=10D:2G:2U for 15/60/120 kHz SCS |
| PRB number |  | 5 PRB |
| PUSCH mapping type |  | Type A and type B for FR1 |
|  |  | Type B for FR2 |
| MCS |  | MCS 2 |
| Inter-slot frequency hopping |  | Disabled |
| Physical/available slots for TBoMS |  | 4 for FDD |
|  |  | 2 for TDD |
| PUSCH repetition |  | Disabled |
| Transform precoding |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions | 4 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | Start symbol | 0 |
|  | Allocation length | 14 for FR1 |
|  |  | 10 for FR2 |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| RV sequence |  | [0 2 3 1] |
| Test metric |  | Test SNR at which the PUSCH achieves 70% of throughput |

Table 2.1-2 Simulation results for PUSCH TBoMS

| Case Number | SCS/kHz | DMRS type | SNR@70% maximum throughput |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD | TDD |
| 1 | 15 | A | -4.53 | -4.46 |
| 2 | 15 | B | -4.50 | -4.41 |
| 3 | 30 | A | -4.52 | -4.27 |
| 4 | 30 | B | -4.48 | -4.24 |
| 5 | 60 | B | / | -4.74 |
| 6 | 120 | B | / | -4.43 |

PUSCH demodulation with Joint Channel Estimation (JCE)

Here we provide the final updated simulation results for PUSCH JCE for alignment, as shown in Table 2.2-2, based on simulation assumption as shown in Table 2.2-1.

Table 2.2-1 Simulation assumption for PUSCH JCE

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | FDD and TDD |
| SCS |  | 15/30 kHz for FR1 (TDD and FDD) |
|  |  | 60/120 kHz for FR2 (TDD) |
| Antenna |  | For FR1: 1T2R 1T4R 1T8R |
|  |  | For FR2: 1T2R |
| Channel Model |  | TDLA30-10 for FR1 |
|  |  | TDLA30-75 for FR2 |
| TDD UL-DL pattern |  | 7D1S2U, S=6D:4G:4U for 15/30kHz SCS |
|  |  | DDSUU, S=10D:2G:2U for 60/120kHz SCS |
| PRB number |  | Full applicable test bandwidth for |
|  |  | 5MHz for 15kHz SCS |
|  |  | 10MHz for 30kHz SCS |
|  |  | 50MHz for 60/120kHz SCS |
|  |  | Interested companies can bring simulation results for the largest CHBW for each SCS. |
| PUSCH mapping type |  | Type A and type B for FR1 |
|  |  | Type B for FR2 |
| MCS |  | MCS 4 |
| Inter-slot frequency hopping |  | Disabled |
| aTDW length (same with cTDW length) |  | 2 for TDD |
|  |  | 8 for FDD |
| PUSCH repetition number |  | 2 for TDD |
|  |  | 8 for FDD |
| Transform precoding |  | CP-OFDM |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | [0 3 0 3] for TDD (repetition number = 2) |
|  |  | [0 0 0 0] for FDD (repetition number = 8) |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | For FR1: DMRS 1+1 and DMRS 1+0 |
|  |  | For FR2: DMRS 1+1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | Start symbol | 0 |
|  | Allocation length | 14 |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| phase and power offset |  | Ideal |
| CFO modelling |  | N/A |
| Test metric |  | Test SNR at which the PUSCH achieves 70% of throughput |

Table 2.2-2 Simulation results for PUSCH JCE

| Case Number | Antenna configuration | SCS/kHz | DMRS type | SNR@70% maximum throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | FDD |  | TDD |  |
|  |  |  |  | DMRS 1+0 | DMRS 1+1 | DMRS 1+0 | DMRS 1+1 |
| 1 | 1x2 | 15 | A | -11.38 | -11.75 | -5.38 | -5.78 |
| 2 | 1x2 | 15 | B | -11.38 | -11.74 | -5.37 | -5.79 |
| 3 | 1x2 | 30 | A | -11.57 | -11.72 | -5.66 | -5.96 |
| 4 | 1x2 | 30 | B | -11.55 | -11.71 | -5.64 | -5.97 |
| 5 | 1x2 | 60 | B | / | / | / | -5.86 |
| 6 | 1x2 | 120 | B | / | / | / | -5.95 |
| 7 | 1x4 | 15 | A | -14.8 | -15.29 | -8.89 | -9.43 |
| 8 | 1x4 | 15 | B | -14.82 | -15.29 | -8.89 | -9.43 |
| 9 | 1x4 | 30 | A | -15.42 | -15.78 | -9.02 | -9.44 |
| 10 | 1x4 | 30 | B | -15.41 | -15.75 | -9.01 | -9.44 |
| 11 | 1x8 | 15 | A | -17.78 | -18.43 | -11.86 | -12.57 |
| 12 | 1x8 | 15 | B | -17.76 | -18.43 | -11.88 | -12.56 |
| 13 | 1x8 | 30 | A | -18.12 | -18.86 | -11.94 | -12.54 |
| 14 | 1x8 | 30 | B | -18.16 | -18.83 | -11.93 | -12.55 |

Conclusion

In this contribution, we provide our final updated simulation results on BS PUSCH demodulation requirements for coverage enhancement for alignment.

Reference

R4-2214315, Email discussion summary for [104-e][326] NR_cov_enh_Demod, RAN4#104-e, Moderator (China Telecom)