3GPP TSG-RAN WG4 Meeting #104-bis-e		R4-2216409

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	6.17.2.2.1

Source:	Intel Corporation

Title:	Discussion on Total Achievable Self-Interference Cancellation

Document for:	Discussion

# 1	Introduction

The previous RAN4 #104 meeting had initial discussion on the feasibility of SBFD, including the topic of self-interference cancellation (SIC).  Agreement was found that SBFD capable gNB will require new/enhanced implementations, and that gNB UL Rx sensitivity degradation should be at least less than 1dB. Company inputs for several parameters to be included in self-interference modelling were presented and discussed.

In this paper we review several self-interference cancellation systems found in the academic literature and discuss with the aim of finding approximate bounds to our assumptions.  This is necessary so that system simulations can be based on a common understanding of parameters.

# Discussion

Self-interference modelling is key to achieving realistic system level simulations of SBFD operation.  With gNB Tx output powers beyond 46dBm for FR1 and Rx sensitives below -95dBm for NR Wide Area BS, there is a significant amount of power that must be isolated between semi co-located Tx and Rx.  This large gap can be covered by three factors: 1) propagation domain isolation such as physical antenna separation, spatial / beam separation, antenna phasing, antenna polarization, absorber, and reflector elements, etc.  2) RF/Analog domain SIC, which can sense or couple the Tx signals to be cancelled before the LNA or in the analog base-band before the ADC, and 3) Digital domain SIC, which typically uses linear, widely linear, or non-linear digital cancellation models to reduce any remaining interference to below the Rx sensitivity level.

The SIC from the first two factors, propagation, and RF/analog SIC, must reduce the interference to a level sufficiently small so as to avoid intermodulation issues at the LNA input from non-linearity in the Rx chain.  Current FR-1 specifications for Wide Area BS limit the input signals to below -52dBm.  Additionally, the input signal must also allow the Rx chain to operate in a high gain mode to meet refsense requirements as well as to avoid saturation of the ADC.

Typical sources of Tx and Rx non-idealities that must be modelled for self-interference cancellation include: PA non-linearity, LNA non-linearity, IQ-mismatch in the cancellation, and phase noise differences between LOs.

### 2.1 Self-Interference modeling

The following table was included but not agreed in the WF from RAN4 #104 summary of discussion:

Table 1: Summary table for FR1 from RAN4 #104

| Factors | R4-2211562 | R4-2212493 | R4-2212486 | R4-2212620 | R4-2212848 | R4-2213690 For Medium range BS | R4-2212117 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spatial isolation | 80 dB Separated panel | 80dB | 70 -80 dB Separated panel | 65dB in large enough BW | 100+ dB needed for Macro, feasibility FFS , | 50dB | >70dB |
| Frequency isolation | 45 dB From SI RP-193190 | 45 | 45 dB | - | 45dB With guard band | 45dB | 45dB |
| Beam nulling /isolation | 5~10 dB | TX beam:10dB RX beam:10dB | ~10 dB | - | FFS | - | +3dB on top of beamforming isolation |
| Digital IC | 10~15 dB From SI RP-193190 | 10db | 30-50 dB | - | FFS | [30] for digital IC or transmitter sub-band filtering |  |
| Overall RSIC | 140~150 dB as capable | 145dB | 140 – 185 dB as capable | - | 100+ dB needed for Macro | 125dB | >100dB |

The table includes several rows that contribute to the overall SIC budget.  Frequency isolation was agreeable from all of the input papers.  Yet other parameters have some wide variation amongst companies.  This is somewhat to be expected, as there are numerous ways to implement a full-duplex system and there are inevitably trade-offs between the categories.  However, in order to have meaningful and agreeable simulations we need to find a common range that is acceptable.  To that end, in the next section, we review the academic literature to find some trends and bounds to help narrow the range of values found in this table.

### 2.2 Literature Review of Full Duplex cancellation systems

In [1] a nonlinear MIMO full-duplex transceiver model is presented.  In addition to PA non-linearity, IQ mis-match and phase noise, their model includes -10dB cross-talk between the Tx MIMO paths before and after the PAs which they show to be another considerable contributor.  Their results are for 20MHz LTE waveforms with 25dBm output at 2.4GHz.  While performing a SIC model for MIMO can result in very large number of parameters to estimate, they show that some reduction is feasible while still achieving good SIC.

In [2], a full-duplex system is presented using passive antenna isolation for 75dB of SIC, followed by 7dB of RF cancellation and 25dB of non-linear digital cancellation.  The system operates up to 80MHz BW at 2.56GHz with a 29dBm PA.  In [2], the authors also present measurement results of Tx-Rx isolation of their antenna, comparing anechoic chamber results to outdoor, and two different indoor scenarios.  The outdoor isolation is within 1dB of the anechoic chamber results, but the indoor results reduce the Tx-Rx isolation by 5-7dB.

In [3], an electrically tuned RF duplexer is presented with measurement results in a full-duplex system.  Passive isolation of ~60dB is achieved for a 100MHz BW.  The Tx to Rx isolation provided by the duplexer is shown to be inversely proportional to BW.

In [4], a full duplex transceiver is presented operating on a 20MHz BW signal at 920MHz with a PA output power of 20dBm.  A circulator is followed by RFSIC.  The circulator gives 20dB of SIC, the RFSIC achieves 36dB of cancellation, and digital SIC achieves 34dB for a total cancellation of 91dB.

In [5], a full duplex cancellation system is presented, based on modelling the digital cancellation with Volterra-series.  The BW is 20MHz with a 2.4GHz signal and 30dBm PA output. The system uses a circulator followed by an RF canceller which achieve 32dB and 49dB of SIC respectively.  The Volterra based non-linear digital canceller achieves 34dB of cancellation, which is shown to be a 5dB improvement over MP based non-linear digital cancellation.  In [5], they also show results for a power sweep of the PA output from 20dBm to 33dBm.  For this case, linear digital cancellation degrades at higher powers, but the non-linear digital cancellation performs similarly well for all power levels.

In [6], a study of high isolation between Tx and Rx is performed to evaluate antenna isolation degradation.  Construction non-idealities, as well as reflections from scattering objects in the environment are shown to limit the antenna isolation to ~60dB.  An RF canceller subsystem is added in the presence of reflection objects and shown to achieve an additional 32dB of SIC.

In [7], a full duplex cancellation radio is studied with cancelling a 100MHz BW at 2.4GHz.  The passive antenna isolation achieved is 27dB, with 15dB of further RF cancellation, for 42dB total cancellation with passive + RF.

In [8], a full duplex antenna system is presented using end-fire arranged antenna arrays.  -66dB to-76dB of isolation is achieved for separation greater than 8 wavelengths operating in the 2.4GHz band.

In [9], a self-interference cancellation system operating at 3.5GHz, over a 120MHz BW with 0dBm Tx output power is presented.  The system used orthogonally polarized antennas to achieve 42dB of passive isolation.  30.6dB of digital cancellation is achieved.

The following Table summarizes the results from papers reviewed:

Table 2 - Summary of SIC from papers

| Ref | Propagation Domain Isolation | Center Freq | Bandwidth | Passive Isolation | RF SIC | Dig SIC | Passive+RF SIC | Total SIC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [1] | Ant isolation | 2.4 GHz | 20MHz | 40 | 23.3 | 45.8 | 63.3 | 109.1 |
| [2] | Ant isolation | 2.56 GHz | 80MHz | 75 | 7 | 25 | 82 | 107 |
| [4] | Circulator | 920 MHz | 20MHz | 20 | 36 | 34 | 56 | 90 |
| [5] | Circulator | 2.4 GHz | 20MHz | 32 | 42.3 | 34.3 | 74.3 | 108.6 |
| [6] | Ant isolation | 2.45 GHz | 30MHz | 60 | 30 | NA | 90 | - |
| [7] | Ant isolation | 2.4 GHz | 100MHz | 27 | 15 | NA | 42 | 42 |
| [9] | Ant polarization | 3.5 GHz | 120MHz | 42 | NA | 30.6 | - | 72.6 |

In [10], a wide survey of papers with 52 different systems of self-interference cancellation (SIC) were reviewed.  All the papers, operate below 6GHz, with instantaneous bandwidths up to 120MHz.  All papers employed more than one type of SIC.  Total SIC included propagation based, analog/RF cancellation and digital cancellation.  Total isolation from all three categories did not exceed 115dB in any paper.  Many similar conclusions exist to the papers reviewed in this work and summarized in Table 2.

### 2.3 Analysis

The papers reviewed in the previous section and summarized in Table 1.  The papers use center frequencies ranging from 900MHz to 3.5GHz, and Bandwidths from 20MHz to 120MHz.  The works demonstrate certain trends on the potential for SIC in a full-duplex system.

A first observation is that the Passive Isolation and the RF SIC have a wide variation in range across the papers.  There seem to be certain design trade-offs that improve one while degrading the other.  However, when combining the passive isolation in addition to the RF SIC, the range is more predictable.  The combined metric is useful, since it represents the total isolation between the Tx PA output and the Rx LNA input, which is a critical concern to avoid overdriving the Rx LNA.  Of the papers reviewed, none exceeded 90dB for this combination.

Observation 1: Combining the Passive Isolation and RF Self Interference metrics leads to a more predictable metric and is useful for identifying the potential isolation between the PA output and LNA input.  Of the papers reviewed, none exceeded 90dB for this combination.

Comparing center frequency versus total SIC, shows there is no noticeable dependency.  Several papers in the review did mention that achieving passive and RF isolation is more difficult at wider bandwidths.

When looking at the digital SIC, the range is mostly in between 30 and 35dB.  All of the papers with this range of digital SIC used a non-linear digital cancellation approach, although the methods varied, and included Volterra series, neural network techniques, and others.  The one case with 45dB, was for a MIMO system with a linear model before and after the non-linear model.

Comparing total SIC in Table 2 it appears that 110dB of total SIC is an upper limit.  When adding the 45dB of frequency isolation from the RAN4 discussion in Table 1, this results in an upper bound of 155 of Overall RSIC.

Observation 2: Comparing total SIC in Table 2 it appears that 110dB of total SIC is an upper limit.  When adding the 45dB of frequency isolation from the RAN4 discussion in Table 1, this results in an upper bound of 155dB of Overall RSIC.

Typical power levels in the literature review are between 15dBm and 33dBm, whereas our RAN4 simulation scenario has agreed to power of 46 to 49dBm for FR1.  At higher Tx power levels, SIC is even more critical.  Further additional SIC methods may be needed to develop for this higher power levels.

Proposal 1: We should add a limitation of Overall RSIC to 155dB for simulations.

Proposal 2: Based on the survey, typical values for simulation should be 80dB for Passive + RF SIC, 30dB for digital SIC, and 45dB for frequency isolation.

# 3	Conclusions

In summary, we have reviewed the academic literature for recent publications of full-duplex cancellation systems.

Observation 1: Combining the Passive Isolation and RF Self Interference metrics leads to a more predictable metric and is useful for identifying the potential isolation between the PA output and LNA input.  Of the papers reviewed, none exceeded 90dB for this combination.

Observation 2: Comparing total SIC in Table 2 it appears that 110dB of total SIC is an upper limit.  When adding the 45dB of frequency isolation from the RAN4 discussion in Table 1, this results in an upper bound of 155dB of Overall RSIC.

Proposal 1: We should use an upper bound of 155dB for Overall RSIC for simulations.

Proposal 2: Based on the survey, typical values for simulation should be 80dB for Passive + RF SIC, 30dB for digital SIC, and 45dB for frequency isolation.

# 4	References

- [1] D. Korpi, L. Anttila, and M. Valkama, “Nonlinear self-interference cancellation in MIMO full-duplex transceivers under crosstalk,” EURASIP J. Wireless Commun. Netw., Feb. 2017, Art. no. 24.

- [2] D. Korpi, M. Heino, C. Icheln, K. Haneda, and M. Valkama, “Compact inband full-duplex relays with beyond 100 dB selfinterference suppression: Enabling techniques and field measurements,” IEEE Trans. Antennas Propag., vol. 65, no. 2, pp. 960–965, Feb. 2017

- [3] B. Debaillie et al., “In-band full-duplex transceiver technology for 5G mobile networks,” in Proc. Eur. Solid-State Circuits Conf. (ESSCIRC), Sep. 2015, pp. 84–87.

- [4] M. Emara, P. Rosson, K. Roth, and D. Dassonville, “A full duplex transceiver with reduced hardware complexity,” in Proc. IEEE Global Commun. Conf. (GLOBECOM), Dec. 2017, pp. 1–6.

- [5] D. Korpi, M. Turunen, L. Anttila, and M. Valkama, “Modeling and cancellation of self-interference in full-duplex radio transceivers: Volterra series-based approach,” in Proc. IEEE Int. Conf. Commun. (ICC), May 2018, pp. 1–6

- [6] K. E. Kolodziej, J. G. McMichael, and B. T. Perry, “Simultaneous transmit and receive antenna isolation improvement in scattering environments,” in Proc. IEEE Int. Symp. Antennas Propag. (APSURSI), Jul. 2014, pp. 2228–2229.

- [7] T. Huusari, Y.-S. Choi, P. Liikkanen, D. Korpi, S. Talwar, and M. Valkama, “Wideband self-adaptive RF cancellation circuit for fullduplex radio: Operating principle and measurements,” in Proc. IEEE Veh. Technol. Conf. (VTC Spring), May 2015, pp. 1–7.

- [8] N. Honma, M. Tsunezawa, Y. Yamamoto, and Y. Kashino, “Realizing in-band full-duplex MIMO with feasible RF-chain: Key challenges,” in Proc. IEEE Conf. Antenna Meas. Appl. (CAMA), Dec. 2017, pp. 284–287.

- [9] H. Li et al., “Self-interference cancellation enabling high-throughput short-reach wireless full-duplex communication,” IEEE Trans. Wireless Commun., vol. 17, no. 10, pp. 6475–6486, Oct. 2018.

- [10] K. E. Kolodziej, B. T. Perry, and J. S. Herd, “In-band full-duplex technology: Techniques and systems survey,” IEEE Trans. Microw. Theory Techn., vol. 67, no. 7, pp. 3025–3041, Jul. 2019.