3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216436

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R18 Discussion on 4Tx FWA

Source:	OPPO

Agenda item:	6.6.2

Document for:	Approval

- Introduction

In Rel-18, 4Tx for FWA/CPE/vehicle/industrial devices was one of the objectives for FR1 enhancement as below [1]. This paper will discuss these aspects.

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4] - Investigate framework and architecture Example bands: - TDD bands: n41, n77/n78 - FDD bands: n1 - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 4Tx - First priority: 4x4 UL MIMO - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port - PA configuration assumption: - First priority: 4x23dBm - Second priority: 2x23dBm + 2x26dBm, 4x26dBm - UE power class - First priority: PC 1.5 - Second priority: PC2/PC3, and/or new power class if needed - Note 1: PC1.5 is only applicable for TDD bands - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97 - NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first. - NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition.

- Discussion

  - UE assumptions

In last RAN4 meeting, the WF on UE assumptions [2] was approved in which several open issues need to be clarified. Rel-18 FR1 enhancement WID [1] captures several UE types like CPE/FWA/vehicle/industrial devices, each represent one UE type, but no clear description on how they will be handled in requirement definition. And in last meeting they are discussed but no conclusion.

UE type and requirements

The WF is as below table, and there mainly have two aspects, one is about the RF parameter assumptions comparing to handheld UE, the other is about the requirement definition whether one set or different sets of requirements are defined.

Different views are expressed: reusing handheld parts or considering better parts. Further inputs are expected such as: - Possible convergence or different sets of requirements? - Common to 4 types of UEs or specific to a certain type? In addition, some RF requirements can be revisited because of larger form factor, i.e., better isolation or longer PCB trace/feeder for example. Opinions are also invited on this aspect. RF component assumptions can be further considered during discussion for 4Tx and 8Rx RF requirements.

Generally, CPE/FWA/vehicle/industry devices could have relaxed design constraints comparing to smartphone, therefore it would be not proper to reuse the handheld assumptions. For example, in another WF [3] the MPR requirement definition assumption already have different assumptions comparing to handheld UE. Meanwhile, it can also be seen that the vehicular UE has used different antenna isolation assumption comparing to CPE/FWA devices. This will lead to different requirements for them, but no differentiation of CPE/FWA is needed in FR1.

Agreement (GTW): For 4Tx MPR requirement, - the high antenna isolation compared to handheld UE is assumed for CPE and FWA device. - the same antenna isolation as for handheld UE is assumed for vehicular UE.

Observation 1:    Vehicle UE has used different antenna isolation assumption comparing to CPE/FWA devices.

Proposal 1:         No differentiation of CPE/FWA in FR1 enhancement requirements.

Proposal 2:         Consider separate requirements for vehicle UE in FR1 when necessary.

Currently it is not quite clear about the definition of industrial device since there are many types of them and the shape, size, requirements probably could be different according to the use cases. This may need to be clarified by proponent companies or operators, before it is clarified probably we can reuse the CPE/FWA assumptions for it.

Observation 2:    Many types of industrial devices exist and form factors could be diverse, further clarifications are needed from proponents.

Proposal 3:         Reuse CPE/FWA assumptions for industrial devices before clear definition is made for it.

SAR for CPE

Regarding SAR issues, generally in our view it is more relaxed and can be left to UE implementation. This kind of handling is not new, we have done this for FDD HPUEs.

Proposal 4:         SAR issue can be left to UE implementation for at least CPE/FWA/vehicle devices.

Roaming for CPE

Regarding the international operation captured in the WF [2], it is not quite clear the necessity of this discussion. Is the intention to discuss the country specific requirements to be met by CPE? If it is then maybe NS value related requirements can solve this issue? Some of CPE devices could be roaming to other countries, while some may not. This discussion probably needs more clarification on the purpose itself.

International operation For CPE, possibility of operation in a country other than its home is still diversified. Further inputs are expected, including views on the other type of devices. It is suggested to identify whether there are requirements impact due to international operation/roaming in next meeting.

Observation 3:    Some of CPE devices could be roaming to other countries, while some may not.

Proposal 5:         International roaming can be supported by some of CPEs, and the NS value based requirement definition approach can be applied if there is different requirements among countries.

  - PC1.5 clarification

PC1.5 was agreed to be prioritized with 4x23dBm UE architecture. And there are different views on the definition of UE power class, i.e. per antenna port or per band. This shouldn’t be any confuse in RAN4 as it has been discussed so many times in the TxD and ULFPTx topics in Rel-17. In our view, it is per band defined rather than per port, this is clearly defined in 38.306 UE capability as shown in figure 1.

Figure 1 UE power class definition in 38.306

Proposal 6:         Align the understanding that UE power class is per band defined, rather than per antenna port.

  - 4Tx requirements for different antenna port configurations

The above power class discussion in section 2.2 potentially will have impact on UE requirements to be met when it fallback from 4 antenna ports to 2 antenna ports then to 1 antenna port. The 4 antenna ports and single antenna port are necessary from requirement definition perspective since one for UL MIMO the other for single layer. However, for 2 antenna ports scenario whether it is needed for 4Tx UE is unclear.

Observation 5:    It is not clear whether 4Tx UE has to meet 2Layer UL MIMO requirements.

As shown in figure 2, the 4x23dBm UE can achieve PC1.5 with 4PAs activated in 4Layer UL MIMO and in single antenna port with the help of 4Tx TxD, but it should be noticed that 4Tx TxD is not supported in Rel-17 where TxD is only supported for 2Tx.

For 2antena port scenario (2Layer UL MIMO), to achieve PC1.5 with 4x23dBm, UE has to use TxD in each layer, i.e. 2x23 for Layer1 and 2x23 for Layer2. This is a mix of UL MIMO and TxD which is also new for RAN4 requirements. However, before jumping into the requirement definition, it should first be clear of whether the 4Layer capable UE shall meet the requirements for single antenna port, two antenna port, and one antenna port requirements.

Observation 6:    Rel-17 TxD only defined for 2Tx and not support (4Tx TxD) or (2Tx TxD +UL MIMO).

Figure 2 4Tx UE PA configurations for different scenarios

In last meeting, the 3Layer UL MIMO was discussed, and the conclusion is that in this release 3Layer UL MIMO is not supported. This is not because of this 4Tx UE cannot support the 3Layer transmission as can be seen from the figure below copied from 38.211 where UE with 4Tx the 3Layer UL MIMO can be configured by 4port 3Layer codebooks.

Figure 3 3Layer with 4antenna port codebooks in 38.211

Observation 7:    3Layer UL MIMO is supported in RAN1 for 4port UE by configuring three-layer four port codebooks.

Observation 8:    3Layer UL MIMO is not supported in Rel-18 RAN4 requirements though can be supported by UE in implementation.

Similar logic probably could be applied also to 2Layer UL MIMO for 4Tx UE that this kind of UE only need to meet the requirements of 4Layer UL MIMO and single antenna port.

Proposal 7:         4Tx capable UE only need to meet requirement for 4Layer UL MIMO and single antenna port. The 2Layer UL MIMO can be supported by UE but no need to be tested similar as handling of 3Layer UL MIMO where RAN1 support this feature but no requirement in RAN4.

- Conclusion

In this paper, we discussed the 4Tx enhancements for CPE/FWA devices, and focus on UE assumption handling, UE requirements with different antenna port configurations.

Observation 1:    Vehicle UE has used different antenna isolation assumption comparing to CPE/FWA devices.

Proposal 1:         No differentiation of CPE/FWA in FR1 enhancement requirements.

Proposal 2:         Consider separate requirements for vehicle UE in FR1 when necessary.

Observation 2:    Many types of industrial devices exist and form factors could be diverse, further clarifications are needed from proponents.

Proposal 3:         Reuse CPE/FWA assumptions for industrial devices before clear definition is made for it.

Proposal 4:         SAR issue can be left to UE implementation for at least CPE/FWA/vehicle devices.

Observation 3:    Some of CPE devices could be roaming to other countries, while some may not.

Proposal 5:         International roaming can be supported by some of CPEs, and the NS value based requirement definition approach can be applied if there is different requirements among countries.

Proposal 6:         Align the understanding that UE power class is per band defined, rather than per antenna port.

Observation 5:    It is not clear whether 4Tx UE has to meet 2Layer UL MIMO requirements.

Observation 6:    Rel-17 TxD only defined for 2Tx and not support (4Tx TxD) or (2Tx TxD +UL MIMO).

Observation 7:    3Layer UL MIMO is supported in RAN1 for 4port UE by configuring three-layer four port codebooks.

Observation 8:    3Layer UL MIMO is not supported in Rel-18 RAN4 requirements though can be supported by UE in implementation.

Proposal 7:         4Tx capable UE only need to meet requirement for 4Layer UL MIMO and single antenna port. The 2Layer UL MIMO can be supported by UE but no need to be tested similar as handling of 3Layer UL MIMO where RAN1 support this feature but no requirement in RAN4.

- References

[1] RP-221496, New WID on Further RF requirements enhancement for NR frequency range 1 (FR1)

[2] R4-2214449, WF for assumptions on CPE/FWA/vehicle/industrial devices, Softbank

[3] R4-2214450, WF on FR1 4Tx UE RF requirements, vivo