3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216470

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on open issues for timing advance

Agenda item:	4.2.6.5

Document for:	Discussion

Introduction

The WI description for RAN4 work related to NR over NTN was presented in [1]. Following the progress of RAN4 on the topic, it was approved an extension for the WI activities, in order to address eventual open issues. The WI exception list provided in [2] includes RAN4 RRM requirements, with provision for clarifications and addressing possible inconsistencies.  This contribution proposes further clarification for issues regarding the consistency of GNSS-related operation by UEs in NTN.   In [3] and  [4]several issues are listed, of which we discuss several in the following sections.

Discussion

In this document we provide our opinion regarding items left for further investigation in the WF from previous meetings.  We will concentrate our discussions in two topics, corresponding to Open Issues 5-2 and 5-3 in [4]

Regarding Open Issue 5-2 in how to emulate the GNSS signal for NTN UE timing related test cases, we understand that one important element of the UE conformance, is the ability of the UE to acquire the GNSS signal (a third party signal) and translate it into a position to be used by the UE in the 3GPP modem. Therefore, feeding the UE with AT command “Update UE location information” defined in TS38.509 does not verify the conformance of the UE in one important task related to the timing synchronization.

Proposal 1: Regarding Issue 5-2 we prefer Option 1.

Another discussion left for FFS in Open Issue 5-3 regards the usage of a reference propagator model for comparing UE timing and frequency adjustments.

Agreement from previous meeting:

- The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized.

The best way to ensure that UEs using very accurate propagator models are not penalized, is to select the most accurate reference model compared to the expected orbit of the satellite. The intent of the test case is not evaluate the UE’s propagator model, but to ensure that the UE does not deviate significantly from the UL timing expected by the gNB during the reception of UL signals. It is fair to assume that the best way to measure that is if the “reference measurement” is as close as the gNB timing as possible.

Observation: The reference timing for UL transmission, must be as close as possible from the UL timing expected by the gNB.  The best way to ensure that is selecting a  “reference measurement” is as close as the gNB timing as possible.

Proposal: RAN4 to select the reference propagator model to be as accurate as possible.

Conclusion

Based on our discussions presented in this paper, we therefore have the following observation and conclusion:

Observation: The reference timing for UL transmission, must be as close as possible from the UL timing expected by the gNB.  The best way to ensure that is selecting a  “reference measurement” is as close as the gNB timing as possible.

Proposal: RAN4 to select the reference propagator model to be as accurate as possible.

- References

- RP-213691, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- RP-220209, Rel-17 Work Item Exception for NR-NTN-solutions, Thales

- R4-2214471, WF on NR NTN RRM requirements, Qualcomm Inc.

- R4-2214473, WF on performance part for NR NTN, Xiaomi