3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216495

Electronic Meeting, October 10 - 19, 2022

Title: 	Discussion on relevant test environment for SAN

Agenda Item:	4.2.3.1.3

Source: 	Ericsson

Document for:	Discussion

# Introduction

During last RAN4 meetings there was a debate around considering relevant the environmental test conditions for SAN RF requirements. In this contribution we discuss a possible test setup for normal and extreme test environment, considering vacuum conditions and different equipment classes.

The current tentative agreement in the working group is to consider only the normal test environment, as it is already defined for the terrestrial network BS, e.g. in TS 38.141-1/-2. The extreme test environment is not considered, when the core requirements for output power and receiver sensitivity were defined in TS 38.108.

The current 3GPP conformance testing specifications for BS were developed for the radio equipment operated on ground. Terrestrial cellular networks are characterised by a large number of BSs deployed in different environments, ranging from environmentally controlled enclosures to hanging on a pole under the direct influence of weather conditions. The units exposed to conformance testing are usually not deployed in field. The declaration of conformity to 3GPP is usually a way to assume presumption of conformity to the regulatory limits, hence opening the possibility for the equipment to gain easier market access.

Normal test environment, as defined in the current 3GPP specs, assumes ranges of barometric pressure, temperature and humidity that represent the maximum variation expected in the uncontrolled environment of a test laboratory. Extreme test environment is defined and declared by the manufacturer for each specific equipment. In BS conformance testing specifications testing under extreme environment assumes the combinations for max-min temperature and max-min power supply. It is only considered for test of output power and receiver sensitivity, which is considered sufficient for proving compliance given that most of the other transmitter and receiver requirements are related to these parameters.

Space deployed electronic/radio equipment was historically custom designed for each mission and characterised by usage of high-quality components and backed by extensive testing (e.q. space qualification process) in temperature controlled vacuum chambers, in presence of radiation sources, etc. The units under test were also the units deployed in space and for this reasons clean rooms were necessary when handling the equipment. What is “normal” and “extreme” test environment is usually mission specific, depending on the construction of the whole spacecraft, orbit type, etc. Radio equipment is usually tested integrated in the spacecraft, not as a single unit. To guarantee regulatory limits for RF unwanted emissions all components involved in RF transmissions are considered during the test.

Discussion

“Why do we test?”

In telecom industry, testing is usually employed to prove compliance to technical specifications, regulations and quality targets. Testing ensures the equipment meets the:

- operational targets (e.g. frequency stability, modulation quality, etc.)

- regulations (max power, interference control, environment, EMC, etc)

- quality expectations – works as intended in the assumed use cases.

To a certain extent the 3GPP conformance testing specifications are designed according to these guidelines. The quality of the standards have become the main enabler for using them as reference for market access by the national regulators and telecom operators across the world.

Unlike the terrestrial network equipment (TN), the non-terrestrial equipment (NTN) is facing more harsh operating conditions compared to the ones currently considered in the 3GPP specifications for BS. For the satellite access node (SAN) the particularity of having components deployed in space cannot be ignored. For this reason we believe that testing of a limited set of parameters (output power and rx sensitivity) under vacuum conditions (or close to) need to be considered, together with exposure to the extreme temperatures and power supply values declared by the manufacturer to be supported in the intended use case.

Given the fact that SAN has components that are deployed in space, the compliance to the limits needs to be assessed through measurements performed in a test environment that resembles as much as possible the real-life operation of the equipment and its intended use. For this reason, we cannot neglect the fact that the radio equipment will be operated in vacuum, with no air cooling, no humidity and under the constant stress of ionising radiation.

In figure 1 we show the measurement reference points for output power and reference sensitivity for a SAN type1-H, also indicating the NTN and non-NTN components under test.

Figure 1  Placement of different instrumentation and EUT components in the measurement lab

We observe two types of SAN equipment that are subject to testing:

- SAN NTN equipment – deployed in space

- SAN non-NTN equipment – deployed on the ground

They can be tested at the same time, but placed in different test environments, according to the relevance for the intended use. A thermally controlled vacuum chamber for the space equipment can be considered. A simpler thermally controlled enclosure is sufficient for the non-NTN equipment.

Proposals

Proposal 1: Define two equipment classes to differentiate between NTN and non-NTN components of SAN

Proposal 2: Consider different enclosures for the two equipment classes:

- Thermally controlled vacuum chamber for SAN NTN equipment

- Environmental enclosure for SAN non-NTN equipment

Proposal 3: Define testing under extreme test environment in a similar manner as for the NR BS, at the extreme points for power supply and temperatures supported by the equipment, as declared by manufacturer.