3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216556

E-meeting,10th Oct – 19th Oct,2022

Agenda item:	6.22.1.1

Source: 	ZTE Corporation

Title: 	Discussion on system parameter for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share some initial views from the coexistence study for NTN in Ka-band.

The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022.

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4]

  - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4]

  - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4]

  - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4]

  - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4].

  - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4]

Discussion

## 2.1. Operating bands and band numbering

In the previous RAN-P discussions, there were lots of debates on the definition of Ka band for NTN. From my understanding, note 2 in the approved way forward RP-211596 could be good starting point. If there are any further updates in the coming RAN-P meeting or we reached the consensus in RAN4 if possible, then the frequency range of Ka-band for NTN could be further updated.in this release.

In addition, regarding the band numbering for NTN Ka-band, just to simplify the discussions for Ka-band in Rel-18, we think that the same approach as Rel-17 NTN approach could be adopted which means Ka band are numbered from the highest value of FR2 bands. Based on the current RAN2 NR band signalling design in TS 38.331, the highest value should be 1024 for FR2 bands.

FreqBandIndicatorNR ::= INTEGER (1..1024)

Based on the above considerations, we make the following proposal for NTN Ka-band.

Table 2.1-1. Frequency range for NTN Ka band

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [n1024] | [27500 MHz | – | 30000 MHz] | [17700MHz | – | 20200 MHz] | FDD |

Proposal 1: to define the band in Table 2.1-1 for NTN Ka-band as starting point.

## 2.2. Channel bandwidth

For the supported channel bandwidth for NTN in Ka-band, according to Table 6.1.1.1-5 and Table 6.1.3.2-1 of 38.821, maximum BW per beam is 400MHz for Ka-band. In addition, Ka-band in UL part is also somehow largely overlapping with TN band n257, therefore it should be also reasonable to reuse the same channel bandwidth of n257 for NTN Ka band.

Table 2.2-1. supported chennel bandwidth and SCS for band n1024 in TS 38.101-5

| Operating band | SCS (kHz) | UE channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| [n1024] | 60 | 50 | 100 | 200 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 4001 |  |  |  |
| [NOTE 1: This UE channel bandwidth is optional in this release of the specification.] NOTE 2: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |  |

Table 2.2-2. supported chennel bandwidth and SCS for band n1024 in TS 38.108

| Operating band | SCS (kHz) | UE channel bandwidth (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 800 | 1600 | 2000 |
| [n1024] | 60 | 50 | 100 | 200 |  |  |  |  |
|  | 120 | 50 | 100 | 200 | 4001 |  |  |  |
| [NOTE 1: This UE channel bandwidth is optional in this release of the specification]. NOTE 2: This SCS is optional in this release of the specification. |  |  |  |  |  |  |  |  |

Proposal 2: to define the supported channel bandwidth for NTN Ka-band as in Table 2.2-1 and Table 2.2-2.

In addition, regarding whether 400MHz channel bandwidth should be optional or not for NTN VSAT UE in Rel-18, this might need more discussion in RAN4. More feedback from the both operators and VSAT vendors are necessary.

Proposal 3: to further discuss whether 400MHz channel bandwidth should be optional or not for NTN VSAT UE in Ka-band. More feedback from the both operators and VSAT vendors are encouraged.

## 2.3. Spectral utilization

For the spectral utilization for NTN Ka-band, technically speaking, the spectral utilization should be more or less dependent on the waveform, ACLR, out of band emission, ACS and EVM requirements etc. However based on the experience of Rel-17 NR over NTN discussion, it’s most likely that SAN RF requirement for NTN Ka-band would be relaxed compared with the RF requirement of the terrestrial BS, therefore we don’t see the problem to follow the spectral utilization of terrestrial NR system.

Proposal 4: to reuse the same spectral utilization requirement of FR2 TN system for NTN Ka-band.

## 2.4. Channel raster and sync raster

The channel raster for NTN Ka-band could be be the same as band n257, i.e. 60/120kHz to the simplify the VSAT implementation complexity for initial access. In addition, sync raster could use similar approach as band n257.

Proposal 5: to use the same channel raster for n1024 and the NR-ARFCN in the Table 2.4-1 and GSCN in Table 2.4-2.

Table 2.4.1. NR-ARFCN for NTN Ka-band

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| [n1024] | 60 | TBD – <1> – TBD |
|  | 120 | TBD – <2> – TBD |

For NR-ARFCN design for NTN Ka-band, the existing low boundary in FR2 is assumed as 24.25GHz which is larger than lower boundary of Ka-band, therefore the range of NREF in NTN spec should be lower than 2016667.

RF reference frequency is designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [2016667...3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster [TS 38.101-2]

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

Table 2.4.1. GSCN for NTN Ka-band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n1024] | 120 kHz | Case D | TBD- <1> - TBD |
|  | 240 kHz | Case E | TBD - <2> - TBD |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: SS Block SCS of 960 kHz is not used for initial access. |  |  |  |

The same reason as NR-ARFCN for NTN Ka-band, the range of GSCN for NTN Ka-band should be updated in the NTN spec

Table 5.4.3.1-1: GSCN parameters for the global frequency raster [TS 38.101-2]

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

In addition, it should be noted that NTN Ka-band is not belonging to any FR1 or FR2 definition. From our understanding, if we want to apply SSB block pattern D and SSB pattern E for NTN Ka-band, some updates in TS 38.213 is needed at least.

In addition,

-	Case D - 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 4,8,16,20+28⋅n. For carrier frequencies within FR2, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

-	Case E - 240 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes 8,12,16,20,32,36,40,44+56⋅n. For carrier frequencies within FR2-1, n=0, 1, 2, 3, 5, 6, 7, 8.

Conclusions

In this contribution, we want to share some initial views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Proposal 1: to define the band in Table 2.1-1 for NTN Ka-band as starting point.

Proposal 2: to define the supported channel bandwidth for NTN Ka-band as in Table 2.2-1 and Table 2.2-2.

Proposal 3: to further discuss whether 400MHz channel bandwidth should be optional or not for NTN VSAT UE in Ka-band.

Proposal 4: to reuse the same spectral utilization requirement of FR2 TN system for NTN Ka-band.

Proposal 5: to use the same channel raster for n1024 and the NR-ARFCN in the Table 2.4-1 and GSCN in Table 2.4-2.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828