3GPP TSG-RAN WG4 Meeting # 104b-e	R4-2216871

# Electronic Meeting, October 10 - 19, 2022

Source: 	Skyworks Solutions, Inc.

Title: 	PC5 NRU UE 100MHz A-MPR for a few LPI and Std NS

Agenda Item:	5.28.2 Common requirements (channel raster, A-MPR for 100MHz CBW) [NR_unlic_enh-Core]

Document for:	Approval

# Introduction

In RAN4#104e, 100MHz in-band PSD limited A-MPR was addressed in [2], and several agreements were captured in [1]. In this contribution we further discuss the 100MHz A-MPR for both in-band and OOB limited cases.

# Discussion

In [1] several agreements were captured for 100MHZ A-MPR that are copied here:

- 100MHz A-MPR is studied for OOB emission for NS_28/30/31 channels at 5200, 5300 and 5520MHz

- NS_29: no 100MHz channels

- NS53: A-MPR is studied for 100MHz channels at 5995 and 6055MHz to meet -27dBm/MHz at frequencies < 5925 MHz. Other 100MHz full allocation channels can use MPR while 100MHz partial allocation channels have an A-MPR=Max(4 , MPR)

- NS54: A-MPR is studied for 100MHz channels at 5995 and 6055MHz to meet -27dBm/MHz at frequencies < 5925 MHz

- NS58: A-MPR is studied for the 100MHz channel at 5995MHz to meet -22dBm/MHz at frequencies < 5935 MHz

- NS59: all 100MHz channels can use MPR

- NS60: A-MPR is studied for the 100MHz channel at 5995, 6055MHz to meet -27dBm/MHz at frequencies < 5935 MHz and 7015, 7035, 7055 and 7075MHz to meet -27dBm/MHz at frequencies > 7125MHz. All other channels can use MPR

## In band PSD for different waveforms and bandwidths

Based on the evaluation described in [2], Table 1 provides the in-band PSD for different waveforms and bandwidths at 20dBm max power that can be used to understand if MPR is sufficient to pass a given in-band PSD or if A-MPR is needed. Table 2 provides the MPR for QPSK and Pi/2 BPSK.

Table 1: Approximated in-band PSD at 20dBm vs NRBint0

|  |  | NRBperInt*2^Mu |  |  |  |  | Full |
| --- | --- | --- | --- | --- | --- | --- | --- |
| allocBW | WF | <6 | =6 | =7 | =8 | =9 |  |
| 20 | CP+DFT | 101 | 9.51 | 9 | 8.51 | 8 | 7.5 |
| 40 | CP+DFT | 7 | 6.5 | 6 | 5.5 | 5 | 4.5 |
| 60 | CP+DFT | 5 | 4.5 | na | 3.5 | na | 2.5 |
| 80 | CP+DFT | 4 | 3.5 | na | 2.5 | na | 1.5 |
| 100 | CP+DFT | 3 | 2.5 | na | 1.5 | na | 0.5 |
|  |  | NOTE 1: +0.5 for 100W00100 |  |  |  |  |  |

Table 2: PC5 MPR for lowest order modulations

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 1.5 | ≤ 2.5 |
|  | QPSK | ≤ 1.5 | ≤ 2.5 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 |

When integrating allocation factors shown in the two tables, it is possible to obtain an in-band PSD table at the MPR level rather than 20dBm. This results in Table 3.

Proposal: the following table is used to determine the cases that require A-MPR due to in-band PSD limitation.

Table 3: Approximated in-band PSD at MPR level vs NRBint0

|  |  | NRBperInt*2^Mu |  |  |  |  |  | Full |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| allocBW | WF | <6 | =6 | =7 | =8 | =9 |  |  |
| 20 | DFT | 7.51 | 71 | 6.5 | 61 | 5.5 | 6 |  |
| 40 |  | 4.5 | 4 | 3.5 | 3 | 2.5 | 3 |  |
| 60 |  | 2.5 | 2 | na | 1 | na | 1 |  |
| 80 |  | 1.5 | 1 | na | 0 | na | 0 |  |
| 100 |  | 0.5 | 0 | na | -1 | na | -1 |  |
| 20 | CP | 6.51 | 61 | 5.5 | 51 | 4.5 | 4 |  |
| 40 |  | 3.5 | 3 | 2.5 | 2 | 1.5 | 1 |  |
| 60 |  | 1.5 | 1 | na | 0 | na | -1 |  |
| 80 |  | 0.5 | 0 | na | -1 | na | -2 |  |
| 100 |  | -0.5 | -1 | na | -2 | na | -3 |  |
|  |  | NOTE 1: +0.5 for 100W00100 |  |  |  |  |  |  |

## 100MHz A-MPR for NS that are only in-band PSD limited

For NS_59, there is no OOB requirement and in-band PSD is 5dBm/MHz. Based on Table 3: only 20MHz channel or sub-band BW requires A-MPR for DFT-s-OFDM full and interlace and some CP-OFDM partial. 100MHz CBW can be added without any specific change in the A-MPR table in 38.101-1.

For NS_29 since there are no feasible 100MHZ channels, there is no need to add 100MHz A-MPR

Proposal for NS_59 and NS_29 100MHz A-MPR:

- The clause 6.2F.3.9 A-MPR for NS_59 in 38.101-1 is valid as is and 100MHz channels can be supported and added from Release 17.

- The clause 6.2F.3.3	A-MPR for NS_29 in 38.101-1 is valid as is and 100MHz channels cannot be supported. This can be added as a note in Release 17.

## 100MHz A-MPR for A few OOB limited cases

Herein, we are sharing our preliminary results for two out of band requirements which apply to the lowest 100MHz channel at 5995MHz:

- -27dBm/MHz at 20MHz offset applicable to NS53, 54 and 60 for n96

- -22dBm/MHz at 10MHZ offset applicable to NS58 in n102

In Table 4, we show the raw PA output results that compares the MPR limit to the NS53/54/60 and NS58 OOB emission requirements. The best and worst cases for WB operation are covered.

Table 4: Raw PA output power limit for MPR and a set of A-MPR for corner case 100MHz waveforms

| waveform |  |  |  | MPR gating |  | NS_53/54/60 | NS_58 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| coding | BW | sub-bands | type | SEM | ACL -27dB | -27dBm/MHz @ 20MHz offset | -22dBm/MHz @ 10MHz offset |
| DFT | 100M | 11111 | full | 24.3 | 24.6 | 24.4 | 25.2 |
| DFT | 100M | 11111 | 1RB interlace | 23.7 | 24 | 23.2 | 24.1 |
| DFT | 100M | 10000 | 1RB interlace | 24.6 | 24.2 | 25.5 | 24.3 |
| CP | 100M | 11111 | full | 23.0 | 23.1 | 23.1 | 24 |
| CP | 100M | 11111 | 1RB interlace | 23.3 | 23 | 22.5 | 23.4 |
| CP | 100M | 10000 | 1RB interlace | 23.7 | 23.2 | 25.1 | 23.5 |

Observations:

- For fully allocated RBs and sub-bands, MPR is sufficient with -27dBm/MHz @ 20MHz offset following closely

- For 1RB interlace DFT-s-OFDM and full sub-bands MPR is sufficient for NS_58. However, for NS_53/54/60, A-MPR is needed with ~1dB higher than MPR

- For 1RB interlace CP-OFDM, and full sub-bands, MPR is sufficient with -27dBm/MHz @ 20MHz offset following closely

- For the worst case partial sub-band 1000, for NS53 and NS60, these are anyhow in-band PSD limited, but for NS54 there is no in-band PSD limitation and MPR is sufficient but marginal for NS58

- Based on this result only the first 100MHz channel may require A-MPR for NS53/54/60

Proposal:

- MPR is sufficient for all 100MHz channels waveforms for NS_58

- For NS53/54/60 MPR is sufficient for all complete sub-band cases except for DFT 1RB interlace where A-MPR must be further evaluated

- For NS53 and NS60, 1000 sub-band allocation is in-band PSD limited

- For NS54 partial sub-bands should not need A-MPR based on CP and DFT 1000 cases.

# Conclusions

In this contribution, we address the in-band PSD for different cases and provide the following table as a tool to determine which cases may require A-MPR.

Proposal: the following table is used to determine the cases that require A-MPR due to in-band PSD limitation.

Table 3: Approximated in-band PSD at MPR level vs NRBint0

|  |  | NRBperInt*2^Mu |  |  |  |  |  | Full |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| allocBW | WF | <6 | =6 | =7 | =8 | =9 |  |  |
| 20 | DFT | 7.51 | 71 | 6.5 | 61 | 5.5 | 6 |  |
| 40 |  | 4.5 | 4 | 3.5 | 3 | 2.5 | 3 |  |
| 60 |  | 2.5 | 2 | na | 1 | na | 1 |  |
| 80 |  | 1.5 | 1 | na | 0 | na | 0 |  |
| 100 |  | 0.5 | 0 | na | -1 | na | -1 |  |
| 20 | CP | 6.51 | 61 | 5.5 | 51 | 4.5 | 4 |  |
| 40 |  | 3.5 | 3 | 2.5 | 2 | 1.5 | 1 |  |
| 60 |  | 1.5 | 1 | na | 0 | na | -1 |  |
| 80 |  | 0.5 | 0 | na | -1 | na | -2 |  |
| 100 |  | -0.5 | -1 | na | -2 | na | -3 |  |
|  |  | NOTE 1: +0.5 for 100W00100 |  |  |  |  |  |  |

We further investigated cases that do not need A-MPR for OOB limits and make the following proposal.

Proposal for NS_59 and NS_29 100MHz A-MPR:

- The clause 6.2F.3.9 A-MPR for NS_59 in 38.101-1 is valid as is and 100MHz channels can be supported and added from Release 17.

- The clause 6.2F.3.3	A-MPR for NS_29 in 38.101-1 is valid as is and 100MHz channels cannot be supported. This can be added as a note in Release 17.

Finally, we also examined several OOB emission cases.

Proposal:

- MPR is sufficient for all 100MHz channels waveforms for NS_58

- For NS53/54/60 MPR is sufficient for all complete sub-band cases except for DFT 1RB interlace where A-MPR must be further evaluated

- For NS53 and NS60, 1000 sub-band allocation is in-band PSD limited

- For NS54 partial sub-bands should not need A-MPR based on CP and DFT 1000 cases.

# References

- R4-2214438	WF on NR-U MPR/A-MPR, Skyworks Solutions Inc., R4#104e

- R4-2211606 Single carrier NR-U A-MPR for in-band emission limited and 100MHz channels, Skyworks Solutions Inc., R4#104e