3GPP TSG-RAN WG4 Meeting # 104bis-e                                                       R4-2216961

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.2.5

Source:	Moderator (CATT)

Title:	Email discussion summary for [104-bis-e][125] FS_NR_700800900

Document for:	Information

Introduction

This email discussion is to discuss enhancement for 700800900MHz band combinations. The agreements in last RAN4 meeting was captured in R4-2214445.

The targets of the two rounds in this meeting are as following,

- 1st round:

  - Discuss the issues raised in this meeting.

  - Try to agree more study outputs.

  - For the difficult issues such as MSD, Power, etc, try to agree the parameters assumptions for future meetings.

- 2nd round:

  - Approve the WF.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Huawei | Peng (Henry), Zhang | zhangpeng169@huawei.com |
| Xiaomi | Yuan Gao | Gaoyuan23@xiaomi.com |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| ZTE | Wubin Zhou | Zhou.wubin@zte.com.cn |
| Spark NZ | Mansoor Shafi | Mansoor.shafi@spark.co.nz |
| Spark NZ | Gajan Shivanandan | Gajan.shivansndan@spark.co.nz |
| Skyworks Solutions Inc. | Dominique Brunel | Domnique.brunel@skyworksinc.com |
| Murata | Pushp Trikha | ptrikha@psemi.com |
| vivo | Shuai Zhou | shuai.zhou@vivo.com |
| China Telecom | Bo Liu | liubo1@chinatelecom.cn |
| Qualcomm | Antti Immonen | aimmonen@qti.qualcomm.com |
| Meta | Suhwan Lim | suhlim@meta.com |
| Apple | James Wang | fucheng_wang@apple.com |
| CATT | Huiping Shan | shanhuiping@catt.cn |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: General issues

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215730 | CATT | Skeleton of TR 38.872 |
| R4-2216137 | vivo | Proposal 1: To endorse the following contents for 700/800/900 MHz NR inter-band CA in the TR: - Operating bands for CA (Table 1) - Channel bandwidths per operating band for CA(Table 2) - UE co-existence requirement (Table 3~Table 8) - ∆TIB and ∆RIB values - REFSENS requirements (MSD) - Maximum output power |

Open issues summary and company views collection

Sub-topic 1-1

Issue 1-1: Skeleton of TR 38.872

- Proposals

  - The TR skeleton R4-2215730

- Recommended WF

  - Approve the TR skeleton

| Company | Comments |
| --- | --- |
| Skyworks | It might be work adding the sub-topics per band combinations as suggested in R4-2216137 |
| vivo | Sub-clauses for band combinations can be added in the TR skeleton. |
| CATT | Thanks for the comments. Originally, I didn’t propose the detail structure because I was not sure if it’s good to restrict the contents in the SI TP. It seems there’re many issues need to be studied. The solutions may also depend on different architectures. As there’re TPs propososed in this meeting. We can discuss the detail architecture in the TPs. It will be revised according to the comments from MCC due to the TR upload check. |

Sub-topic 1-2

Issue 1-2: TR UE RF requirements structure and some general proposals

- Proposal in R4-2216137:

To endorse the following contents for 700/800/900 MHz NR inter-band CA in the TR:

- Operating bands for CA (Table 1)

- Channel bandwidths per operating band for CA(Table 2)

- UE co-existence requirement (Table 3~Table 8)

- ∆TIB and ∆RIB values

- REFSENS requirements (MSD)

- Maximum output power

- Recommended WF

Moderator comments: The TR is report for the study item. So from rapporteur’s view, it’s preferred to include more information than the TR for basket CA WI. The detail analysis for the study can be captured in the TR from moderator’s understanding.

| Company | Comments |
| --- | --- |
| XXX |  |
| Huawei | We provided the TP 6070/6071/6072, which contain these contents. |
| Xiaomi | Generally OK with the contents for specifying new band combination but not sure of whether all these are needed for a study item phase. |
| ZTE | For the contents of a new NR CA combination, we think the template of TR38.718-02-01 can be used as baseline. On top of that, some information/content like RF architecture, concrete MSD analysis from each companies(rather than just a value), etc, are proposed to be added, due to it is SID. Therefore, regarding the contents in R4-2216137, we are not OK. For example, Maximum aggregated bandwidth is not needed in BCS information table. In addition, we need to provide full picture of co-existence studies including harmonic/Rx harmonic, and IMD based on FULL range of each band. Frequency restriction is just the solution to avoid the IMD issues. |
| OPPO | For the 1st bullet, frequency restriction in the operating bands, we see there are some discussions about how to use this information like whether to use it as guidance for the multiplexer design, etc. Maybe more discussion is needed on how to handle the situation that once another request on part of the spectrum is raised later, for example to define another specific band combination or reuse this one. |
| Apple | These proposed contents are related to UE RF requirements which can only be derived after the UE architectures feasibility studies. We propose to add clauses on UE architecture considerations. |
| Skyworks | We agree that these are the topics to be studied/added per band combinations but are not ready to capture technical content yet. We do not agree with some of the conclusions that no MSD is needed for DL n5 and DL n28 |
| Murata | Topics ok, but some technical conclusions cannot be endorsed as Skyworks mentioned as CA_n5-n28 is subject to cross band noise MSD , even with the use of a split band filter for n28. |
| vivo | At least we add these topics as subclause for each band combination in the TR skeleton. |
| Meta | We need to add section for the reference or example RF architectures for the specific band combinations. Also we need to add new clause to inform the new RF components (Quadplexer, Triplexer and other specific filters) if the new RF components are applicable to support specific CA band combinations. |
| Qualcomm | These topics need to be studied, but as commented by some other companies not ready to agree on the technical content yet |
| Nokia | At least the listed aspects would be impacted by the introduction of band combinations being studied if it’s introduced. However, as we discussed in CA_n5-n8, resolutions on how to handle n5 DL and n8 UL may impact on some other aspects, e.g., if Apple’s solution is adopted. Therefore, “at least” is needed not to exclude other possibilities at this stage. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: Skeleton of TR 38.872 | There were comments in the 1st round that sub-clauses should be added. The rapporteur clarified that the intention was to keep some flexibility and the sub-clauses can be discussed in TPs in this meeting. Rapporteur also received comments from MCC about the format problem. Tentative agreements: Discuss further on the sub-clauses in the TPs revised by Huawei. Recommendations for 2nd round: Revise the skeleton for the format problem. Discuss the sub-clause structure in the TPs. |
| Issue 1-2: TR UE RF requirements structure and some general proposals | Generally companies are ok with the proposed issues but are not ready to agree all of the contents such as MSD, possible solutions, etc. Companies also showed preference to include more analysis in SI TR than WI TR. Tentative agreements: The structure is generally ok, they can be further discuss and check in the TPs. Other valuable studies not listed are not excluded to be captured in the TR. Recommendations for 2nd round: Further check the structure in the revised TPs by Huawei. |

Discussion on 2nd round (if applicable)

Topic #2: CA_n5-n8

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215445 | Murata Manufacturing Co Ltd. | Observation 1: The Q of the antenna match is too high to accommodate a 2-antenna approach with band n5 and band n8 on the same antenna. The optimum VSWR 2:1 tuning BW required is over 136MHz. TIS (Total Isotropic Sensitivity) and TRP (Total Radiated Power) will be significantly impacted. Radiated performance is a primal factor for link performance. Observation 2: With legacy filtering for band n5 and band n8, there is no relief from any antenna isolation, and due to the shared spectrum of n8 TX and n5 RX, there will be significant TX leakage and TX blocking. Observation 3: A LB-LB n5 RX/n8 RX diplexer is required for 3 antenna approach. Proposal 1: Do not consider 2-antenna architecture for CA_n5-n8 when using legacy filtering with n5/n8 spectrum restriction. Observation 4: No CIM5 is expected due to restricted spectrum Observation 5: TX leakage or n5 RX band noise due to n8 TX gets no filter rejection with legacy n8 TX duplexer filter. The upper edge of n5 restricted RX has no guard band from the low end of n8 TX. Reduced resource allocation along with TX power reduction may be required for n8 TX UL channel bandwidths. Observation 6: Expected n8 TX attenuation of n5 RX legacy filter is not expected to attenuate n8 TX blocker to RX OOB blocker range 1 level of -30dBm, so TX power would have to be reduced to prevent RX large signal compression and saturation. Proposal 2: Agree on the n5 RX filter attenuation level at n8 TX frequency and n8 TX power reduction if using legacy n5 RX filter. Proposal 3: Agree on the restricted resource blocks as well as lower TX power to compensate for no TX leakage attenuation at n5 RX frequency due to legacy n8 TX filter. Proposal 4: Compute MSD requirements after agreed architecture and parameter assumptions for next RAN4#105. |
| R4-2215662 | Apple | Observation 1: One possibility to enable CA_n5-n8 operation with 2-antenna implementation is to allow only non-simultaneous Rx/Tx between n5 DL and n8 UL in conjunction with a triplexer in main path. Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer. Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation. Observation 4: Using two antenna to aggregate n5 and n8 may not be the optimal antenna arrangement to divide the sub-1GHz band frequency range as the lower frequency antenna still needs to cover up to 282 MHz (from APT600 band to n5). Proposal 1: The frequency range restriction of the combination shall not be used as the RF multiplexer implementation guideline as the filter design should accommodate the full-range operation when being operated as a single band for all the constituent bands. Proposal 2: For CA_n5-n8 with 2-antenna implementation, the feasibility of a single triplexer for CA_n5-n8 with acceptable filter isolation and insertion loss needs to be studied. Proposal 3: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor. Proposal 4: Semi-full-duplex operation with non-simultaneous Rx/Tx between n5 DL and n8 UL is required for CA_n5-n8 irrespective of 2- or 3-antenna implementation. |
| R4-2215896 | ZTE Corporation | Observation 1. RF architecture is shared by 1UL single band operation and 2UL CA to support the xUL/2DL CA. Proposal: To discuss whether if it is feasible to restrict the UL configuration is only in n5 for 1UL/2DL NR CA_n5-n8. |
| R4-2215897 | ZTE Corporation | Observation 1. No IMD products falls into n8 DL considering the frequency ranges restriction of n5 and n8. Proposal 1. No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8. Observation 2. No harmonic issues for DL CA_n5-n8 with single band UL configuration no matter with full frequency range and frequency restriction Observation 3. The values of delta Tib,c/Rib,c and cross band MSD relay on the filter performance. Observation 4. If the UL is restricted in band n5 in 1UL/2DL CA_n5-n8, the single band n5 OOB blocking requirements could be applied. |
| R4-2216039 | Xiaomi | Observation 1: With the spectrum restriction, there is no IMD issue for CA_n5-n8 with two uplink transmission. Observation 2: 3 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem based on the commercial smartphone design. Currently 3 antenna architecture is much more preferred in real implementation for CA_n5A-n8A. Proposal 1: It is proposed to use the RF front end architecture in figure 1 (2 antenna) and figure 2 (3 antenna) as baseline for further study. Proposal 2: For CA_n5A-n8A, RAN4 should clarify which set of RF requirement, e.g., worst case, will be used to define minimum requirements by considering all UE RF architecture. |
| R4-2216072 | Huawei, HiSilicon | TP for CA_n5-n8 |
| R4-2216136 | vivo | Proposal 1: It is suggested to clarify the impact on RF architecture for frequency restriction for CA_n5-n8. Observation 1: For CA_n5-n8, with 2UL bands transmission, there is no potential interference for DL n5 and n8. |
| R4-2216340 | Qualcomm Finland RFFE Oy | Observation: Antenna aspects, Conducted RF performance, and RF FE architecture together define the feasibility of an LB NR CA band combination Observation: Discussing and agreeing an antenna bandwidth threshold value for LB-LB CA could help in feasibility discussions Observation: Transmitting UL’s from different antennas would allow better optimization of each TRP MSD analysis using coarse component assumptions was conducted for three different RF FE filter lineups. Observation: CA_n5-n8 is not feasible using full band n5/n8 RF filters and concurrent reception within 869…880 DL and 904…915 UL Observation: RF FE would not be the bottleneck for CA_n5-n8 using full band n5/n8 RF filters and non-concurrent reception within 869...880 DL and 904…915 UL but both the UL and DL throughput would be degraded making the benefit of DL/UL CA very questionable Observation: If CA between the restricted frequency ranges of n5 and n8 using concurrent reception within 869…880 and transmission within 904…915 is supported, UE needs both full n5 and n8 RF filters and dedicated filters for the restricted frequency ranges Observation: When the feasibility analysis for different CA_n5-n8 architectures is done, RAN4 must do a very important decision on how to handle the non-overlapping frequency ranges within overlapping bands. Proponent operator’s input as well as inputs from other companies would be very valuable for further discussions Observation: For NR CA using dedicated RF filters for restricted frequencies with two antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz Observation: For NR CA using dedicated RF filters for restricted frequencies with three antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz |
| R4-2216679 | Skyworks Solutions Inc. | Proposal: - Even if a 3rd antenna is added to accommodate CA_n5-n8, in single band operation or other CA/DC cases, the Architecture should enable the use of only the two best antennas and n5/n8 UL use of the main antenna. - For the 3rd antenna it should be clarified whether or not this is a dedicated low band antenna or a borrowed antenna used for higher frequency bands. Proposal on alternative approaches: - Architectures with a n8 UL filter with restricted bandwidth should be studied to prevent high MSD for band n5 - Alternatively, CA_n5-n8 only uses n5 as UL - Alternatively, CA_n5-n8 only use a restricted set of UL allocations in n8 Proposal on dual UL: With the 3rd antenna allowing to transmit n5 and n8 on separate antennas and using the n5 and n8 restricted range, there is no issue with dual transmission nor 2UL IMDs. No MSD due to IMD is needed but there should still be a dual UL MSD test point with NA for MSD and a note clarifying the applicable frequency ranges. |

Open issues summary and company views collection

Sub-topic 2-1: UE RF architecture for RF requirement analysis

Issue 2-1: Whether 2 antenna architecture is down selected for the requirements study

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

  - Option 1

| Company | Comments |
| --- | --- |
| Huawei | 2 antenna architecture can be captured into the TR as a reference for UE vendor implementation. Based on the contributions, 2 antenna architecture may have different implementation for CA_n5-n8. For deriving the requirements of CA_n5-n8, we can consider 3 antenna architecture as a baseline. |
| Xiaomi | Prefer option 2. 2 antenna architecture can be studied as a candidate RF architecture but we prefer 3 antenna architecture as a baseline for sake of specifying RF requirement as well. |
| ZTE | We would like to know whether ‘down selected’ means the RF requirements will not be studied for 2 antenna architecture in this SID? It could be foreseen that the requirements between 2 antenna architectures and 3 antenna architectures would be different. |
| OPPO | Using one antenna to cover the whole range of n5+n8 is difficult in our view, and performance cannot be guaranteed though simple in implementation. Ok with Option 1 in WI phase for requirement, but now Option 2 probably is ok for study phase. |
| Apple | Option 1 We have concern on whether 3 LB antenna can fit into the form factor for most smartphones with acceptable radiative performance. This is also the leading concern on differentiating the FWA/CPE and smartphone requirements. |
| Skyworks | 2 Antenna architecture should anyhow be the baseline for the single band operation. For CA, in our view since this is a study phase we need to discuss only what is study and goes in the TR. 2 antenna operation should be captured in the TR but only for the case where simultaneous n5Rx and n8Tx is not supported. If simultaneous n8Tx/n5Rx is supported no matter how high the MSD could be, there is already an n5 blocking issue. |
| Murata | 3-antenna is preferred due to optimal radiated performance and UE OEM feedback, so option 2, but we are open to study the conducted requirement of the 2 antenna approach. |
| Meta | We respect the agreements on GTW session. But the required UE RF requirements shall be specified based on the reasonable and feasible RF architecture for smart phone type device. |
| Qualcomm | GTW agreement is a good baseline going forward. For the TR, both 2 and 3 antenna analysis could be included, but the minimum requirements should be specified using worse case. |

GTW discussion on Oct. 11:

Spark: Band n5 and n8 are owned by different operators. The antenna of n5 and n8 are collocated in the same tower and we need some small gap. How will it be implemented in handset.

Vivo: we support option 2. It is hard to down-selected to 2 antenna. 3 antenna is feasible. We should not preclude 3 antenna architecture. The minimum requirement should be defined to allow the feasibility of implementation.

Xiaomi: this is SI. We are OK to provide the full picture for all the UE RF architectures. If down-selected, we prefer to use 3 antenna. We observed using one antenna to cover wide band cannot guarantee the performance. We prefer 3 antenna.

Apple: we can look at the different architecture, 2 and 3 antenna. We have concern on that 3 antenna cannot fit the smart phone. We would like to consider 2 antenna.

Xiaomi: 3 antenna is feasible according to the smart phone design.

Qualcomm: If we down-select, what should we do for 3 antenna. If we agree with 2 antennas, what is the outcome?

Skyworks: our preference is all architectures are valid and for study. But the requirements should be based on the worst case, i.e., 2 antenna, or the requirements should enable two architectures.

ZTE: same question as Qualcomm. What should we do for the other architecture if we down-select one.

Murata: we prefer to use 3 antenna. But we are not against to look at the performance of 2 antenna. UE should be flexible. We should look at the radio performance.

Huawei: we can specify the requirements including all the kinds of implementations. UE vendors want to make implementation flexible.

China telecom: we share the similar view as Murata. We prefer to use 3 antenna which is more feasible. If we downselect to 2, it relies on the dedicated filter.

Moderator: The study can capture all the architectures. The requirement can be based on one and be decided in WI.

Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 2-2: Which architecture is based for the UE RF requirement if two architectures are allowed?

- Proposals

  - Option 1: the architecture with worst requirements

  - Option 2: the architecture with best requirements

- Recommended WF

| Company | Comments |
| --- | --- |
| Huawei | Question for clarification: will one kind of RF architecture lead to be worst (or best) for all the requirements. Not sure whether the requirements are only referred to REFSENS? |
| Xiaomi | Option 1. The worst case can be used to specify RF requirements, e.g. REFSENS. It should be based on further study whether all requirements for one architecture are the worst case. |
| ZTE | Similar question with Huawei. Which architectures lead the worst/best requirements? In addition, if we only study the requirement using 3 antenna architecture at this stage, then what does worst/best requirement means? |
| OPPO | Option 1, if two architectures are considered feasible, then it should be the worst case to derived the minimum requirements. |
| Apple | In our view, it should be based on the feasible architecture with least impact on UE implementation complexity and cost. It also should not cause performance degradation for other frequency bands and combinations. |
| Skyworks | We are in a study phase, in our view it is too early to decide what requirement is finally captured and for which architecture but in general worst case should be baseline. |
| Murata | TBA. The requirements should be satisfied with any feasible implementation. |
| CATT | It can be decided in WI phase as agreed in GTW, There’s no need to discuss this issue in SI phase. |
| Vivo | This issue is kind of related to Issue 2-1. In our understanding, we don’t need to rush to define UE RF requirements in the Study phase. We are open to study the RF requirements impact based on 2 or 3 antenna architecture. In the WI phase, we can define the rf requirement to allow all the feasible architecture. |
| Meta | RAN4 define the minimum RF requirements in 3GPP specification. So option 1 is acceptable to us. |
| Qualcomm | Requirements should be specified using the worst case, however that worst case must still be feasible in order to be specified. Especially for this combination the feasibility has so many aspects, e.g simultaneous n8 UL/n5 DL, restriction to n5 UL only, # of antennas, etc, so the worst case is not only dictated by REFSENS. |

Sub-topic 2-2: Feasibility study issues

Issue 2-3: RF parameters for requirements analysis

- Proposals

  - The following parameters are needed for the feasibility study

    - n5 Tx filter attenuation at n8 Rx frequency range

    - n5 Rx filter attenuation level at n8 Tx frequency range

    - n8 Tx filter attenuation at n5 Rx frequency range

    - n8 Rx filter attenuation level at n5 Tx frequency range

    - antenna ISO

    - RF front end loss

- Recommended WF

  - TBA

Moderator: The above proposals are from the contributions in this meeting and last meeting. Please companies comment if above parameters are sufficient for the future analysis.

| Company | Comments |
| --- | --- |
| Huawei | Companies are encouraged to provide the above parameters. |
| Xiaomi | OK with the proposal. |
| ZTE | Since we are discussing whether to down select the 2 antenna architecture above, so are the listed parameters for a specific architecture? |
| OPPO | Ok with the proposals. The interference between these two bands need to be analyzed. |
| Spark | Bands n 5 and n8 have DL/UL adjacent to each other and the isolation requirements are very large especially for handsets |
| Apple | We propose to consider non-simultaneous Rx/Tx between n5 DL and n8 UL to minimize the cost and complexity in RF front-end filter implementation. |
| Skyworks | In our view it is not important to collect all the front end data since the main issue is related to the n8UL for n5DL where there is little help if the full band duplexers are used. We need to agree how we want to solve this issue first. At least antenna isolation should be 10dB for REFSENS/MSD calculations. Anyhow for these two there is frequency overlap so not sure what attenuation we are talking about.: - n5 Rx filter attenuation level at n8 Tx frequency range - n8 Tx filter attenuation at n5 Rx frequency range |
| Murata | Both TX blocking and TX leakage has a large impact. All the parameters listed are important assuming the legacy filtering approach. The proponent of the band combination needs to also suggest the lower acceptable limit for TX power and allocated resources from a coverage standpoint. |
| vivo | OK with the proposals for feasibility study. |
| Meta | In feasibility study phase, we can discuss the detail RF parameters for those parameters for CA_n5-n8. But we also similar view with SKW if we consider common filter for n5 and n8, then these two parameters are not clear what attenuation levels are expected due to frequency overlapping. |
| Qualcomm | The relevant RF FE parameters are well known by the companies, so in our view companies have the freedom to provide analysis using the parameters up to the level of details they want to. Echoing what Skyworks is saying we should first solve how do we want to solve this issue. To do that we need to agree which of the solutions are we targeting for: -Simultaneous/non-simultaneous TX/RX within the restricted portions of n5/n8 spectrum? -UL CA/only n5 UL? |

Issue 2-4: Whether the filter can be dedicated, based on the restricted frequency range.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

| Company | Comments |
| --- | --- |
| Huawei | For real implementation, the filter can be dedicated for CA_n5-n8 to achieve a good performance. For REFSENS requirements, we can consider a solution to include all kinds of implementation. Common duplexer implementation may lead a worse REFSENS performance. |
| Nokia | If a UE uses “additional” dedicated filters adjusted to spectrum holdings, the UE with such filters shall meet all the requirements for n5 and n8 single band operation without any frequency restriction. |
| ZTE | We see company was already provide some studies based on dedicated filter in this meeting. |
| OPPO | To clarify the meaning of dedicated, to design a dedicated filter for the restricted spectrum is always feasible, but to ask UE implement a dedicated filter only for the restricted spectrum range should be careful because the demands of new frequency range could show and it is not desired for UE to implement multiple dedicated filter for different Operator demands. A global harmonized frequency range should be considered. So if possible, we prefer a harmonized filter if possible rather than a dedicated filter for specific demands. Therefore, more prefer Option 2, if it means mandate UE to implement a dedicated filter for restricted frequency range. |
| Apple | Option 2 Using dedicated filter based on restricted frequency range would not be a common practice in UE implementation which is also not the 3GPP convention in deriving the UE RF requirements. |
| Skyworks | In our view this CA is useless if n8UL simultaneous to n5 DL is allowed and full band n8 UL filter is used. |
| Murata | In our view, dedicated filtering increases the legacy stand-alone insertion loss if both single band and CA band combination is used in a specific region. |
| Vivo | Dedicated filter is only one solution to solve the overlap between n5 DL and n8 UL. We can include it in the feasibility study. |
| Meta | We are fine to consider both common and dedicate filter in study phase. |
| Xiaomi | Share the similar view with Oppo and Apple. We also have concern on the dedicated filter design. It is not preferred to use separate filters for single carrier and CA mode. For the feasibility study we are ok to consider that. |
| China Telecom | We agree with moderator’s suggestion on GTW. Single band filter is the baseline and dedicated filter could be analyzed in the study item. |
| Qualcomm | In order to make progress in this topic, we really need to recognize the following: -If full n5/n8 RF filters are used, UL CA cannot be used concurrently with n5 DL and no n8 UL can be used in DL CA =>significant drop to throughput making this CA benefit very questionable -If UL CA/concurrent transmission on n8 UL and n5 DL are required, the only way to make it work are dedicated filters, which obviously add burden to RF FE in many ways -CA_n5-n8 would be restricted to DL CA with only n5 UL, if full n5/n8 RF filters are used. We are ok to study both but we need to capture the facts associated in each of the options into TR. |
| MediaTek | Option 2. Share view with Oppo, Apple and Xiaomi. It is controversial if we agree on dedicate filter since RAN4 has agreement on UE shall still support full single band. We are a bit confused shall RAN4 define single band requirement based on dedicate filter? Or shall dedicate filter fulfill single full band requirement? |

GTW discussion on Oct. 11:

Skyworks: in our analysis, we show band n5 is heavily impacted. In our view, either we do not allow simultaneous transmission of UL n8 with DL n5 or we need find solution. It is too early to decide whether we can avoid the dedicated filter. We can decide it based on the analysis.

Qualcomm: if agreed to use the restrict frequency range and dedicated filter, does it mean that we specify the new bands?

OPPO: We have clarification on the question: does it mean filter design is dedicated or UE implement the specific filter? From UE implementation perspective, we should be careful. The frequency range is the specific operator demand. In the future, maybe other operators will request the dedicated filter.

Apple: Option 2. In our view, using dedicated filter based on restricted frequency range is not common. In 3GPP we often use the common filter. In this case, we should avoid to use the dedicated filter. The previous agreement is to consider supporting the full range.

Murata: in previous agreement, we should look at the common filter. If we use the dedicated filter and has no impact on legacy operation, we will degrade the inserssion loss. We prefer option2 and look at the solution.

Spark: when n5 is transmission mode and n8 is the reception mode, there will be problem. We should look at the solution. Users should meet the bar in terms of isolation.

Huawei: for the real implementation, if we want to achieve the good performance, we may consider the dedicated filter to achieve it. We can study such implementation.

Skyworks: in our analysis, we show even without considering MSD, the transmission will block the receiver anyhow. Unless we have agreement that n8 is not allowed to receive we do not see how to work. We can use sub-band.

Murata: we have the same concern on Skyworks and Spark.

Moderator: use the single band filter as baseline. Dedicated filter can also be analysed.

Issue 2-5: Possible solutions for CA_n5-n8

- Proposals

  - Option 1: n8 Tx restricted RBs

  - Option 2: n8 TX power reduction

  - Option 3: non-simultaneous Rx/Tx between n5 DL and n8 UL

  - Option 4: restrict the UL configuration is only in n5 for 1UL/2DL NR CA_n5-n8

- Recommended WF

Moderator: Please companies comment if all of the solutions are needed or only some of them are needed. How to use them, combined or can be separated. If there’re other solutions, please also comment.

| Company | Comments |
| --- | --- |
| Huawei | Currently, it’s very hard to choose one solution. It’s up to UE implementation and network scheduling. |
| Nokia | At this stage, all the options other than Option 4 should stay. It is apparent that Option 4 is the option when all the other three Options are not selected and there is nothing to do as study. Hence, Option 4 is not needed to be included for the study. |
| ZTE | Seems all options would be possible. But if band n8 Tx is allowed, then the OOB blocking of single band n5 would not be applied to CA_n5-n8. |
| OPPO | All can be considered for further study in our view to solve the interference issues. |
| Spark NZ | We have bands 5 and 8 concurrently operating in NZ and to resolve BS to BS interference we had guatdbands plus filters that provided in excess of 50 dB isolation. How will this be realsied in handsets? |
| Apple | Option 3 |
| Skyworks | If dedicated n8 UL filter is not allowed then the only option is option 3 or 4 (which seem to be the same). If so we do not see the need for 2UL in this CA either. |
| Murata | Non-simultaneous RX/TX requires network scheduling to operate FDD band in a time division manner. This imposes a extra switch for legacy stand-alone band operation. Dedicated filtering requires a bypass option which involves extra insertion loss for legacy stand-alone. Legacy filtering requires option 1 and 2. How much reduction and the lower acceptable limit determines the band combination’s feasibility. |
| vivo | all options can be considered for feasibility study. |
| Meta | We also think that all candidate solutions are considered in feasibility study. Then we can down select the solutions. |
| Qualcomm | This depends entirely on the full/dedicated RF filter decision. |
| CHTTL | Regarding option 1 is it correct understanding that the restricted RB is for testing only? |
| MediaTek | Options can be considered as part of the solution. |

Issue 2-6: IMD for 2UL CA_n5-n8

- Proposals

  - Option 1: No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

  - Option 2:

- Recommended WF

  - Option 1.

| Company | Comments |
| --- | --- |
| Huawei | OK with option 1. |
| Xiaomi | OK with option 1. |
| ZTE | OK with option 1. Next step, we can discuss how to implement it in the specification, as proposed R4-2216679 below. |
| OPPO | Option 1 is ok. |
| Apple | We are fine with Option 1 since the operation frequency range restriction has been agreed as the prerequisite for this band combination. |
| Skyworks | Regardless from the fact that there is no IMD issue (which we agree) we still have the issue with simultaneous n8UL and n5DL so unless this is solved in the 1UL case we do not see that 2UL can work. |
| Murata | Option 1 if the band combination is feasible with simultaneous n8TX and n5RX. |
| Vivo | OK with Option 1. |
| Meta | Fine with option 1 |
| China Telecom | Ok with option 1. |
| Qualcomm | Option 1 is ok. |
| CHTTL | In general we are fine, just one clarification question that the frequency ranges restriction will be applied forever on this band combination? |

Sub-topic 2-3: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 2-7: Other observations and proposals

  - Observations in R4-2216340

  - Observation: For NR CA using dedicated RF filters for restricted frequencies with two antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

  - Observation: For NR CA using dedicated RF filters for restricted frequencies with three antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz

| Company | Comments |
| --- | --- |
| Huawei | If we use the dedicated RF filters with 2 or 3 antennas, the worst-case MSD using estimated RF filter characteristics is ~5dB for 5MHz CC within 869…880MHz. If we use the dedicated RF filters, why do we need 3 antennas architecture? |
| Apple | Is there any dedicated filter data for reference for both duplexers and triplexers? |
| Skyworks | If dedicated filter should be used, in our view it should be only the n8UL range. We do not agree with the MSD calculation as it assumes that the PA is at the -110dBm/Hz noise floor which is not true as there are the IMDs of the wanted RBs and their images. |
| Murata | We need to investigate dedicated filter feasibility |
| Qualcomm | To Apple: The RF filters parameters were estimates To Skyworks: The PA noise was an estimate with 25 UL RB’s at upper corner of 10MHz n8 DL so IMD’s of the artefacts are not very high peaks. We will review the number next time. |

  - Proposals in R4-2216679

  - Even if a 3rd antenna is added to accommodate CA_n5-n8, in single band operation or other CA/DC cases, the Architecture should enable the use of only the two best antennas and n5/n8 UL use of the main antenna.

  - For the 3rd antenna it should be clarified whether or not this is a dedicated low band antenna or a borrowed antenna used for higher frequency bands.

| Company | Comments |
| --- | --- |
| OPPO | In our view these are possible in implementation whether the 3rd antenna is for dedicated low band antenna or borrowed antenna, and also possible to use the two best for single band operation. |
| Skyworks | In our view, if needed, the third antenna should only cover the specific aspects of this combination as it is anticipated that this one would have lower performance so the architecture should be such that the single band and other CA combinations can operate with the two best antennas. It is important to also understand what performance can be expected from the third antenna. |
| Qualcomm | To make the most of feasibility study, OEM comments on the OTA performance on the third antenna would be very welcomed; if the 3rd antenna is very bad, then combination which worst fine in conducted money might not work well in the field. |

  - Proposals in R4-2216679

  - Proposal on dual UL: With the 3rd antenna allowing to transmit n5 and n8 on separate antennas and using the n5 and n8 restricted range, there is no issue with dual transmission nor 2UL IMDs. No MSD due to IMD is needed but there should still be a dual UL MSD test point with NA for MSD and a note clarifying the applicable frequency ranges.

| Company | Comments |
| --- | --- |
| ZTE | We are fine with this proposal. There was already some configurations to keep the MSD as N/A with note in the spec. |
| OPPO | In our view, there is no need to further check the IMD if it is deemed no issue for dual transmission. The MSD testing is only defined for the cases whether MSD is defined. |
| Skyworks | Even if there no IMD issue with 2UL we still have a huge n5 MSD when n8 is transmitting and this is true whether this is single or dual UL (only the power is 3dB below) |
| Qualcomm | Content is fine, but whether we need dual UL MSD test point with N/A MSD depends on if we eventually specify dual UL for this case or not. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216072 TP and discussion on CA_n5-n8 Huawei, HiSilicon | ZTE: Basically we are fine to use TR38.718-02-01 as baseline. But considering it is SID TR, so we think it should add more concrete studies like MSD studies (at least we see other company provide MSD analysis). |
|  | Skyworks: we do not agree that Fig 5.X.1.1 is the reference architecture for 3 antennas (there are other options). For 2 antenna case there is need to clarify Which UL configurations are valid. We are not sure we can agree yet that there could be multiple capabilities for different ranges. In our view the frequency restrictions should be captured in notes as we have always done and not alter the band definitions. Since we have not agreed yet how to solve the n8UL issue it is not clear what 1UL MSD is captured nor if 2UL is valid. |
|  | Qualcomm: The reference architecture is premature to be captured in TP. Also, the group has not discussed what happens if we eventually agree to use restricted filters; one option would be to specify new bands according to those frequency ranges. Hence we cannot agree to capture bands n5 and n8 yet to be used for this CA yet as it might indicate no new bands are being discussed if frequency restricted filters are required. |
|  | CATT: Thanks for the good TP and comments/suggestions. The UE RF architecture assumption clause can be kept without the contents to see future discussions. The title for this clause can be modified to “UE RF architecture candidates”. And it can be stated as “In the study item phase, two antenna and 3 antenna architecture can be used in the feasibility analysis.” For the Tib/Rib and REFSENS clause, the contents can be removed then wait for more agreements. For rapporteur perspective, I also would like to see more background for the requirements in the TR. For the bands clause, propose to add some figures to show the relationship of the bands. The figures in this summary are copied from Apple contributions. They looks good and can be candidates. The above comments also apply to the other two TPs. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: Whether 2 antenna architecture is down selected for the requirements study | GTW agreement: - Both 2 and 3 antenna architectures will be analysed in the study item It will be decided in WI phase which one of two UE architectures will be used to specify the requirements. Recommendations for 2nd round: Capture the above agreement in the WF and no further discussion in 2nd round. |
| Issue 2-2: Which architecture is based for the UE RF requirement if two architectures are allowed? |  |
| Issue 2-3: RF parameters for requirements analysis | Based on the discussion in 1st round, majority companies thinks there should be some freedom for companies to assume the parameters needed for the analysis. So there’s no need to agree any list. Tentative agreements: Companies are encouraged to provide the RF parameters when the feasibility study is conducted. Recommendations for 2nd round: Confirm the tentative agreement in 2nd round. |
| Issue 2-4: Whether the filter can be dedicated, based on the restricted frequency range. | In the 1st round discussion, the views are divergent. Some companies commented that common filter should be used by single band and CA. Some companies showed concerns on the performance if single band filter is used. So moderator recommends that both can be analyzed by companies then the performance and some possible solutions can be discussed. The final decision can be made in WI phase. Tentative agreements: No agreement in 1st round. Candidate options: Both single band filter and dedicated filter can be used in the study. Recommendations for 2nd round: Confirm the above candidate option in the WF. |
| Issue 2-5: Possible solutions for CA_n5-n8 | According to the comments in the 1st round, majority companies think all of the solutions may be considered but it’s early to agree anything now. Tentative agreements: All of the solutions can be candidates at current stage. The detail solution for the corresponding RF architecture can be analyzed and discussed in future meetings. Recommendations for 2nd round: Confirm the above tentative agreement in the WF. |
| Issue 2-6: IMD for 2UL CA_n5-n8 | All companies are ok with no IMD proposal. Tentative agreements: No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8. Recommendations for 2nd round: Confirm the above tentative agreements in the WF. |
| Issue 2-7: Other observations and proposals | Companies provided detail comments on the detail analysis. It seems it’s hard to agree anything in this meeting. So moderator recommends we don’t have any agreement and no discussion in 2nd round but the MSD analysis is encouraged in future meetings. Tentative agreements: No agreement. Recommendations for 2nd round: No further discussion. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216072 TP and discussion on CA_n5-n8 Huawei, HiSilicon | To be revised. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: CA_n5-n28

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215663 | Apple | Observation 1: For CA_n5-n28 with 2-antenna implementation, the antenna design needs to cover the entire spectrum range of 191 MHz simultaneously which is equivalent to a 24% bandwidth ratio that would exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination likely would be compromised. Observation 2: For CA_n5-n8 with 3-antenna implementation, there is no additional insertion loss in both n5 and n8 main signal paths as compared to single-band implementation since n5 and n8 signals do not need to be combined through a multiplexer. Observation 3: Having two antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the two antenna as compared to single-antenna implementation. Proposal 1: For CA_n5-n28 with 2-antenna implementation, the feasibility of a single quadplexer in main path and a single triplexer in both main path and diversity path needs to be studied. Proposal 2: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor. Proposal 3: For CA_n5-n28, the REFSENS impact on n28 DL caused by frequency proximity between n5 UL and n28 DL and insufficient cross-band isolation needs to be addressed. |
| R4-2215898 | ZTE Corporation | Observation 1. No IMD MSD for 2UL/2DL CA_n5-n28, and the left requirements are delta Tib,c/Rib,c and cross band isolation requirements should be defined for 1UL/2DL CA_n5-n28. Observation 2. delta Tib,c/Rib,c and cross band isolation requirements have already been defined for 1UL/2DL CA_n5-n28 in the TS38.101-1. Observation 3. There may no further work for 2UL/2DL CA_n5-n28. |
| R4-2216040 | Xiaomi | Observation 1: 3 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem based on the commercial smartphone design. Currently 3 antenna architecture is much more preferred in real implementation for CA_n5A-n28A. Proposal 1: It is proposed to use the above RF front end architecture in figure 1 (2 antenna) and figure 2 (3 antenna) as baseline for further study. Proposal 2: For CA_n5A-n28A, RAN4 should clarify which set of RF requirement, e.g., worst case, will be used to define minimum requirements by considering all UE RF architectures. |
| R4-2216070 | Huawei, HiSilicon | TP for CA_n5-n28 |
| R4-2216345 | Qualcomm Finland RFFE Oy | Observation: Antenna aspects, Conducted RF performance, and RF FE architecture together define the feasibility of an LB NR CA band combination. Observation: Discussing and agreeing an antenna bandwidth threshold value for LB-LB CA could help in feasibility discussions Observation: Transmitting UL’s from different antennas would allow better optimization of each TRP MSD analysis using coarse component assumptions was conducted for two different RF FE filter lineups. Observation: There are several different n28-n5 RX triplexer implementation options with different tradeoffs Observation: The worst-case CA_n5-n28 Cross-band MSD using estimated RF filter characteristics in two antenna solution is 15.5dB for 5MHz n28 CC Observation: The worst-case CA_n5-n28 Cross-band MSD using estimated RF filter characteristics in three antenna solution is 13.8dB for 5MHz n28 CC Observation: In case smartphone design allows >10dB antenna isolation between LB’s, MRC MSD is significantly improved |
| R4-2216870 | Skyworks Solutions Inc. | Proposal: only two antenna architectures are studied for CA_n5-n28. Proposal on CA_n5-n28 dual UL test point: The worst-case MSD is evaluated for: - 30MHz n28 UL at the top of the band with 25RB at the top of the channel - 20MHz n5 UL at the bottom of the band with 20RB at the bottom of the channel. |

Open issues summary and company views collection

There’re two directions for the analysis for this CA. One is that 1UL CA requirements can be reused for 2UL CA. There’re also some contributions providing the detail analysis for 2UL CA requirements. If 1UL CA requirements can be reused, this CA can be closed. If not, the similar discussion, such as architecutre, RF parameters, etc, with other CA should be continued.

Sub-topic 3-1: Reusing the requirements of 1UL CA

Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28

- Proposals

  - Option 1: Yes

  - Option 2: No

- Recommended WF

Moderator: If companies comment no, please comment which requirement should be redefined and why.

| Company | Comments |
| --- | --- |
| Huawei | Option 1. Yes, it’s basic fallback rules. The requirements for 1UL CA_n5-n28 have been specified in current spec. |
| Xiaomi | Option 2. Whether to reuse the requirements of 1UL is dependent on the UE RF architecture. |
| ZTE | As mentioned by huawei, 1UL is the basic fallback for 2UL, which means the requirements for 1UL should be applied in advanced when 2UL is supported. However, for delta T/R values, it depends on whether the RF architecture for 1UL and 2UL are the same, if not, then delta T/R value may be different for 1UL and 2UL, otherwise, delta T/R value are the same for 1UL and 2UL |
| OPPO | Option 2, there is IM3/5 fall into n28 DL for 2UL case. And not clear the meaning of “reuse” here, does it mean no 2UL CA requirements to be defined and just use the 1UL requirement? |
| Apple | It may depend on whether we reuse the assumed 1UL CA architecture or not. |
| Skyworks | Option 2: No because in 2UL case the band n28 is subject to the combined cross band interference of n5 and n28UL and a specific MSD is needed as described in our paper. |
| Murata | AS for the way Issue 3-1 is being asked, No. Option 2. However, in our view, the MSD for DC_5_n28 should be applicable to CA_n5-n28 pending architectural assumption of the original contribution since DC_5-n28 involved 2 ULs. |
| vivo | It should be clarified which RF requirement can be reused based on what RF architecture assumption. |  |
| Meta | Option 2, RAN4 need to study the 2UL IMD issues and cross band isolation in the CA band combinations. |  |
| Qualcomm | As agreed by the group there are no IMD’s. We are open to analyse the cases pointed out by Skyworks (Cross-band MSD), however intuitively n28 MSD for any of the BW combinations for n5, n28 would not exceed currently specified [17.5] |  |

GTW discussion on Oct. 11:

OPPO: for the issue itself, does it mean when 2 UL fallbacks to 1 UL then 1 UL requirement will be applied, or 1 UL requirement will be applied to 2 UL. If it is latter one, it is no OK due to IMD issue.

Moderator: reuse 1 UL requirements for 2UL CA.

Apple: Can any companies provide the information what UE architecture is assumed to be derived the performance of 1UL in terms of filter, duplexer…

Skyworks: Disagree with using 1UL because 2UL is used and then n28 will be impacted by both n28 and n5 transmission. In this case, band n28 DL is not subjected to IMD but subjected to image… The MSD will be different than when only one UL.

Huawei: To Skyworks, you refer to MSD due to cross band isolation. For 2UL of this two combination, the only open requirements are MOP and coexistence requirements, which are not very difficult.

Xiaomi: we have concern on delta R and delta T. We should first agree on whether architecture for 1 UL and 2 UL should be the same.

ZTE: this one UL and two DL were studied in Rel-17. To Apple, we need time to check the original TP. For others, Delta T and Delta R depends on RF architectures.

Skyworks: to 2 UL case, it is cross band. But it is cross bands which transmit simultaneously. If we does not specify MSD, when UE is tested, we have issue.

Murata: Understand Skyworks concern. It could be some missing interruption and needs more investigation.

Sub-topic 3-2: Feasibility study if needed

Moderator: if the decision of Issue 3-1 is no, then continue the feasibility study for this CA.

Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis

- Proposals

  - Option 1: Only 3 antenna

  - Option 2: Only 2 antenna

  - Option 3: Both, the requirements are based on the worst case

  - Option 4: Both, the requirements are based on the best case

Moderator: This discussion should be limited to this specific CA.

| Company | Comments |
| --- | --- |
| Huawei | From implementation perspective, both RF architecture should be allowed. However, we can consider to specify requirements to include both implementation. |
| Xiaomi | Option 3. We are ok to consider both 2 antenna and 3 antenna architecture but prefer to use the worst case for RF requirements. |
| ZTE | We are not sure the further work here. Since n5-n28 1UL/2DL requirements have already been defined in R17 spec, and no IMD MSD issue for 2UL/2DL. |
| OPPO | Option 3 is preferred if consider both, and ok with Option 1. |
| Spark NZ | We support Xiaomi |
| Apple | We propose using 2 antenna as baseline to define requirements. |
| Skyworks | For requirement we should use the worst case 2 antenna implementation, whether a third antenna is implemented for radiating or MSD performance it is a UE implementation choice. |
| Murata | We are fine with option 3. Requirements in the past have sometimes been analyzed with more than one architecture. Option 4 is NOT acceptable. |
| Meta | Support Option 3 |
| Skyworks | Option 3. As this is feasibility study, both 2 and 3 antenna solutions should be investigated. Requirements should be specified according to worst case, which at least in conducted domain should be 2 antennas. |

GTW discussion on Oct. 11:

Skyworks: 2 and 3 antennas will have significant performance. We are fine to study the performance. We should use 2 antennas as baseline for requirements if there are not too much difference.

Moderator: have the same agreement for issue 2-1

Xiaomi: OK with it.

ZTE: we are not sure. For 2UL there is no IMD issue. Should we re-open the discussions in Rel-17?

Skyworks: we do not see the point. We are discussing the additional requirements based on the same architecture. We do not agree that the work is finished.

Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 3-3: RF parameters for requirements analysis

- Proposals

- Recommended WF

Moderator: Please comment in case further analysis is needed.

| Company | Comments |
| --- | --- |
| Skyworks | We have proposed the UL configurations to be evaluated for dual UL. Component parameters assumptions will be provided in the MSD analysis but we do not see the need to agree them. |

Sub-topic 3-3: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 3-4: Other observations and proposals

  - Observations in R4-2216345

  - Observation: The worst-case CA_n5-n28 Cross-band MSD using estimated RF filter characteristics in two antenna solution is 15.5dB for 5MHz n28 CC

  - Observation: The worst-case CA_n5-n28 Cross-band MSD using estimated RF filter characteristics in three antenna solution is 13.8dB for 5MHz n28 CC

  - Observation: In case smartphone design allows >10dB antenna isolation between LB’s, MRC MSD is significantly improved

| Company | Comments |
| --- | --- |
| Huawei | The estimated results seem better than current requirement specified in the spec. |
| ZTE | Are we going to revisit the existing R17 requirements in R18 SID? |
| OPPO | RF filter characteristics needs to be aligned before calculating the worst case MSD. More study is needed on this. |
| Skyworks | We do not agree with the PA noise assumption at -105dBm/Hz. The n28 DL is subject to IMD3 and IMD5 of wanted RBs and their image from both n8 and n28 ULs. |
| Murata | We need to bring analysis next meeting. |
| Xiaomi | We expect more study in the next meeting. |
| Qualcomm | To Skyworks: The IMD3/IMD5 products are included in the TXHD line. I will combine those next time to make it more clear. |

  - Proposals in R4-2216870

Proposal on CA_n5-n28 dual UL test point: The worst-case MSD is evaluated for:

  - 30MHz n28 UL at the top of the band with 25RB at the top of the channel

  - 20MHz n5 UL at the bottom of the band with 20RB at the bottom of the channel.

| Company | Comments |
| --- | --- |
| Huawei | The proposed test point doesn’t seem to be aligned with current specification. |
| ZTE | If the MSD test point are updated, we think it should be done in TEI work, like what we have done in the past, rather than in this SID. |
| Skyworks | These are the UL allocation for the REFSENS in 38.101-1 Table 7.3.2-3 only the position is changed for worst case 2UL and consistent with Table 7.3A.6-1 for n5. |
| Murata | We prefer to use the position and allocation for REFSENS as we are computing MSD, which is referenced to the single band position and allocation. We are open to look at the variation in MSD as a function of positioned allocation. |
| Qualcomm | If we go with this case, the MSD (on top of -78.5dB n28 30MHz REFSENS) would likely be small. Agree that the actual REFSENS for n28 would be worst case, but the amount of MSD would be much less than [17.5] agreed for 5MHz n28 in DL CA_n5-n28 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216070 TP and discussion on CA_n5-n28 Huawei, HiSilicon | ZTE: 1UL content have already been included in R17(?) basket WID TR. Seems no need to repeat here again. |
|  | Skyworks: MSD due to interference of the dual UL of n28 and n5 in n28DL is needed and different than the 1UL n5 case. |
|  | Qualcomm: The MSD section needs further considerations as discussed in above section. |
|  | CATT: Agree with ZTE that no need to repeat the study for the completed CA. Some referring could be ok. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28 | According to the 1st round discussion, some companies think 2UL can reuse 1UL requirement and no new requirements are needed. Some companies think 2UL cross band ISO issue should be revisited. We have agreed in last meeting that no IMD issues for 2UL CA. So moderator recommends to revisit the 2UL cross band MSD in next meeting. Tentative agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting. Recommendations for 2nd round: Confirm the above tentative agreement in the WF. |
| Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis | GTW agreement: - Both 2 and 3 antenna architectures will be analysed in the study item - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements. Recommendations for 2nd round: Capture the above agreement in the WF, no further discussion in 2nd round |
| Issue 3-3: RF parameters for requirements analysis | The comments for this issue are not many, but moderator would like to provide some guideline for future meetings. Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the RF parameters when the feasibility study is conducted. Recommendations for 2nd round: Confirm the above candidate option in the 2nd round. |
| Issue 3-4: Other observations and proposals | Companies provided detail comments on the detail analysis. There were some comments that CA_n5-n28 and DC_28-n5 are already specified. Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5. Recommendations for 2nd round: Confirm the above candidate option in the WF. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216070 TP and discussion on CA_n5-n28 Huawei, HiSilicon | To be revised |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4: CA_n8-n20-n28

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215664 | Apple | Observation 1: For CA_n8-n20-n28 with 2-antenna architectures, the antenna design needs to cover the entire spectrum range of 257 MHz simultaneously which is equivalent to a 31% bandwidth ratio that would far exceed the bandwidth ratio for a typical planar antenna design in a smartphone. As a result, the radiative performance for the combination is expected to be compromised. Observation 2: For CA_n8-n20-n28 with 3-antenna implementation, the 6 UL/DL bands can be shuffled between the two main path antenna to allow wider sub-band frequency separations for easier triplexer or duplexer design. Observation 3: For CA_n8-n20-n28 with 4-antenna implementation, there is no additional insertion loss in n8, n20, and n28 main signal paths as compared to single-band implementation since n8, n20, and n28 signals do not need to be combined through a multiplexer. Observation 4: Having 3 antenna in the main signal path not only avoids the more complicated multiplexer implementation and the associated additional insertion losses, but also allows narrower frequency coverage for each of the 3 antenna as compared to single-antenna implementation. Observation 5: The MSD mechanisms are highly dependent on the multiplexer filter isolation to the aggressor UL bands and the IMD products in the victim DL bands which may only be available after the multiplexer feasibility studies. Proposal 1: For CA_n8-n20-n28 with 2-antenna architectures, the feasibility on a low-band pentaplexer or quadplexer needs to be studied. Proposal 2: Implementation of more than 2 low-band antennas in a smartphone needs to be investigated, with narrower bandwidth and regressed radiating performance expected due to the limitation in form factor. Proposal 3: For CA_n8-n20-n28, the REFSENS impact (MSD) due to self-band isolation, cross-band isolation, and 2UL IMD needs to be addressed. |
| R4-2216041 | Xiaomi | Observation 1: Either 3 antenna architecture or 4 antenna architecture is able to be accommodated in a smartphone so the space of smartphone is not a problem based on the commercial smartphone design. Currently either 3 antenna architecture or 4 antenna architecture is much more preferred in real implementation for CA_n5A-n28A. Proposal 1: It is proposed to use the above RF front end architecture in figure 1 (2 antenna), figure 2 (3 antenna) and figure 3 (4 antenna) as baseline. Proposal 2: For CA_n8-n20-n28, RAN4 should clarify which set of RF requirement, e.g., worst case, will be used to define minimum requirements by considering all agreed UE RF architecture. |
| R4-2216071 | Huawei, HiSilicon | TP for CA_n8-n20-n28 |
| R4-2216346 | Qualcomm Finland RFFE Oy | Observation: Antenna aspects, Conducted RF performance, and RF FE architecture together define the feasibility of an LB NR CA band combination. Observation: Discussing and agreeing an antenna bandwidth threshold value for LB-LB CA could help in feasibility discussions Observation: Transmitting UL’s from different antennas would allow better optimization of each TRP MSD analysis using coarse component assumptions was conducted for three different RF FE filter lineups. MSD due to IMD3 is 25dB for some other LB-LB cases already specified in 3GPP (e.g. for CA_n5A-n14, CA_8A-20A). These results are well aligned with those. Notably, increasing the number of antennas does not significantly reduce the IMD3 MSD. If smartphone design allows >10dB antenna isolation between LB’s, there is significant improvement in MRC MSD. More rigorous MSD analysis can be performed once the candidate RF architecture options are more clear. Observation: IMD3 MSD analysis results using coarse parameter assumptions are aligned with other existing LB-LB IMD3 MSD results Observation: Increasing the number of antennas does not significantly reduce the IMD3 MSD Observation: In case smartphone design allows >10dB antenna isolation between LB’s, MRC MSD is significantly improved |
| R4-2216876 | Skyworks Solutions Inc. | Proposal: only two and three antenna architectures are studied for CA_n8-n20-n28 Proposal on CA_n8-n20 and CA_n20-n28 test points: The worst-case MSDs are evaluated for: - CA_n8-n20: - 20MHz n20 UL at the top of the band with 20RB in the middle of the channel - 15MHz n8 UL at the bottom of the band with 20RB in the top of the channel - CA_n20-n28: - 20MHz n20 UL at the bottom of the band with 20RB in the middle of the channel - 20MHz n28 UL at the top of the band with 20RB in the top of the channel |

Open issues summary and company views collection

Sub-topic 4-1: UE RF architecture downselection

Issue 4-1: UE RF architecture down selection

- Proposals

  - Option 1: 2 antenna is down selected

  - Option 2: 4 antenna is down selected

  - Option 3: none is down selected.

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Huawei | From implementation perspective, we don’t need to exclude any RF architecture. But we can consider to specify the RF requirements for CA_n8-n20-n28 based on the 3 antenna RF architecture. |
| Xiaomi | No down selection is acceptable to us. If down selection has to be allowed our preference is to consider 3 antenna and 4 antenna architecture for feasibility study. |
| ZTE | We would like to know whether ‘down selected’ means the RF requirements will not be studied for 2/4 antenna architecture in this SID? It could be foreseen that the requirements between 2/4 antenna architectures and 3 antenna architectures would be different. |
| OPPO | Option 1 or 3. Disagree with Option 2. Four antennas can be supported by smartphone with large size for example the folding UE which have the size similar as PAD. |
| Apple | The EN-DC counterpart of this 3-band combination DC_8-20_n28 has already been specified in Rel-17, which however was only targeted for FWA devices due to the split antenna implementation in the main signal path where the additional low-band antenna dimension may not be accommodated by a smartphone form factor. Our preference is to use 2 antenna as baseline architecture to define RF requirements for smartphone. |
| Skyworks | We agree that no implementation should be excluded but the requirement should be for the worst case (2 antenna) or we first need to agree how the UE may declare different antenna architectures. In any case we do not think that the requirement should be based on 4 antennas. |
| Murata | 3-antenna as baseline as per GTW and not preclude 2 or 4 antenna. |
| Meta | We are fine on the agreement in GTW session. |
| Qualcomm | GTW is a good baseline. |

GTW discussion on Oct. 11:

Skyworks: in our view, 4 antenna case does not bring in any performance improvement.

OPPO: 3 antenna is OK as baseline. But 4 antenna is also feasible. In the market, we have the big form factor like folded UE. For 2 antenna, we think the performance is bad.

Apple: we propose to exclude 4 antenna architecture. 3 antenna is connected to each individual band. Then there is no additional insertion loss. Probably 4 antenna is not common design.

Qualcomm: agree with Skyworks. 4 antenna may not bring in the significant gain. 3 antenna is too way forward.

Xiaomi: for 2 antenna, it is rather difficult for low band design multi-plexer design. We would like to give priority to 3 and 4 antennas.

Murata: In previous similar analysis, we use 3 antenna.

Samsung: down-select 4 antennas.

Apple: we still want to include 2 antenna architecture. It is possible option for UE to consider.

Agreement:

- Use 3 antenna UE architecture as the baseline for evaluation.

  - Do not preclude 2 and 4 antennas in the study item

Issue 4-2: How to define the requirements when more than one architectures are allowed for CA_n8-n20-n28

- Proposals

  - Option 1: Best case

  - Option 2: Worst case

- Recommended WF

Moderator: This discussion should be limited to this specific CA.

| Company | Comments |
| --- | --- |
| Huawei | We can consider to specify the RF requirements for CA_n8-n20-n28 based on the 3 antenna RF architecture. |
| Xiaomi | Option 2. RAN4 is to specify minimum RF requirements so the worst case is more applicable. |
| ZTE | Which architectures lead the worst/best requirements? |
| OPPO | Option 2 is ok. |
| Apple | Our preference is to use 2 antenna as baseline architecture to define RF requirements for smartphone. |
| Skyworks | The requirement should capture and enable the worst case implementation unless we agree on some BW threshold or derive worst case MSD for two and three antennas with dedicated signaling. In order to conclude we need first to see if the difference is large. |
| Murata | Option 2 |
| CATT | It can be decided in WI phase as agreed in GTW, There’s no need to discuss this issue in SI phase. |
| Meta | Option 2. |
| Qualcomm | The requirements should be made according to worst case which still is feasible. E.g case which have LB 2 PA’s transmitting from same antenna with IMD3 having >20dB MSD should be carefully considered as the likelihood of someone actually implementing it is limited. As the number of LB’s in LB CA increase, 2 antenna becomes less and less likely implementation option. |

Sub-topic 4-2: Feasiblity study

Issue 4-3: RF parameters for requirements analysis

- Proposals

- Recommended WF

Moderator: Please comment for further analysis in next meeting.

| Company | Comments |
| --- | --- |
| Skyworks | UL configuration is proposed in our contribution, component parameters can be provided in the MSD analysis but we do not see the need to agree on them. Antenna isolation should be 10dB |
| Murata | Need WF for initial list. |

Sub-topic 4-3: Other observations and proposals

Moderator: In this secition, other observations and proposals are captured. Companies can comment below every propopal/observation below.

Issue 4-4: Other observations and proposals

  - Observations in R4-2216346

  - Observation: IMD3 MSD analysis results using coarse parameter assumptions are aligned with other existing LB-LB IMD3 MSD results

  - Observation: Increasing the number of antennas does not significantly reduce the IMD3 MSD

| Company | Comments |
| --- | --- |
| XXX |  |

  - Proposals in R4-2216876

Proposal on CA_n8-n20 and CA_n20-n28 test points: The worst-case MSDs are evaluated for:

  - CA_n8-n20:

    - 20MHz n20 UL at the top of the band with 20RB in the middle of the channel

    - 15MHz n8 UL at the bottom of the band with 20RB in the top of the channel

  - CA_n20-n28:

    - 20MHz n20 UL at the bottom of the band with 20RB in the middle of the channel

    - 20MHz n28 UL at the top of the band with 20RB in the top of the channel

| Company | Comments |
| --- | --- |
| Huawei | In 38.101-3, MSD due to IMD for DC_20_n8 and DC_20_n28 have been specified. We can consider to reuse these test point. |
| ZTE | If the MSD test point are updated, we think it should be done in TEI work, like what we have done in the past, rather than in this SID. |
| Skyworks | In our proposal we are defining UL configurations with IMD in the third band also so we do not see the link to the two band DC case. |
| Murata | Prefer to keep original single band REFSENS allocation and position and calculate MSD as the value is referenced from REFSENS. The IMD fall back combinations can then be used similar to DC_20_n8. |
| Meta | We are same page with Huawei. The existing test configuration in DC_20_n8 and DC_20_n28 will be used for CA_n8-n20 and CA_n20-n28. |
| Qualcomm | Prefer to keep original test points for 20-8 go from there, however we need co consider test points for 3rd band MSD. Using 5MHz CC’s where possible we get the worst case MSD. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216071 TP and discussion on CA_n8-n20-n28 Huawei, HiSilicon | ZTE: except the architecture information, the TP looks like a basket WID TP. Although we agree to use TR38.718-02-01 as baseline template, maybe some more concrete studies can be added (pending on discussion). |
|  | Skyworks: we do not agree with the two antenna architecture. Or more need to be captured. Ok with 3 antenna case but there may be others. In our view 4 antenna case should not be covered. With n28 frequency restriction n28 DL is only subject to IMD5 of n20+n8UL |
|  | Qualcomm: The reference antenna architectures are problematic, as there may be several others. It would be good not to capture them yet, but in next meeting instead after we have more analysis at hand. |
|  | CATT: For 1UL CA_n8-n20, CA_8-n28, I suggest the details need not to be captured in the TR. If some clarification is needed, maybe just referring the spec and the original TR could be ok. The clauses may be separated to the following to be clear: UE RF architecture candidates CA_n8-n20 CA_n8-n28 CA_n20-n28 CA_n8-n20-n28 |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 4-1: UE RF architecture down selection | GTW Agreement: - Use 3 antenna UE architecture as the baseline for evaluation. - Do not preclude 2 and 4 antennas in the study item Recommendations for 2nd round: Capture the above agreement in the WF, no further discussion in 2nd round |
| Issue 4-2: How to define the requirements when more than one architectures are allowed for CA_n8-n20-n28 | To follow the agreement for CA_n5-n8 and CA_n5-n28, the tentative agreement is as following, Tentative agreement: - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements. Recommendations for 2nd round: Confirm the above tentative agreement in the WF. |
| Issue 4-3: RF parameters for requirements analysis | No many comments are provided in the 1st round. Moderator has the similar suggestions with the other two CA. Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the RF parameters when the feasibility study is conducted. Recommendations for 2nd round: Confirm the above candidate option in the WF. |
| Issue 4-4: Other observations and proposals | There were some comments for the test points in the 1st round that DC_20_n8 and DC_20_n28 are specified in spec, they can be reused. The response was that the third band MSD should consider the UL configuration. Moderator’s understanding is that it’ll be good that the previous study outputs are used as much as possible. So the suggestions are, Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the MSD analysis for CA_n8-n20-n28 considering the already specified 1UL CA and DC. Recommendations for 2nd round: Confirm the above candidate option in the 2nd round. |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216071 TP and discussion on CA_n8-n20-n28 Huawei, HiSilicon | To be revised |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on CA_n5-n8 | Xiaomi |  |
|  | WF on CA_n5-n28 | ZTE |  |
|  | WF on CA_n8-n20-n28 | Skyworks |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215445 |  | CA_n5-n8 Feasibility and assumptions for UE-UE RF Requirements | Murata Manufacturing Co Ltd. | Noted |  |
| R4-2215662 |  | On CA_n5-n8 architectures | Apple | Noted |  |
| R4-2215663 |  | On CA_n5-n28 architectures | Apple | Noted |  |
| R4-2215664 |  | On CA_n8-n20-n28 architectures | Apple | Noted |  |
| R4-2215730 |  | Skeleton of TR 38.872 | CATT | Noted |  |
| R4-2215896 |  | Feasible on CA band combination of n5-n8 | ZTE Corporation | Noted |  |
| R4-2215897 |  | RF requirements on CA band combination of n5-n8 | ZTE Corporation | Noted |  |
| R4-2215898 |  | RF requirements on CA band combination of n5-n28 | ZTE Corporation | Noted |  |
| R4-2216039 |  | Discussion on feasibility study for CA_n5-n8 | Xiaomi | Noted |  |
| R4-2216040 |  | Discussion on feasibility study for CA_n5-n28 | Xiaomi | Noted |  |
| R4-2216041 |  | Discussion on feasibility study for CA_n8-n20-n28 | Xiaomi | Noted |  |
| R4-2216070 |  | TP and discussion on CA_n5-n28 | Huawei, HiSilicon | Revised |  |
| R4-2216071 |  | TP and discussion on CA_n8-n20-n28 | Huawei, HiSilicon | Revised |  |
| R4-2216072 |  | TP and discussion on CA_n5-n8 | Huawei, HiSilicon | Revised |  |
| R4-2216136 |  | Further discussion on feasibility aspects for 700800900MHz CA band combinations | vivo | Noted |  |
| R4-2216137 |  | Further discussion on RF requirement impacts for 700800900MHz CA band combinations | vivo | Noted |  |
| R4-2216340 |  | Considerations on CA_n5-n8 | Qualcomm Finland RFFE Oy | Noted |  |
| R4-2216345 |  | Considerations on CA_n5-n28 | Qualcomm Finland RFFE Oy | Noted |  |
| R4-2216346 |  | Considerations on CA_n8-n20-n28 | Qualcomm Finland RFFE Oy | Noted |  |
| R4-2216679 |  | CA_n5-n8 with restricted frequency range | Skyworks Solutions Inc. | Noted |  |
| R4-2216870 |  | CA_n5-n28 MSD for 2UL case | Skyworks Solutions Inc. | Noted |  |
| R4-2216876 |  | CA_n5-n20-n28 MSD for 2UL case | Skyworks Solutions Inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents