3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2217139

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.5.2

Source:	Moderator (Samsung)

Title:	Email discussion summary for [104-bis-e][206] NR_feMIMO_RRM_2

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

This email discussion summary covers the contributions submitted under the following AI:

- 4.5.2	RRM performance requirements

  - 4.5.2.1	General (test configurations, side condition and etc)

  - 4.5.2.2	Test cases for unified TCI state switching

  - 4.5.2.3	Test cases for L1-RSRP measurement on cells with different PCI

  - 4.5.2.4	Test cases for TRP specific BFD and LR

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Discuss the open issues and conclude. Agree on some draft CRs if possible.

- 2nd round: According to the 1st round discussion, strive to agree on draft CRs.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Samsung (Moderator) | Yanze Fu | yanze.fu@samsung.com |
| Ericsson | Venkat | Venkatarao.gonuguntla@ericsson.com |
| Apple | Manasa Raghavan | Manasa.raghavan[AT]apple.com |
| Huawei | Hong Li | no.li@huawei.com |
| MediaTek | ChihKai Yang | ck.yang@mediatek.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Test cases for unified TCI state switching

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216364 | vivo | Observation 1 It can be inferred from TS 38.213 that if pathlossReferenceRS-Id-r17 is not configured, it would mean PL RS is ‘included in’ UL TCI, and the corresponding source RS of UL TCI should be DL RS, and can be used as pathloss RS. Proposal 1 RAN4 assumes that source RS of UL TCI can be used as pathloss RS if pathlossReferenceRS-Id-r17 is not configured. Therefore, do not explicitly configure pathloss RS in joint TCI case and UL TCI test case. Proposal 2 RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list. Proposal 3 RAN4 may clarify in Note 4 of A.3.16.A.2-1 by adding the following sentence. ‘The TCI state of the TRS is the same as TCI.state.1 except that the additionalPCI field is also configured with PCI 0.’ In this case, no need to introduce a new TRS configuration or new TCI configuration. Proposal 4 Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE. Proposal 5 No need to specify “90%” threshold for the rate of correct events in the unified TCI state switching test case. |
| R4-2216365 | vivo | Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA |
| R4-2215745 | Samsung | Proposal 1: For PL-RS configuration in joint TCI test case, prefer not to configure pathloss RS. Proposal 2: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”. |
| R4-2215766 | MediaTek Inc. | Draft CR on TC for joint unified TCI state switching in FR2 NR SA |
| R4-2216822 | Ericsson | CR on maintenance of UL TCI state switching of FR2 PCell |

Open issues summary

Sub-topic 1-1: Pathloss RS configuration

Issue 1-1-1: Pathloss RS configuration in joint TCI test case

- Proposals

  - Option 1 (vivo)

    - RAN4 assumes that source RS of UL TCI can be used as pathloss RS if pathlossReferenceRS-Id-r17 is not configured. Therefore, do not explicitly configure pathloss RS in joint TCI case and UL TCI test case.

  - Option 1a (Samsung)

    - For PL-RS configuration in joint TCI test case, prefer not to configure pathloss RS.

  - Option 2 (MTK (CR-2215766))

    - PL-RS is configured. UE should maintain PL-RS before and after TCI state switch in the test.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We think some of the test cases are already defined with maintained and some are defined with non-maintained. We think even for proposal 1 and 2, we need to discuss if the PL-RS is maintained or not. |
| vivo | Support option 1. Not sure what does it mean by ‘not to configure pathloss RS’ in option 1a. If it means that the optional IE pathlossReferenceRS-Id-r17 is absent, then we are also OK to option 1a. For option 2, not sure what does it mean by ‘PL-RS is configured’. Does it mean ‘pathlossReferenceRS-Id-r17’ present in UL TCI or joint TCI? Or it means PL-RS inferred by UL TCI, i.e. implicit configuration, is also included? |
| Apple | We support option 2, to configure PL-RS and is part of the Joint active TCI list, so it is measured and maintained before the Joint TCI switch. We are not sure what is the purpose of not configuring PL-RS in the testcase. |
| Huawei | PL-RS in joint/UL TCI state can be either explicitly configured or implicitly indicated (same as QCL-typeD source RS). We prefer that PL-RS in joint/UL TCI state is explicitly configured. |
| Moderator | GTW discussion on Oct 11 is captured as below. It is suggested to continue the discussion. |
| Samsung | To Ericsson: to my understanding, there were 6 test cases in bigCR agreed in last meeting. All two TCs for UL TCI is defined as PL-RS is not maintained. For two Joint TCI test cases, there is no decision. So in the WF(2214483) in last meeting, it is still open Pathloss RS configuration in joint TCI test case - Option 1: Pathloss RS is maintained - Option 2: Do not configure pathloss RS in joint TCI case In our understanding, PL-RS can be not configured in jointTCI. If it is not configured, and use the same principle in UL TCI test case, which PL-RS is not maintained. But we can accept PL-RS is explicitly configured but it is not maintained. |
| MediaTek | Prefer option 2. Prefer to explicitly configure PL-RS in the joint/UL TCI state and the PL-RS is maintained to reduce the test time (note: non-maintained case is already tested in UL TCI.). |
| Apple2 | As we commented in the GTW, we should explicitly configure the PL-RS in the test case for all UL and joint TCI state switch, since we are testing the TCI state switch/ PL-RS switch functionality and not fallback procedures if it is optionally not configured. |

GTW on Oct 11:

Discussion:

Huawei: we support to configure the PLRS explicitly for joint TCI state swiching and UL TCI state test cases.

Apple: we also prefer to configure explicitly. PLRS is configured but whether it is maintained or not defpends on the test cases.

Samsung: in joint TCI test cases since the PLRS can be option al field we prefer not to configure it. Use the same principle in the UL test cases where the PLRS is not maintained.

Vivo: in the last meeting there was one LS sent to RAN1 about what the cases are if the PLRS is not configured. We should revisit this one after RAN1 feedback.

Nokia: we have similar view as vivo.

Ericsson: when the PLRS is not configured explicitly the source could be the PLRS. It is typical case and we should test it. We could wait for RAN1 feedback and come back to it.

Apple: in our understanding the LS was about clarifying the active UL TCI state list and the relation of maintaining PLRS. It is a different issue. We need to check further on the RAN1 spec about default behaviour when PLRS is not configured. We should focus on the switching itself in the test cases in stead of testing the fall back behaviours.

Ericsson: in other WI, RAN1 spec mentions that the default behaviour is to use the source RS as the PLRS. We could reuse.

Huawei: we agree with Apple that the tests are for TCI state switching but not to verify the default behaviour.

Issue 1-1-2: How to define PL-RS of target TCI?

- Proposals

  - Option 1 (vivo)

    - RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | If it is QCL-D, does it guarantee PL-RS maintained or not-maintained. We do not understand the impact of this proposal. |
| Vivo | Support option 1. We support to ‘only define test case when PL-RS is not maintained’. If it is neither the same RS as nor QCL-D to the source RS of target TCI, we think it would ensure that UE will not maintain the PL-RS before TCI switching. |
| Apple | Could proponents please clarify of this is for Joint TCI or UL TCI state switch? For eparate UL TCI we agreed on tet case with PL-RS not maintained, so the target TCI state should be known but not in the active UL TCI list. |
| Huawei | There are two TCI states configured in the test, and the two TCI states need to be included in TCI state list by RRC signaling. And the active TCI state list is indicated by MAC-CE command. It is not clear for us why the QCL relations between PL-RSs of the TCI states in active TCI state list need to be clarified. Whether PL-RS is maintained or not does not depend on the QCL relations. |
| Moderator | GTW discussion on Oct 11 is captured as below. It is suggested to continue the discussion. |
| Samsung | We don’t understand how this option to affect the test cases. Could vivo explain it more for how to modify current TCs? |
| vivo2 | To Samsung, if option 1 is agreeable then in A.5.5.11.2.1.2, the following can be removed. Two SSB are not QCL-D with each other, and UE is assumed not maintaining Pathloss RS of SSB#1 before the TCI is switched. No revision is needed for A.7.5.13.2.1 |
| MediaTek | Still unclear what is the relation between the removed sentence and the proposal. Besides, why we only need to remove the sentence in A.5.5.11.2.1.2 but not in A.7.5.13.2.1 (both are UL TCI state switch). |
| Apple2 | The PL-RS maintained condition depends on whether the target PL-RS is in active Joint/UL list. We don’t think the QCL-Type D determines maintained or not maintained. |

GTW on Oct 11:

Discussion:

Vivo: we should specify the TC when the PLRS is not maintained. Which means that the PLRS is not QCL-ed type D with any RS that is within the active TCI state list.

Nokia: this also the discussion point in core discussion. How to specify the definition of maintained PLRS has impact on the delay requirements. We need to go to core discussion first.

Huawei: whether it is maintained or not has nothing to do with the delay. When there are over 4 RS configured the UE could not maintain all of them.

Apple: we agree with Huawei. If the PLRS is in the active TCI state list, the UE is expected to maintain the PLRS. We sent an LS to RAN1 to ask about the correct behaviour if the number is over 4.

Vivo: to clarify, why do we need to configure more than 4 RS in the test cases.

Nokia: we do not have TCI state list for UL. We can have up to 4 tracked by the UE according to RAN1 agreement. It is ok to wait for the reply.

Apple: in the unified TCI state framework in R17, we have UL TCI state/joint TCI state list. We don’t need to configure more than 2 RS in the list if we assume the PLRS is maintained.

Nokia: we don’t have a clear agreement if the UE tracks the timing on the RS in the list.

Issue 1-1-3: How to configure maintained PL-RS / NOT maintained PL-RS in the test case

- Proposals

  - Option 1 (Samsung)

    - In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We already defined test cases for both maintained and non-maintained. Not sure why we need to discuss this now. |
| vivo | Support option 1. |
| Apple | We have agreement in previous meetings that PL-RS is maintained for joint TCI state switch test and not maintained for separate UL TCI state switch test. In both cases the PL-RS should be known ie configured for L1-RSRP measurement. In case of maintained PL-RS it is in active UL TCI state list and in case of not-maintained, it is not in active UL TCI state list. |
| Huawei | Whether to maintain PL-RS is up to UE implementation, which cannot be configured. |
| Samsung | To Ericsson and Apple: to my understanding, there were 6 test cases in bigCR agreed in last meeting. It is agreed all two TCs for UL TCI is defined as PL-RS is not maintained. For two Joint TCI test cases, there is no decision. So in the WF(2214483) in last meeting, it is still open In addition, for joint TCI test cases in A.7.5.13 &A.5.5.11 Editor note’s: FFS whether the CSi-RS #0 is “maintained” or “not configured” Please correct if my misunderstanding. How to configure maintained PL-RS / NOT maintained PL-RS in the test case - Proposal 1: If an original PL-RS is configured, it is a maintained PL-RS; If a new RS is configured as PL-RS, it is a “not maintained PL-RS” - Proposal 2: Whether to maintain PL-RS is up to UE implementation. The value of delay in test requirement can be calculated based on the worst case. - Proposal 3: Before sending the TCI switch command, UE is expected to measure on the active TCI list and maintain the PL-RS. - Proposal 4: For non-maintain PL-RS, PL-RS is configured in TCI state and TCI state is unknown. For this open issue in last meeting, firstly, we support PL-RS is not maintained in all test cases (UL TCI TCs have been already agreed). And in the test setup, configure a new RS as PL-RS , it is PL-RS is not maintained. |
| Apple2 | Thanks, Samsung, for the clarification. We are fine to configure PL-RS as not maintained for all Joint and Ul TCI switch test cases. If only 1 active TCI state is configured, and with TCI switch a new TCI state and new PL-RS is activated, then it would be PL-RS not maintained. The PL-RS and RS in UL/Joint TCI state should be the same or QCL Type D with each other, and the target UL /joint TCI should be configured for L1 measurement to ensure known state. |

Sub-topic 1-2: Additional PCI related configuration

Issue 1-2-1: TRS configuration for cell with different PCI in the test case

- Proposals

  - Option 1 (vivo)

    - RAN4 may clarify in Note 4 of A.3.16.A.2-1 by adding the following sentence. ‘The TCI state of the TRS is the same as TCI.state.1 except that the additionalPCI field is also configured with PCI 0.’ In this case, no need to introduce a new TRS configuration or new TCI configuration.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| vivo | Support option 1. |
| Huawei | New TRS configuration is needed for indicating that the TRS in DL TCI state associated with additional PCI is QCL-typeD to a SSB with the same additional PCI. |

Issue 1-2-2: Whether to provide RRC configuration of the cell of additional PCI?

- Proposals

  - Option 1 (vivo)

    - Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| vivo | Support option 1. |
| Apple | Not sure what this proposal intends. The necessary information for cell with additional PCI should be provided as part of serving cell RRC configuration. |
| Samsung | We don’t understand what is the modification in the test cases. Could vivo explain more? |
| vivo2 | To Samsung, at least the Cell 3 in table A.5.5.11.3.1.1-3 can be removed. Configuration of Cell 3 is not provided in the other test cases when cell with different PCI is considered. |
| MediaTek | We prefer to keep it as it is. Because this case is for TCI state switch which means UE will receive the signals from the cell 3. |
| Apple2 | Thanks Vivo for the clarification. Perhaps we need some update to the Cell 3 parameters, we only configure the ones necessary for the test/ Additional PCI setting, others can be set as N/A. |

Sub-topic 1-3: Others

Issue 1-3-1: The rate of correct events for unified TCI state switching test case

- Proposals

  - Option 1 (MTK (CR-2215766))

    - Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the unified TCI state switching test case.

  - Option 2 (vivo)

    - No need to specify “90%” threshold for the rate of correct events in the unified TCI state switching test case.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with option 1 as we are defining this for most of the test cases. |
| vivo | We prefer option 2. We prefer to keep consistence with legacy TCI state switching related test cases. |
| Apple | Support option 1. We see that in Rel-15/16 requirements there is an inconsistency – the UL spatial relation info switch test cases have “ The rate of correct events observed during repeated tests shall be at least 90%.”, where as the Active DL TCI state switching cases done. In our understanding both should have this and we can bring CRs in next meeting to make it consistent for Rel-15/16/17 test requirements. |
| Huawei | Option 1. “90% rate of correct events during repeated tests” shall be specified for all the RRM test cases. |
| Samsung | We think the reason why 90% in introduced in some of RRM test cases is because in A.2 “The statistical nature depends on the type of requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a DUT passing the test actually meets the requirements and determines how many times a test has to be repeated and what the pass and fail criteria are. Those aspects are not covered by TS 38.133. The details of the tests on how many times to run it and how to establish confidence in the tests are described in TS 38.533 [5]. This Annex establishes the variable to be used in the test and whether it can be viewed as statistical in nature or not. All have in common that the UE is required to perform an action observable in higher layers (e.g. camp on the correct cell) within a certain time after a specific event (e.g. when a new strong pilot or reference signal appears). The delay time is statistical in nature for several reasons, among others that several of the measurements are performed by the UE in a fading radio environment. The variations make a strict limit unsuitable for a test. Instead there is a condition set for a correct action by the UE, e.g. that the UE shall camp on the correct cell within X seconds. Then the rate of correct events is observed during repeated tests and a limit is set on the rate of correct events, usually 90% correct events are required. How the limit is applied in the test depends on the confidence required, further detailed are in TS 38.533 [5].” For most RRM tests, some evaluation such as RSRP or fading environment, it needs the rate of correct events. But for TCI state switching, there is few variation. We think it is fine to not add the rate. But if all companies agree to add it and also R15/16 cases, we can accept adding it to make consistent with all cases. |
| MediaTek | Support option 1 since it has been defined in R16 spatial relation TC. |

Companies views’ collection for 1st round

Open issues

Please add comments in tables in 1.2

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216365 vivo | Moderator: DraftCR, correction to A.7.5.13.3 (NR SA + FR2 + Cell with different PCI; DL TCI ) |
|  | Ericsson: OK |
|  | Huawei: the changes for TCI and TRS configuration are not correct. |
| R4-2215766 MediaTek Inc. | Moderator: DraftCR, correction to A.7.5.13.1 (NR SA + FR2 + Serving Cell; Joint TCI) |
|  | Ericsson: OK |
|  | vivo: Pending on above issues. |
| R4-2216822 Ericsson | Moderator: CR, correction to A.7.5.13.2 (NR SA + FR2 + Serving Cell; UL TCI) |
|  | vivo: OK |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Issue 1-1-1: Pathloss RS configuration in joint TCI test case | 7 Companies provided comments in 1st email discussion and GTW discussion. Tentative agreements: N/A Candidate options: In the email discussion and GTW discussion, the candidate options can be reorganized as: Option 1: RAN4 assumes that source RS of UL TCI can be used as pathloss RS if pathlossReferenceRS-Id-r17 is not configured. Therefore, do not explicitly configure pathloss RS in joint TCI case and UL TCI test case. (vivo) Option 2: Configure the PLRS explicitly for joint TCI state swiching and UL TCI state test cases (Huawei, Apple, MTK) Option 3: Wait for RAN1 feedback for LS (vivo, Nokia, Ericsson) For the issues 1-1-1-b whether PL-RS is maintained or not maintained: Option 1: Pathloss RS is maintained in joint TCI test case (MTK) Option 2: Pathloss RS is not maintained in joint TCI test case (Samsung, Apple) Recommendations for 2nd round: Continue discussion |
| Issue 1-1-2: How to define PL-RS of target TCI? | 7 companies provided comments in 1st email discussion and GTW discussion. - Option 1 (vivo) - RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list. Tentative agreements: N/A Recommendations for 2nd round: Continue discussion. Companies still have unclear and concern on option 1 and vivo provides the potential spec impact if option 1 is agreeable. Please keep further discussion. |
| Issue 1-1-3: How to configure maintained PL-RS / NOT maintained PL-RS in the test case | 5 companies provided comments in 1st discussion. Tentative agreements: N/A Candidate options: Option 1: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”. (Samsung, vivo) Option 2: Whether to maintain PL-RS is up to UE implementation, which cannot be configured (Huawei) Recommendations for 2nd round: Continue discussion. |
| Issue 1-2-1: TRS configuration for cell with different PCI in the test case | Only two companies provided comments in 1st round. Candidate options: - Option 1 (vivo) - RAN4 may clarify in Note 4 of A.3.16.A.2-1 by adding the following sentence. ‘The TCI state of the TRS is the same as TCI.state.1 except that the additionalPCI field is also configured with PCI 0.’ In this case, no need to introduce a new TRS configuration or new TCI configuration. - Option 2 (Huawei) - New TRS configuration is needed for indicating that the TRS in DL TCI state associated with additional PCI is QCL-typeD to a SSB with the same additional PCI. Tentative agreements: N/A Recommendations for 2nd round: Companies are encouraged to provide more comments. |
| Issue 1-2-2: Whether to provide RRC configuration of the cell of additional PCI? | 4 companies provided comments in 1st round. - Option 1 (vivo) - Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE. vivo proposed option 1 and explained example of spec impact. Apple/Samsung/MTK have concern on it. Recommendations for 2nd round: From moderator’s perspective, could we stop discussion in 2nd round and vivo bring such CR in next meeting to discussion? |
| Issue 1-3-1: The rate of correct events for unified TCI state switching test case | 6 companies provided comments in 1st round. - Option 1 (MTK, Ericsson, Apple, Huawei, Samsung) - Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the unified TCI state switching test case. - Option 2 (vivo, Samsung) - No need to specify “90%” threshold for the rate of correct events in the unified TCI state switching test case. Recommendations for 2nd round: If no strong objection of option 1, could we agree option 1 based on majority views? |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 1-1-1: Pathloss RS configuration in joint TCI test case

- Option 1: RAN4 assumes that source RS of UL TCI can be used as pathloss RS if pathlossReferenceRS-Id-r17 is not configured. Therefore, do not explicitly configure pathloss RS in joint TCI case and UL TCI test case. (vivo)

- Option 2: Configure the PLRS explicitly for joint TCI state swiching and UL TCI state test cases (Huawei, Apple, MTK)

- Option 3: Wait for RAN1 feedback for LS (vivo, Nokia, Ericsson)

| Company | Comments |
| --- | --- |
| Ericsson | We do not see much difference between option 1 and option 2. From the testing perspective, whether PL-RS is directly configured, or UE assumes source RS of UL TCI as PL-RS, both are same. Even in the case of option 2, Source RS of UL TCI can be configured explicitly as PL-RS. Whether explicitly configured or implicitly configured, we do not see much difference from testing complexity. If we have to chose between explicit and implicit config of PL-RS, we prefer implicit config as that may be most practical scenario. |
| Samsung | We can accept option 2 and config CSI-RS as PL-RS in two joint TCI test cases. |
| Apple | We support option 2. We need to test fallback behaviour aspects in RRM testcase. |
| Intel | Prefer Option 2. |
| vivo | We think option 3 would be better at this stage. For option 2, if CSI-RS is configured, we think it is OK to use the source RS or UL TCI. We prefer not to consider SSB for PL-RS since the core part has not finished. |
| Huawei | Support option 2. The purpose of TCI test cases is to verify the TCI state switching delay requirements, rather than to verify the signalling design. Explicitly PL-RS configuration will avoid the ambiguity in the test. |

Issues 1-1-1-b whether PL-RS is maintained or not maintained:

- Option 1: Pathloss RS is maintained in joint TCI test case (MTK)

- Option 2: Pathloss RS is not maintained in joint TCI test case (Samsung, Apple)

| Company | Comments |
| --- | --- |
| Ericsson | In the test case we are not configuring more than 4 RS and we can test maintained case. Prefer option 1. |
| Samsung | Prefer option 2. In the test such as 7.5.13.1, two TCIs are defined. CSI-RS can be configured as PL-RS but not maintained. |
| Apple | We support option 2. If we configure target PL-RS as maintained, it has to be in the active TCI state list for UL/joint TCI list and can only be applicable to UEs that support > 1 active TCI state. It would be more straightforward and applicable to UEs without additional UE capability of supporting > 1 activated TCI state in list. |
| Intel | Prefer option 2. |
| vivo | Not clear about the issue. PL-RS associated to (or included in) the target TCI is not maintained before UL TCI switching, but should be maintained after UL TCI switching. Not sure whether RAN4 is discussing the case before joint TCI switching or after TCI switching for this issue. |
| Huawei | If only two joint TCI states in total are configured to UE during the test, UE can be assumed to maintain the PL-RS in the two joint TCI, and option 1 is fine for us. |

Issue 1-1-2: How to define PL-RS of target TCI?

- Option 1 (vivo)

- RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list.

Companies still have unclear and concern on option 1 and vivo provides the potential spec impact if option 1 is agreeable.

| Company | Comments |
| --- | --- |
| Apple | We are not sure of this is correct, as maintained PL-RS is based on whether its in active TCI list for Joint/UL TCI in our understanding. Also, the UE is not maintaining PL-RS for any RS QCL Type D – but we have a restriction that in RAN4 the requirements are applicable only when the PL-RS is the same or QCL-TypeD to RS in UL TCI state. |
| vivo | Support option 1. We think RAN4 can proceed TC design by considering the case that does not depend on core part remaining issue. If no QCL-D between source and target UL TCI, the UE will definitely not track the UL TCI, and the PL is not estimated/maintained before TCI switching. |
| Huawei | The PL-RS of the target TCI shall be QCL type D with the source RS of the target TCI. |

Issue 1-1-3: How to configure maintained PL-RS / NOT maintained PL-RS in the test case

- Option 1: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”. (Samsung, vivo)

- Option 2:  Whether to maintain PL-RS is up to UE implementation, which cannot be configured (Huawei)

| Company | Comments |
| --- | --- |
| Ericsson | If we agree implicit configuration for PL-RS, for the known TCI state test cases, PL-RS can be maintained. |
| Samsung | For test cases, we suggest option 1. |
| Apple | As we commented in 1st round - If only 1 active TCI state is configured, and with TCI switch a new TCI state and new PL-RS is activated, then it would be PL-RS not maintained. The PL-RS and RS in UL/Joint TCI state should be the same or QCL Type D with each other, and the target UL /joint TCI should be configured for L1 measurement to ensure known state. |
| Intel | For PL-RS not maintain case, prefer that PL-RS is included in UL TCI state and PL-RS is identical to source RS in target UL TCI state. it’s the first activation for the UL TCI state. L1-RSRP based on the RS has been reported before. one question to option 2, if PL-RS is in the active TCI state list, it is maintained. Therefore, if source RS in UL TCI is in active TCI state list and RL-RS is the same as the source RS, is PL-RS configured as maintained? |
| vivo | We think the UL TCI should be known before the TCI state switching, i.e. the UE beam for UL transmission should be known. However, L1-RSRP measurement is not the same as PL-RS measurement. The sample of L1 measurement, in our understanding, can not be used for PL-RS measurement. Therefore, the UL TCI can be known but the PL-RS can be not maintained. Based on above understanding, we think option 1 is better. |
| Huawei | UE is assumed to be able to maintain no more than 4 PL-RS in total. If only two TCI states in total are configured to UE during the test, UE can be assumed to maintain the PL-RS in the two TCI states. If more than 4 TCI states in total are configured to UE, then whether a PL-RS in a TCI state is maintained is up to UE implementation. |

Issue 1-2-1: TRS configuration for cell with different PCI in the test case

- Option 1 (vivo)

- RAN4 may clarify in Note 4 of A.3.16.A.2-1 by adding the following sentence. ‘The TCI state of the TRS is the same as TCI.state.1 except that the additionalPCI field is also configured with PCI 0.’ In this case, no need to introduce a new TRS configuration or new TCI configuration.

- Option 2 (Huawei)

- New TRS configuration is needed for indicating that the TRS in DL TCI state associated with additional PCI is QCL-typeD to a SSB with the same additional PCI.

Companies are encouraged to provide more comments

| Company | Comments |
| --- | --- |
| Apple | Support option 2. |
| vivo | We think this issue can be directly discussed in the CR. The concern of option 2 is already addressed in the CR. |

Issue 1-3-1: The rate of correct events for unified TCI state switching test case

- Option 1 (MTK, Ericsson, Apple, Huawei, Samsung)

- Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the unified TCI state switching test case.

- Option 2 (vivo, Samsung)

- No need to specify “90%” threshold for the rate of correct events in the unified TCI state switching test case.

If no strong objection of option 1, could we agree option 1 based on majority views?

| Company | Comments |
| --- | --- |
| Ericsson | We agree with clarification provided from proponents of option 2. However, we do not have strong view on both options. We are fine with either option. |
| Samsung | We can accept option 1 to make all test cases aligned. |
| Apple | Support option 1. |
| vivo | Prefer option 2 but also fine to option 1, if companies agree to revise the R15/R16 TCs accordingly. Since no R15/R16 maintenance is discussed in this meeting, we prefer to FFS this issue. |
| Huawei | Support option 1. |

Issue 1-2-2: Whether to provide RRC configuration of the cell of additional PCI?

- Option 1 (vivo)

- Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE.

- Option 2

- Others

| Company | Comments |
| --- | --- |
| Apple | We understand the proposal, but how would cell with additional PCI configured in test without complete configuration? |
| vivo | We prefer option 1. We think additional PCI can be used in the TCI configuration, but there is no other configuration of cell of additional PCI. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Pathloss RS configuration in joint TCI test case | 6 companies provide comments. No consensus. - Option 1: RAN4 assumes that source RS of UL TCI can be used as pathloss RS if pathlossReferenceRS-Id-r17 is not configured. Therefore, do not explicitly configure pathloss RS in joint TCI case and UL TCI test case. (Ericsson) - Option 2: Configure the PLRS explicitly for joint TCI state switching and UL TCI state test cases (Huawei, Apple, MTK, Ericsson) - Option 2a: Configure CSI-RS as the PLRS explicitly for joint TCI state switching and UL TCI state test cases (vivo, Samsung) - Option 3: Wait for RAN1 feedback for LS (vivo, Nokia,) GTW agreements on 10/19: Agreements: - Configure CSI-RS as the PLRS explicitly for joint TCI state switching and UL TCI state test cases. |
| Issues 1-1-1-b/Issue 1-1-3: Whether PL-RS is maintained or not maintained in joint TCI test cases and how to config? | 3 companies prefer PL-RS is maintained in joint TCI test cases while other 3 companies prefer PL-RS is not maintained in joint TCI test cases. - Option 1: Pathloss RS is maintained in joint TCI test cases (MTK, Huawei, Ericsson) - Option 1a: If only two TCI states in total are configured to UE during the test, UE can be assumed to maintain the PL-RS in the two TCI states. If more than 4 TCI states in total are configured to UE, then whether a PL-RS in a TCI state is maintained is up to UE implementation. (Huawei) - Option 1b: implicit configuration for PL-RS, for the known TCI state test cases, PL-RS can be maintained (Ericsson) - Option 2: Pathloss RS is not maintained in joint TCI test cases (Samsung, Apple, Intel) - Option 2a: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS” (Samsung, vivo) - Option 2b: In the test cases, only define the test cases for PL-RS is not maintained. If only 1 active TCI state is configured, and with TCI switch a new TCI state and new PL-RS is activated, then it would be PL-RS not maintained. The PL-RS and RS in UL/Joint TCI state should be the same or QCL Type D with each other, and the target UL /joint TCI should be configured for L1 measurement to ensure known state. (Apple) - Option 2c: PL-RS is included in UL TCI state and PL-RS is identical to source RS in target UL TCI state. it’s the first activation for the UL TCI state. L1-RSRP based on the RS has been reported before. (Intel) |
| Issue 1-1-2: How to define PL-RS of target TCI? | - Option 1 (vivo) - RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list. - Option 2 (Huawei, Apple) - In the test cases, the PL-RS of the target TCI shall be QCL type D with the source RS of the target TCI. |
| Issue 1-2-1: TRS configuration for cell with different PCI in the test case | The comments are collected in CR directly |
| Issue 1-2-2: Whether to provide RRC configuration of the cell of additional PCI? | - Option 1 (vivo) - Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE. - Option 2 - Others |
| Issue 1-3-1: The rate of correct events for unified TCI state switching test case | 6 companies provided comments and are fine to option 1. Option 1: - Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the unified TCI state switching test case. |

Topic #2: Test cases for L1-RSRP measurement on cells with different PCI

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215974 | Samsung | Draft CR on TC of L1-RSRP measurement on cells with different PCI |
| R4-2216366 | vivo | Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA |

Open issues summary

No.

Companies views’ collection for 1st round

Open issues

No.

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215974 Samsung | Moderator: DraftCR, correction to A.6.6.4.6 (NR SA + FR1) |
|  | Ericsson: OK |
|  | vivo: Need more discussion. The understanding of 600ms+40ms+80slot can be the following for 15kHz SCS case: 600ms: SSB identification time 80 slot: L1-RSRP measurement time 40ms: margin for L1 reporting. The understanding of 600ms+40ms+80slot can be the following for 30kHz SCS case: 600ms: SSB identification time 80 slot + 20ms: L1-RSRP measurement time (totally 60ms) 20ms: margin for L1 reporting. Samsung: To vivo: we think 660ms=600ms+60ms. For L1, how 40ms is calculated. When DRX on, there is 1.5x scaling. In legacy tests, we think there is also mistake. In A.6.6.4.1(no DRX) and A.6.6.4.2(DRX), the requirements are both 640ms. It is not aligned with the formula in core part. Vivo2: To Samsung: As we explain above, for the 15kHz SCS case, 60ms is shorter than L1 reporting period, which is 80 slots, i.e 80ms. If revision is needed, we prefer to use ‘40 slots’ instead of ‘20ms’, so as to keep consistent with the 15kHz SCS case. Keeping consistent with R15/R16 is also important. |
| R4-2216366 vivo | Moderator: DraftCR, correction to A.6.6.4.6 (NR SA + FR1) |
|  | Ericsson: OK |
|  | Huawei: some changes are overlapped with R4-2215974 Samsung: some changes are overlapped with R4-2215974 |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Test cases for TRP specific BFD and LR

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216821 | Ericsson | - RAN4 to agree following table as test configuration for T1 to T5 FR; RS type; DRX Config; T1; T2; T3; T4; T5; D1. FR2; SSB; Non-DRX; 1s; 2.61s; 1.64s; 0; 1.01s; 0.97s. FR2; CSI-RS; DRX; 1s; 5.43s; 10.26; 0s; 0.31s; 0.27s. - RAN4 to agree to test following - For BFR on SpCells, CFRA and CBRA based BFR is configured for different test cases - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases - If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended for SSB based BFD. |
| R4-2216823 | Ericsson | CR on maintenance of test cases for TRP specific BFR |
| R4-2215358 | Intel | Proposal 1: Design intra-cell TRP specific BFR test case. Proposal 2: For intra-cell TRP specific BFR test case, if SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended. Proposal 3: For intra-cell TRP specific BFR test case, if CSI-RS is configured as BFD-RS, they are overlapped in the same symbol while in different frequency domain, the duration time will be extended. The duration time will be: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; CSI-RS; DRX; 1; 10.81; 10.28; 0; 0.57; 0.53. |
| R4-2215746 | Samsung | Proposal 1: In FR2 TRP specific BFR test case, SSB/CSI-RS should be overlapped for TRP1 and TRP 2 to test PTRP = 2. Proposal 2 : For FR2 TRP specific BFR test case based on SSB, BFD/CBD evaluation periods are 2 times scaled. The time periods should be update as: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 5.17; 3.24; 0; 1.97; 1.93. For FR2 TRP specific BFR test case based on CSI-RS, follow the same rule. |
| R4-2216284 | Huawei, HiSilicon | DraftCR on maintaining TRP specific BFR test cases |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 Test Config

Issue 3-1-1: Whether intra-cell TRP or inter-cell TRP specific BFR test cases are designed?

- Proposals

  - Option 1 (Intel)

    - Design intra-cell TRP specific BFR test case.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We are only testing or assuming one TRP may fail at any time. From that perspective we do not see much difference if we define intra-cell or inter-cell. We are fine with option 1 too. |
| Apple | We support the proposal. The current test cases already are based on this assumption in our understanding. |
| Huawei | Option 1 is fine for us. |
| Moderator | GTW discussion on Oct 11 is captured as below. It is suggested to continue the discussion. |
| Samsung | As mentioned, now we have 6 TRP-specific BFR tests. 4 are CSI-RS based BFD TCs and two are SSB based TCs. In Big CR agreed in last meeting, for 4 CSI-RS based BFD, A.4.5.5.8 A.5.5.5.8 (CSI-RS-based beam failure in the set (q0,0), (q0,1) configured for a serving PSCell and a cell with PCID different from the serving cell) A.6.5.5.7 CSI-RS q0,0 q0,1 are for the same cell A.7.5.5.9 CSI-RS q0,0 q0,1 are for the same cell From our side, for CSI-RS based BFR, we support intra cell. For two test cases based on SSB BFD, A.4.5.5.7&A.7.5.5.10, now q0,0 and q0,1 are from the same cell. But by our understanding of section 6 in TS38.213 It is clear mentioned “A UE can be provided, for each BWP of a serving cell, a set q0 of periodic CSI-RS resource configuration indexes by failureDetectionResourcesToAddModList and a set q1 of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt or candidateBeamRSSCellList for radio link quality measurements on the BWP of the serving cell. Instead of the sets q0 and q1, for each BWP of a serving cell, the UE can be provided respective two sets q0,0 and q0,1 of periodic CSI-RS resource configuration indexes by failureDetectionSet1 and failureDetectionSet2 that can be activated by a MAC CE [11 TS 38.321]” And for inter-cell “If a UE - is not provided coresetPoolIndex or is provided coresetPoolIndex with a value of 0 for first CORESETs on an active DL BWP of a serving cell, - is provided coresetPoolIndex with a value of 1 for second CORESETs on the active DL BWP of the serving cells, and - is provided SSB-MTCAdditionalPCI SS/PBCH block indexes associated with a physical cell identity other than the one provided by physCellId in ServingCellConfigCommon can be provided in either q1,0 or q1,1 set and the corresponding q0,0 or q0,1 set is associated with the physical cell identity.” From the description above, we think it is not clearly indicate UE provided in q0,0 and q0,1 of SSB for the same cell. From RAN2 spec 38.331, it is not restriction, q0,0 and q0,1 can be SSB or CSI-RS. For simplify, we prefer to SSB of q0,0 and q0,1 are from serving cell and additionalPCI because it is clearly defined in 38.213. For issue 3-1-3, we can compromise to SSB are not overlapped. |
| Intel | we support option 1. There is no much difference between intra-cell TRP and inter-cell TRP. for legacy BFD, there is similar question. No SSB based BFD is clearly defined in RAN1 while there is no limitation from RAN2. SSB based BFD requirement is still defined in RAN4. |
| MediaTek | Support option 1.Becuase we think they are not much different, we think it is sufficient to only test intra-cell case. Besides, in TS 38.213, there is no explicit wording to point out the SSB can be BFD-RS. To our understanding, it would be safe if we can send LS to RAN1 and RAN2 to clarify it. |

GTW on Oct 11:

Discussion:

Intel: all the TC are designed for intra-cell currently. We need to make sure SSB index are different in the test cases. So the measurement times are not scaled due to overlap. But for the CSI-RS based tests, overlap is observed and measurement time is scaled.

Huawei: we are ok to option 1.

Samsung: we have 6 test cases for BFR among which 4 are CSI-RS and 2 are SSB based ones. We prefer to use SSB from different PCI for SSB based test cases. For CSI-RS BFR we agree with using intra-cell as the assumption.

Apple: we support option 1. It is efficient to define intra-cell cases. There is no necessary to define test cases under SSB from different PCI.

MediaTek: we also support option 1. We share the same view with Apple.

Samsung: from RAN1 spec 38213, the UE can be provided with two sets of RS-s with different PCI. There is no clear definition for SSB from the same cell. Is it the case?

Apple: our understanding on the RAN1 spec SSB based BFD RS for Q00 and Q01 is not configurable but RAN2 spec they have the signalling ready. The BFD RS can be signalled as either CSI-RS or SSB.

Samsung: for safety we can define inter-cell SSB based test cases.

Nokia: for SSB based TC we go with intra-cell but for CSI-RS TC we could go with inter-cell assumption.

Ericsson: we support Nokia opinion. The only issue here is whether the SSB is overlapped from the two TRP. So the configuration is clear for SSB-based test cases.

Chairman suggest to continue the discussion and come back later.

Issue 3-1-2: Beam recovery method configured in the test case

- Proposals

  - Option 1 (Ericsson)

    - RAN4 to agree to test following

      - For BFR on SpCells, CFRA and CBRA based BFR is configured for different test cases

      - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We support Option 1 |
| Apple | We need not test all combinations in Rel-17 test cases, since it is already tested in R16 test cases. Okay to use option of configuring dedicated BFR resource for all tests. |
| Ericsson2 | We checked internally and we are fine with CFRA configuration for SPCell. |
| Moderator | GTW discussion on Oct 11 is captured as below. It is suggested to check whether the tentative agreement can be agreeable |
| Samsung | For BFR on SpCells, we are fine with CFRA based BFR is configured for all test cases. |
| Intel | Fine with that for BFR on SpCells, CFRA based BFR is configured. |
| Apple2 | We propose to have CFRA based BFR for tests with SpCells |

GTW on Oct 11:

Discussion:

Ericsson: here whether RA or dedicated BFR resource is configured for the test cases. We propose to have some TC to use RA and some to use dedicated BFR.

Apple: we are wondering if we need to test all combinations. We have full test list for RA test cases. We need to further check on the configuration of CFRA and CBRA for SpCells. We are fine for the SCells using dedicated BFR.

Vivo: CBRA based BFR is optional UE feature.

Agreement:

- For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

- For BFR on SpCells, FFS in the 1st round in this meeting whether CFRA based BFR is configured

Issue 3-1-3: If SSB is configurated as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not?

- Proposals

  - Option 1 (Ericsson, Intel)

    - If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended.

  - Option 2 （Samsung）

    - In FR2 TRP specific BFR test case, SSB/CSI-RS should be overlapped for TRP1 and TRP 2 to test PTRP = 2.

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | For intra-cell TRP specific tests they are not overlapped. We need to first agree on the scenario (intra-cell vs inter-cell) and in the test setup also we need to make this assumption clear. |
| Apple | Based on assumption of intra-cell scenario, the SSBs cannot be overlapped. Support option 1. |
| Huawei | For intra-cell TRP specific BFR, option 1 is supported. |
| Moderator | GTW discussion on Oct 11 is captured as below |
| Samsung | In issue 1-1-1, intra-cell with two SSB sets is a valid scenario, ssb resources must be non-overlapped. If it is inter-cell, it can be overlapped or not. We can compromise to SSB are not overlapped in inter-cell. So for the two test cases for SSB based BFR, the only issue is about the test parameter whether they are configured for the same cell or not. Other requirements including T1~T5 can be finalized. |
| Intel | Support option 1 with intra-cell TRP. |
| MediaTek | Support option 1 since we prefer to only test intra-cell scenario. |

GTW on Oct 11:

Session chair: let’s further check this together with 3-1-1 and comeback in the 2nd round.

Issue 3-1-4: If SSB is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5?

- Proposals

  - Option 1 (Ericsson, Intel, Huawei)

| FR | RS type | DRX Config | T1 | T2 | T3 | T4 | T5 | D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1s | 2.61s | 1.64s | 0 | 1.01s | 0.97s |

  - Option 2 (Samsung)

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 5.17 | 3.24 | 0 | 1.97 | 1.93 |

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Apple | Supporrt Option 1. |
| Samsung | I think for how to define T1~T5. Companies have the same understanding. The only different in option 1 and option 2 is about the scaling factor = 1 or = 2. If we can have consensus on Issue 3-1-3, this issue can be resolved accordingly. |
| Intel | Support option 1. |

Issue 3-1-5: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, whether CSI-RSs are overlapped or not?

- Proposals

  - Option 1 (Ericsson, Intel, Samsung)

    - Yes

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 is fine. |
| Apple | Okay with Option 1. |
| Moderator | GTW discussion on Oct 11 is captured as below |
| Intel | Follow the GTW agreement. |

GTW on Oct 11:

Agreement:

- If CSI-RS is configured as BFD-RS for TRP specific BFR test cases, CSI-RSs are considered as overlapped.

Issue 3-1-6: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5?

- Proposals

  - Option 1 (Ericsson)

| FR | RS type | DRX Config | T1 | T2 | T3 | T4 | T5 | D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | CSI-RS | DRX | 1s | 5.43s | 10.26 | 0s | 0.31s | 0.27s |

  - Option 2 (Intel, [Huawei (CR R4-2216284)])

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | CSI-RS | DRX | 1 | 10.81 | 10.28 | 0 | 0.57 | 0.53 |

- Recommended WF

  - Companies’ views are collected in 1st round discussion.

| Company | Comments |
| --- | --- |
| Ericsson | We are only testing one TRP BFD and CBD. We think UE will start CBD for only one TRP whose BFD is detected. From that perspective CBD-RS is not assumed to be overlapped. Hence we support option 1. |
| Apple | We support option 2. Since the BFD-RS are overlapping, the evaluation period is longer due to scaling factor. If CBD RS is configured and overlapped, then longer evaluation period also applies for D1/T5 as we don’t define UE behavior on when to measure CBD RS. |
| Huawei | If BFD/CBD RSs from two TRPs are configured as non-overlapped for this test, then option 1 is fine for us. |
| Samsung | According to the GTW agreement of Issue 3-1-5, option 2 should be used by 2 times scaled. |
| Intel | Support option 2. When BFD-RS and CBD-RS are overlapping, they will be scaled. |
| Ericsson3 | Since the UE behaviour is not specified when to measure the CBD-RS, we do not think having overlapping RS makes any sense. We prefer non overlapping RS for CBD. |

Companies views’ collection for 1st round

Open issues

Please add comments in tables in 3.2

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216823 Ericsson | Moderator: CR, correction to A.4.5.5.8 & A.7.5.5.9 & A.7.5.5.10 |
|  | Apple: Depending on some of the proposal/ issues discussed above. |
|  | Company B |
| R4-2216284 Huawei, HiSilicon | Moderator: CR, correction to A.4.5.5.7 & A.5.5.5.8 & A.6.5.5.7 & A.7.5.5.9 & A.7.5.5.10 |
|  | Ericsson: We can come back to this CR in second round after values and scenarios are more clear. |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |
| Issue 3-1-1: Whether intra-cell TRP or inter-cell TRP specific BFR test cases are designed? | Tentative agreements: N/A. In the email discussion and GTW discussion, the candidate options are reorganized as: Option 1: Design all TRP specific BFR test cases as intra cell. (Intel, Apple, Huawei, MediaTek) Option 2: For TRP specific BFR test cases, for 4 CSI-RS based TCs, design intra cell, for 2 SSB based TCs, design the test case as one serving cell and additionalPCI (Samsung) Option 3: For 2 SSB based TCs, design intra-cell, but for CSI-RS TCs, design inter-cell. (Nokia, Ericsson) Recommendations for 2nd round: Continue discussion. |
| Issue 3-1-2: Beam recovery method configured in the test case | GTW on Oct 11: Agreement: - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases - For BFR on SpCells, FFS in the 1st round in this meeting whether CFRA based BFR is configured Recommendations for 2nd round: Confirm whether the above tentative agreement can be agreeable. |
| Issue 3-1-3: If SSB is configurated as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not? | 6 companies provided comments on two options - Option 1 (Ericsson, Apple, Huawei, Intel, MTK) - If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended. - Option 2 （Samsung） - In FR2 TRP specific BFR test case, SSB/CSI-RS should be overlapped for TRP1 and TRP 2 to test PTRP = 2. Tentative agreements: N/A Recommendations for 2nd round: Continue discussion. Samsung can compromise to SSB are not overlapped and T1~T5 as option 1, if SSB of q0,0 and q0,1 are from serving cell and additionalPCI in parameter table. Based on majority views, can we go with tentative agreements as: SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 2.61; 1.64; 0; 1.01; 0.97. |
| Issue 3-1-4: If SSB is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5? | 4 companies provided comments on two options - Option 1 (Ericsson, Apple, Intel) FR; RS type; DRX Config; T1; T2; T3; T4; T5; D1. FR2; SSB; Non-DRX; 1s; 2.61s; 1.64s; 0; 1.01s; 0.97s. - Option 2 (Samsung) FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 5.17; 3.24; 0; 1.97; 1.93. Tentative agreements: N/A Recommendations for 2nd round: Continue discussion. Samsung can compromise to SSB are not overlapped and T1~T5 as option 1, if SSB of q0,0 and q0,1 are from serving cell and additionalPCI in parameter table. Based on majority views, can we go with tentative agreements as: SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 2.61; 1.64; 0; 1.01; 0.97. |
| Issue 3-1-5: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, whether CSI-RSs are overlapped or not? | GTW on Oct 11: Agreement: If CSI-RS is configured as BFD-RS for TRP specific BFR test cases, CSI-RSs are considered as overlapped. Recommendations for 2nd round: No further discussion. |
| Issue 3-1-6: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5? | 5 companies provided comments on two options - Option 1 (Ericsson) FR; RS type; DRX Config; T1; T2; T3; T4; T5; D1. FR2; CSI-RS; DRX; 1s; 5.43s; 10.26; 0s; 0.31s; 0.27s. - Option 2 (Apple, Huawei, Samsung, Intel) FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; CSI-RS; DRX; 1; 10.81; 10.28; 0; 0.57; 0.53. Ericsson supports option 1 which only extends BFD to 2 times but CBD as legacy time. Majority view is to keep both BFD and CDM as overlapped wither 2times extension. Tentative agreements: N/A Recommendations for 2nd round: Continue discussion. Based on the majority views, can we go with option 2? |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Issue 3-1-1: Whether intra-cell TRP or inter-cell TRP specific BFR test cases are designed?

- Option 1: Design all TRP specific BFR test cases as intra cell. (Intel, Apple, Huawei, MediaTek)

- Option 2: For TRP specific BFR test cases, for 4 CSI-RS based TCs, design intra cell, for 2 SSB based TCs, design the test case as one serving cell and additionalPCI (Samsung)

- Option 3: For 2 SSB based TCs, design intra-cell, but for CSI-RS TCs, design inter-cell. (Nokia, Ericsson)

| Company | Comments |
| --- | --- |
| Ericsson | Option 3 may be good compromise. For option 2, if additional PCI is configured, isn’t it inter-cell case? Can the proponents please clarify |
| Samsung | We support option 2. To Ericsson, for SSB based BFR test cases, we expect there are two cell, one is serving cell, and the other is additional PCI because we don’t sure whether SSB from two TRPs from the same cell is valid due to unclear in 38.213. |
| Apple | Support option 1. |
| Intel | Support option 1. |
| vivo | Option 1 is OK. |
| Huawei | Support option 1. |

Issue 3-1-2: Beam recovery method configured in the test case

GTW on Oct 11:

Agreement:

- For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

- For BFR on SpCells, FFS in the 1st round in this meeting whether CFRA based BFR is configured

Recommended WF:

Agreement:

- For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

- For BFR on SpCells, CFRA based BFR is configured.

Please check if the tentative agreement can be agreeable?

| Company | Comments |
| --- | --- |
| Ericsson | Fine with tentative agreements. |
| Samsung | Support the Recommended WF. |
| Apple | For BFR on SpCells configure CFRA based BFR. |
| Intel | For BFR on SpCells, CFRA based BFR is configured. |
| vivo | For BFD on SpCells, we prefer to re-use R15 methodology: For BFR on SpCells configure CFRA based BFR. |

Issue 3-1-3: If SSB is configurated as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not?

- Option 1 (Ericsson, Apple, Huawei, Intel, MTK)

- If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended.

- Option 2 （Samsung）

- In FR2 TRP specific BFR test case, SSB/CSI-RS should be overlapped for TRP1 and TRP 2 to test PTRP = 2.

Recommendations for 2nd round:

Continue discussion. Samsung can compromise to SSB are not overlapped and T1~T5 as option 1, if SSB of q0,0 and q0,1 are from serving cell and additionalPCI in parameter table. Based on majority views, can we go with tentative agreements as:

- SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as:

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 | 1.64 | 0 | 1.01 | 0.97 |

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Samsung | We can accept SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as in the table |
| Apple | Support option 1. SSB overlapping would only be possible for inter-cell scenario, which we don’t prefer as the configuration for tests (related to Issue 3-1-1) |
| Intel | Support option 1. |
| vivo | Up to issue 3-1-1 |
| Huawei | Support option 1 |

Issue 3-1-4: If SSB is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5?

- Option 1 (Ericsson, Apple, Intel)

| FR | RS type | DRX Config | T1 | T2 | T3 | T4 | T5 | D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1s | 2.61s | 1.64s | 0 | 1.01s | 0.97s |

- Option 2 (Samsung)

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 5.17 | 3.24 | 0 | 1.97 | 1.93 |

Recommendations for 2nd round:

Continue discussion. Samsung can compromise to SSB are not overlapped and T1~T5 as option 1, if SSB of q0,0 and q0,1 are from serving cell and additionalPCI in parameter table. Based on majority views, can we go with tentative agreements as:

- SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as:

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 | 1.64 | 0 | 1.01 | 0.97 |

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 |
| Samsung | We can accept SSB are not overlapped [no matter intra-cell or inter-cell], T1~T5 is defined as in the table |
| Apple | Support option 1 – based on preference for intra-cell and non overlapping SSB. |
| Intel | Support option 1. No scaling is configured. |
| Huawei | Support option 1, for non-overlapping SSB configuration. |

Issue 3-1-6: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5?

- Option 1 (Ericsson)

| FR | RS type | DRX Config | T1 | T2 | T3 | T4 | T5 | D1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | CSI-RS | DRX | 1s | 5.43s | 10.26 | 0s | 0.31s | 0.27s |

- Option 2 (Apple, Huawei, Samsung, Intel)

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | CSI-RS | DRX | 1 | 10.81 | 10.28 | 0 | 0.57 | 0.53 |

Ericsson supports option 1 which only extends BFD to 2 times but CBD as legacy time.

Majority view is to keep both BFD and CDM as overlapped withe 2times extension.

Recommendations for 2nd round:

Continue discussion. Based on the majority views, can we go with option 2?

| Company | Comments |
| --- | --- |
| Ericsson | For CBD, UE may or may not start CBD for other TRP whose BFD did not trigger. We do not know what we are testing here since it is UE implementation and CBD may or may not be started as it is UE implementation. Considering that we think it is reasonable to consider non-overlapping CBD-RS. |
| Samsung | Support option 2 as CSI-RS for BFD and CBD are overlapped and 2 times should be extended. |
| Apple | Support option 2 – the BFD and CBD RS are configured as overlapping so time needs to be extended for BFD and CBD evlaaiton periods. |
| Intel | Support option 2. The evaluation time needs to be scaled. |
| Huawei | Support option 2, for overlapping CSI-RS configuration. |

Summary for 2nd round

Open issues

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Whether intra-cell TRP or inter-cell TRP specific BFR test cases are designed? | 6 companies provide comments in 2nd round. no consensus - Option 1: Design all TRP specific BFR test cases as intra cell. (Intel, Apple, Huawei, MediaTek) - Option 2: For TRP specific BFR test cases, for 4 CSI-RS based TCs, design intra cell, for 2 SSB based TCs, design the test case as one serving cell and additionalPCI (Samsung) - Option 3: For 2 SSB based TCs, design intra-cell, but for CSI-RS TCs, design inter-cell. (Ericsson) |
| Issue 3-1-2: Beam recovery method configured in the test case | 5 companies provide comments in 2nd round and fine with: - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases - For BFR on SpCells, CFRA based BFR is configured. |
| Issue 3-1-3/Issue 3-1-4: If SSB is configurated as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not? How long are the time periods of T1~T5? | 6 companies provide comments. 5 companies support option 1 without text in square bracket. 1 company can compromise to option 1 with text in square bracket. - Option 1 (Ericsson, Apple, Huawei, Intel, MTK) - [No matter SSBs are from the same cell or from serving cell and additionalPCI], If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended. - T1~T5 are: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 2.61; 1.64; 0; 1.01; 0.97. GTW agreements on 10/19: Agreements: For both intra- and inter-cell TRP specific BFR test cases, if SSB is configured as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not? How long are the time periods of T1~T5? - [No matter SSBs are from the same cell, or they are from serving cell and a cell with additionalPCI], If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended. - T1~T5 are: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; SSB; Non-DRX; 1; 2.61; 1.64; 0; 1.01; 0.97. |
| Issue 3-1-6: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5? | 5 companies provide comments. 1 company support option 1 while 4 companies support option 2. - Option 1 (Ericsson) FR; RS type; DRX Config; T1; T2; T3; T4; T5; D1. FR2; CSI-RS; DRX; 1s; 5.43s; 10.26; 0s; 0.31s; 0.27s. - Option 2 (Apple, Huawei, Samsung, Intel) FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; CSI-RS; DRX; 1; 10.81; 10.28; 0; 0.57; 0.53. GTW agreements on 10/19: Agreement: FR; RS; DRX; T1(s); T2(s); T3(s); T4(s); T5(s); D1(s). FR2; CSI-RS; DRX; 1; 10.81; 10.28; 0; 0.57; 0.53. |

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on FeMIMO RRM Performance Requirement and Test Case | Samsung |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2216365 |  | Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA | vivo | Revised |  |
| R4-2215766 |  | Draft CR on TC for joint unified TCI state switching in FR2 NR SA | MediaTek Inc | Revised | Depends on open issues. |
| R4-2216822 |  | CR on maintenance of UL TCI state switching of FR2 PCell | Ericsson | Agreeable |  |
| R4-2215974 |  | Draft CR on TC of L1-RSRP measurement on cells with different PCI | Samsung | Revised | Vivo has concern on change 1. Need further clarification; Change 2 overlapped can be merged to R4-2216366 |
| R4-2216366 |  | Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA | vivo | Agreeable |  |
| R4-2216823 |  | CR on maintenance of test cases for TRP specific BFR | Ericsson | Revised | Depends on open issues. |
| R4-2216284 |  | DraftCR on maintaining TRP specific BFR test cases | Huawei, HiSilicon | Revised |  |
| For other discussion paper |  |  |  |  |  |
| R4-2216364 |  | Discussion on R17 feMIMO test case configurations | vivo | Noted |  |
| R4-2215745 |  | Discussion on remaining issues of test cases for unified TCI state | Samsung | Noted |  |
| R4-2216821 |  | Discussion on test configuration for TRP specific BFR | Ericsson | Noted |  |
| R4-2215358 |  | Discussion on test case configuration for TRP specific Beam Failure Detection and Link Recovery | Intel | Noted |  |
| R4-2215746 |  | Discussion on remaining issues of test cases for TRP specific BFD and LR | Samsung | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2217208 |  | WF on FeMIMO RRM Performance Requirement and Test Case | Samsung | Agreeable | No further comments received |
| R4-2216365 | R4-2217209 | Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA | vivo | Endorsed. | No further comments received |
| R4-2215766 | R4-2217210 | Draft CR on TC for joint unified TCI state switching in FR2 NR SA | MediaTek Inc | Endorsed |  |
| R4-2215974 | R4-2217211 | Draft CR on TC of L1-RSRP measurement on cells with different PCI | Samsung | Endorsed | No further comments received |
| R4-2216823 | R4-2217212 | CR on maintenance of test cases for TRP specific BFR | Ericsson | Endorsed |  |
| R4-2216284 | R4-2217213 | DraftCR on maintaining TRP specific BFR test cases | Huawei, HiSilicon | Endorsed |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents