3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216022

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results on FR2-2 PUSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.3.1

Document for:	Discussion

Background

As per [1], RAN4 left many open issues for FR2-2 PUSCH requirements. In this paper we provide our views.

Discussions

Sub-carrier spacings for PUSCH requirements

The open issues are left as follows:

- Option 1: 120 Hz and 480 kHz - Option 2: 120 kHz, 480 kHz and 960 kHz - Option 3: 120 kHz, 480 kHz and 960 kHz - 960 kHz requirements are declared optional

Similar to other PHY channel, we suggest to not consider 960kHz SCS. Firstly, 960kHz SCS brings much trouble to testability as analysed in previous meetings. Secondly, 960kHz needs too many continuous UL slots which exceeds maximum number of supported HARQ processes. For option 3, we don’t understand the intention since all features are based on declaration.

Proposal 1: RAN 4 to define the PUSCH requirements with 120 SCS and 480kHz SCS.

TX/RX branches for the 64QAM

The open issues are listed as follows:

- Option 3-1: MCS 18 with 1T2R Low and 2T2R Low - Option 3-2: MCS 20 with 1T2R Low - Note: If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2 - Option 3-3: MCS 20 with 1T2R Low and MCS 18 with 2T2R Low - Note: If the final link budget agreement indicate 20dB SNR limit could be applied for FR2-2

We copy our simulation results from another paper in following:

Table 2-1: Simulation results for FR2-2 PUSCH requirements for CP-OFDM

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | -2.0 |
|  |  |  |  | 2x2 Low | 2.6 |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 9.8 |
|  |  |  |  | 2x2 Low | 19.6 |
| 120 | 100 | 16 | TDLD30-650 | 2x2 Low | 11.5 |
| 120 | 100 | 20 | TDLA30-200 | 1x2 Low | 10.9 |
|  |  |  |  | 2x2 Low | 15.2 |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | -3.5 |
|  |  |  |  | 2x2 Low | 0.2 |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 7.3 |
|  |  |  |  | 2x2 Low | 11.3 |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 10.1 |
|  |  |  |  | 2x2 Low | 15.1 |

We can observe that for the target SNR is about 15dB for 2TX,MCS20, which is further lower than 20dB. Therefore, we propose to define requirements for 2T2R for MCS20.

Others

In Rel-15 FR2 PUSCH requirements definition, RAN4 disabled PTRS configuration for QPSK for both CP-OFDM and DFT-S-OFDM. In FR2-2 WI, based on our analysis, QPSK is also susceptible to phase noise, hence Base station always disable PTRS transmission to improve throughput. Therefore, we propose to disabled PTRS transmission for QPSK.

Proposal 2: Disabled PTRS for QPSK for both CP-OFDM and DFT-S-OFDM.

From our simulation results we can find for case with (120kHz SCS/100MHz bandwidth, MCS16, TDLA30-650,2T2R), the target SNR is quite close to 20dB. If we use TDLD30-650 to instead of TDLA30-650, the performance is further improved. Therefore we propose to use TDLD30-650 to instead of TDLA30-650 for this case.

Observation 1: For case with (120kHz SCS/100MHz bandwidth, MCS16, TDLA30-650,2T2R), the target SNR is quite close to 20dB. If we use TDLD30-650 to instead of TDLA30-650, the performance is further improved.

To sum, we propose to use following cases for PUSCH requirements definition:

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-650 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 | TDLA30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |

Proposal 3: Use cases in following table for PUSCH requirements definition:

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-650 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 | TDLA30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |

Conclusion

In this paper we provide our views on open issues for FR2-2 PUSCH requirements. The proposals are:

Proposal 1: RAN 4 to define the PUSCH requirements with 120 SCS and 480kHz SCS.

Proposal 2: Disabled PTRS for QPSK for both CP-OFDM and DFT-S-OFDM.

Observation 1: For case with (120kHz SCS/100MHz bandwidth, MCS16, TDLA30-650,2T2R), the target SNR is quite close to 20dB. If we use TDLD30-650 to instead of TDLA30-650, the performance is further improved.

Proposal 3: Use cases in following table for PUSCH requirements definition:

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-650 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 | TDLA30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |

Reference

[1]   R4-2214388_WF_PUSCH_FR2-2. Nokia, Nokia Shanghai Bell.