3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216018

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Discussions on remain issues on CSI reporting requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.4

Document for:	Discussion

Background

In the last meeting, we provide our simulation results for CQI reporting requirements for FR2-2

Simulation results

The simulation results for CQI distribution and BLER requirements are captured in Table 2-1 (Without Phase noise modelled) and Table 2-2(With Phase noise). We use bandwidth/SCS 100MHz/120kHz and reuse the test parameters of Rel-15 FR2 CQI AWGN test with CQI table 1 is assumed.

Table 2-1: Simulation results without phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100% | 0 | 0 | 100% |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 87.5% |
| 12 | 10 | 100% | 0 | 0 | 88.9% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 0 | 24.0% |
| 18 | 13 | 100% | 0 | 0 | 53.8% |

Table 2-2: Simulation results with phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 0 | 100% | / | / | 0.07 |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 100% |
| 12 | 10 | 100% | 0 | 0 | 100% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 11% | 100% |
| 18 | 13 | 100% | 0% | 100% | 100% |

Discussions

SNR value

We have following options for this issue:

Issue 3-2-2: Target Modulation Order for FR2-2 CQI Requirements: - Option 1: Up to 16 QAM; - Option 2: Include 64QAM; Issue 3-2-3: Whether to reuse CQI requirements test setup and metrics from FR2-1 for 120kHz/100MHz: - Agreement: For FR2-2 CQI requirements for 120kHz/100MHz, reuse test setup and test metrics from FR2-1; - Further discuss whether FR2-1 SNR test points can be reused - Option 1: Yes; - Option 2: Test points: SNR = 6/7 for 16QAM, -2/-1dB for QPSK;

The test SNR should be lower than maximum testable SNR defined by RF part which is 7.7dB for 120kHz/100MHz bandwidth. Based on our simulation results, 7dB SNR is related to 16QAM and phase noise impact can be ignored. Hence we suggest to select 6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Observation 1: SNR corresponding to 64QAM is larger than maximum DL testable SNR.

Proposal 1: Select SNR=6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Conclusion

In the last meeting, we provide our views on remain issues on CQI reporting requirements for FR2-2. The observations and proposals are:

Observation 1: SNR corresponding to 64QAM is larger than maximum DL testable SNR.

Proposal 1: Select SNR=6/7 dB to verify CQI reporting related to 16QAM and -2/-1 dB to verify CQI reporting related to QPSK.

Reference

[1]   R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated